# 3GPP TSG-RAN WG4 Meeting #113                                R4-2418457

# Orlando, US, 18th - 22nd November, 2024

Agenda Item:	7.17.1

Source: 	CMCC

Title: 			 Discussion on general aspects

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #105 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on general aspects.

- Discussion

Issue 1-2: Post deployment testing When operating in the field, two aspects of UE operation may impact performance: - Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance - Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance. For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive: - Option 1: Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs - Validation takes into account the UE hardware in which the model is to be deployed/activated. - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model) - FFS whether validation can be done at the device along with inference for another active functionality(ies) - FFS on the feasibility - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models. - Other aspects not precluded. - Option 2: Using performance monitoring and LCM procedures - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable

The reason to consider post deployment handling is that the change/update/modification of AI/ML model/functionality may be very frequent. It is questionable whether a device that has passed the conformance test with one version of AI/ML model/functionality can also pass the test after the change/update/modification. What’s more, it is also possible that a totally new model is loaded to the device after deployment, which means its performance are not tested and cannot be guaranteed. GCF has procedure about how to deal with changes in certified products, it is feasible to handle the changes in already certified/deployed UEs. The details can be found in our RAN5 contribution[4]. Taking above into consideration, it is essential to consider post deployment handling.

As for the detailed solution, option 1 is not clear since there are too many FFS parts, it is better that proponent could provide more details. As we discussed previously, only relying on conformance testing cannot solve the post-deployment handling issue. Firstly, AI/ML model/functionality performance is sensitive to the scenarios/configuations, it is not possible to enumerate all the possible scenarios/configuations that may be encountered in the field, which is also a heavy test burden. Secondly, as discussed above, the change/update/modification of  AI/ML model/functionality happen after passing the comformance test, which means the conformance tests cannot test the new version after change/update/modificatio. Only relying on  LCM is also questionable. As agreed in previous meeting, the candidates that RAN4 considers to define RRM requirements for LCM procedures include identification, selection, activation, deactivation, switching, fallback to non-AI operation, performance monitoring. Firstly, in our understanding, the main target of LCM is to verify whether UE perform correponding procedure in time when the network indication is received, e.g. activation/deactivation delay requirements may be specified in RAN4. Secondly, for LCM with performance monitoring, its target is to monitor whether there is performance drift for current serving model/functionality. While one of the motivation to have post-deployment handling is to check the performance of new/updated models before it is used to the devices. Taking above into consideration, it seems that option 2 to combine performance monitoring and LCM procedures is a possible way. However, the details about performance monitoring and LCM procedures are not decided in RAN4, which means the basic funtionality are not settled, it is difficult to discuss how to combine these to handle post deployment.

Proposal 1: it is proposed to consider post deployment handling, but the detail about how to handle post deployment can be further discussed after that the basic design about performance metric and LCM procedures are clear.

Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

For the target for AI/ML based beam management, our view is that AI/ML based performance requirement should not be worse than the legacy. According to the discussion in last meeting, some companies mentioned that the AI/ML based performance will be worse than the legacy, since the gain come from overhead reduction. From operator point of view, we do not think that this is a good way to go. If the predicted results are worse than the legacy measured results, it will have impact on the network deployment, which is not preferred.

Proposal 2: for AI/ML based beam management, AI/ML based performance requirement should not be worse than the legacy measurement performance.

Issue 1-7: Number of tests - Proposals - Option1: Each specified AI/ML-enabled functionality shall have RAN4 requirements and pass at least one test for device conformance for this functionality. - Option 2: A minimum set of Associated IDs should be defined as mandatory (mandatorily applicable) for testing of each functionality. - Option 3: RAN4 does not need to define performance requirements that imply the change of Associated ID during the test. - Option 4: others

In last meeting, the testing goals are updated as to verify whether the minimum performance of AI/ML functionality/feature can be achieved, but FFS on whether and how many different test configurations/parameters are used for tests [4].

In our view, the testing goals are high level principle, which are applicable to both inference test and generalization test. For the FFS part, it is necessary to consider different test configurations/parameters at least for generalization test. According to Rel-18 discussion, the goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations.

Proposal 3: it is proposed that different test configurations/parameters are used for tests.

As for how many different test configurations/parameters are used for tests, it can be further discussed based on RAN1/2 progress. Based on RAN1 evaluation, generalization is impacted by many factors. In high level, according to TR 38.843, deployment scenarios, UE distributions, carrier frequency,  TxRU mappings are considered for generalization of CSI compression. And CSI payload sizes,  Tx port numbers, are considered for scalability evaluations for CSI compression. For beam management, the performance is impacted by deployment scenarios, UE mobility, etc. Similar situation for positioning. It seems difficult to decide the scenarios and/or configurations used for generalization tests. On one hand, if we consider all the impacted factors into account, the number of tests may be huge. On the other hand, if the number of tests is limited to reduce the test burden, it is doutful whether generlization is verified well.

According to RAN1 discussion [5], for AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. It can be seen that UE will report the supported configurations  for a certain model/functionality. The scenarios and/or configurations for generalization can be decided based on the supported configuration reported by UE.

In last RAN2 meeting, a LS on applicable functionality reporting for beam management UE-sided model is aent to RAN1. One of the questions is what is the granularity of functionality. For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others. In our understanding, the others means more detailed scenarios and/or configurations, which can be used to decide the scenarios and/or configurations used for test.

Proposal 4: RAN4 requirements and tests are specified at least based on functionality, and the details can be further discussed pending on the RAN1/2 progress on granularity of functionality.

- Conclusion

This contribution provides discussion on general aspects. The proposals are:

Proposal 1: it is proposed to consider post deployment handling, but the detail about how to handle post deployment can be further discussed after that the basic design about performance metric and LCM procedures are clear.

Proposal 2: for AI/ML based beam management, AI/ML based performance requirement should not be worse than the legacy measurement performance.

Proposal 3: it is proposed that different test configurations/parameters are used for tests.

Proposal 4: RAN4 requirements and tests are specified at least based on functionality, and the details can be further discussed pending on the RAN1/2 progress on granularity of functionality.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R5-246707, Discussion on handling of AI/ML in RAN5, China Mobile, AT&T, CAICT, Ericsson, Huawei, MediaTek, ZTE

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface