# 3GPP TSG-RAN WG4 Meeting #113                                                       R4-2418482

# Orlando, USA, 18th – 22rd November, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for less than 5MHz in NTN enhancement

Agenda item:	7.8.3.4

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN evolution [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. In the last meeting we provided our initial views on potential impacts on RRM requirements and in this meeting we will continue to study the concrete RRM requirements.

Discussion

The objectives of Rel-19 in NR NTN evolution are described in [1] including:

2.1 Less than 5MHz for NTN

- In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective

  - 2.1 Less than 5MHz for NTN

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

2.1.1 Requirements for NTN less than 5MHz

We shall study potential impacts based on the contents in the following table. In the last meeting, we RAN4 has already agreed most of requirements:

| RRM requirements | NTN requirements in TS38.133 v18.5.0 | Possible impacts if BW below 5MHz |
| --- | --- | --- |
| IDLE/inactive mode mobility | 4.2C Cell Re-selection for NR UE for Satellite Access | No RRM requirements impact |
| RRC connection mobility control | 6.2C RRC Connection Mobility Control for Satellite Access | - RRC Re-establishment - RRC connection release with redirection for satellite access |
| Handover | 6.1C Handover for SAN | - CHO with L3 measurement - Satellite switch time - Handover delay(RACH-based and RACH-less) |
| UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy | 7.1C, 7.2C, 7.3C | - No impact |
| RLM | 8.1C Radio Link Monitoring for Satellite Access - 8.5C Link Recovery Procedures for Satellite Access | - Radio link monitoring (only SSB based) - Link recovery procedures for satellite access - Candidate beam detection (CBD) |
| Measurements |  | - L3 measurement, define new requirements - L1 RSRP measurement,no impact |

2.1.1.1 RRC Connection release with redirection

- For this issue, the main requirements is the time for identifying the target NR cell in legacy TN and NTN which based on the SSB side conditions, the serving cell SSB Ês/Iot≥-8dB. However, we do not find the change of the SSB side conditions if the RRM requirement will be reused and even there is no RRM requirements for less than 5MHz in TN in spec. Thus, we deem that for RRC connection Release with re-direction, there is no RRM impacts on 3MHz in NTN scenario.

- Observation 1: There is no RRM requirements for less than 5MHz in TN.

- Proposal 1: For RRC connection release with redirection, there is no RRM impacts on  3MHz in NTN scenario.

2.1.1.2 Handover/Conditional handover/Satellite switch

In the last meeting, we reached the agreements as below:

Agreements/ Way-forward: - for CHO without L3 measurement: - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] - FFS on for CHO with L3 measurement: - Tmeasure which is same as the definition for Tidentify_intra_with_index and Tidentify_inter_with_index in issue 1-2-4-2 will be extended - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs]

We will study the related delay requirements for handover.

2.1.1.1.1 Conditional Handover

We also find there is the impact on conditional handover and RAN4 shall consider studying it. Conditional handover contains two parts: Conditional handover with L3 measurement in NTN and Conditional handover without L3 measurement in NTN. In the last meeting, we have already defined requirements on conditional handover without L3 measurement. In this part we mainly focus on the conditional handover delay with L3 measurement.

Conditional handover delay with L3 measurement

UE shall start RRM measurement before the time or distance condition is met. For the conditional handover delay with L3 measurement, the main handover delay is:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

The definition of Tinterrupt is:

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

And the measurement time delay Tmeasure is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

Each component has the different definition and we shall focus on the parameters which will be impacted by punctured PBCH. From my perspective, only Tmeasure and Tinterrupt will be impacted since they depend on PBCH detection. From the TS38.133, we know that the measurement time delay L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause nine which is related to TSSB time index and TPSS/SSS sync. For Tinterrupt, it depends on SSB index reading.

For TΔ, we shall follow the legacy TN extension as below:

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Few companies has one concern that if the Tmeasure and TΔshall be enhanced both. From my perspective, Tmeasure depends on the another clause in TS38.133 and whether  the Tmeasure shall be enhanced or not shall depend on whether RAN4 define new L3 measurement requirements or not. For  TΔ, we shall only follow the same legacy TN  principle.

Observation 2: Tmeasure and Tinterrupt will be impacted since they depend on PBCH detection.

Proposal 2: Whether the Tmeasure in conditional handover with L3 measurement shall be enhanced or not shall depend on whether RAN4 defines new L3 measurement requirements.

Proposal 3: TΔ shall be extended to 3*Trs.

2.1.1.3  L3 measurement

- For the measurement requirements for both intra-frequency and inter-frequency, the main factors to be impacted is the cell and SSB index detection since it depends on the PBCH detection directly. Hence, we shall also focus on this aspect also. Compared to legacy SSB with 20PRBs, the 12PRBs SSB can guarantee the transmission of PSS/SSS but part of PRBs of PBCH are punched. Thus, the new measurement requirements for less than 5MHz have been defined such as cell identification etc. We also can see the following agreements defined in less than 5MHz WI:

Agreement: - SSB index identification delay - Unknown intra-frequency cell (Es/Iot≥-6 dB): [3+4] samples - Unknown inter-frequency cell (Es/Iot≥-4 dB): [3+3] samples

- For unknown cell, we find that the measurement samples were increased based on 12PRBs SSB in order to  compensate the impact of puncturing for unknown cell searching, and the relevant requirements have been captured in the existing TS38.133 as below:

- Legacy TN time index detection requirements: Table 9.2.5.1-3: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; Ceil(3 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. - Legacy TN less than 5MHz time index detection requirements: Table 9.2.5.1-23: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) DRX cycle; TSSB_time_index_intra_less_than_5Mhz. No DRX; max(120ms, 7 x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz. DRX cycle≤ 320ms; max(120ms, ceil(1.5 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz). DRX cycle>320ms; 7 x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz. NOTE 1: FFS When highSpeedMeasInterFreq-r17. - Legacy NTN time index detection requirements: Table 9.2C.5.1-2: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra_less_than_5Mhz. DRX cycle≤ 320ms; max(120ms, ceil (1.5 x 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra_less_than_5Mhz. DRX cycle>320ms; Ceil(3 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra_less_than_5Mhz. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

- Based on the table in above, we find that the time period for time index detection for NTN is different from legacy considering Klayer1_measurement (considering without measurement gap),Kgap (considering with measurement gap) , Kmulti_SMTC and CSSFintra_less_than_5Mhz. However, the number of samples is same. Hence, from my perspective, when considering defining the requirements for less than 5M in NTN scenario, RAN4 shall refer to time period defined in less than 5M with Klayer1_measurement , Kmulti_SMTC and Kgap.

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) for intra-frequency without measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x max(MGRP, SMTC period))x Klayer1_measurement x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Klayer1_measurement x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x Kmulti_SMTC x CSSFintra_less_than_5Mhz |
| NOTE 1: FFS When highSpeedMeasInterFreq-r17 NOTE 2: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1) for intra-frequency with measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz |
| Note 1: CSSFintra_less_than_5Mhz is 1 NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

- We just provide the time period for time index detection for intra-frequency without/with measurement gap. The same situation can be studied for inter-frequency measurement. RAN4 shall refer to the legacy less than 5MHz requirements and legacy NTN requirements when considering defining the new requirements.

- Proposal 4: RAN4 shall define the related requirements for less than 5MHz in NTN scenario :

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) for intra-frequency without measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x max(MGRP, SMTC period))x Klayer1_measurement x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Klayer1_measurement x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x Kmulti_SMTC x CSSFintra_less_than_5Mhz |
| NOTE 1: FFS When highSpeedMeasInterFreq-r17 NOTE 2: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1) for intra-frequency with measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz |
| Note 1: CSSFintra_less_than_5Mhz is 1 NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

Conclusion

In this paper we provided our views on RRM requirements on R18 NR NTN enhancement.

- Observation 1: There is no RRM requirements for less than 5MHz in TN.

- Proposal 1: For RRC connection release with redirection, there is no RRM impacts on  3MHz in NTN scenario.

Observation 2: Tmeasure and Tinterrupt will be impacted since they depend on PBCH detection.

Proposal 2: Whether the Tmeasure in conditional handover with L3 measurement shall be enhanced or not shall depend on whether RAN4 defines new L3 measurement requirements.

Proposal 3: TΔ shall be extended to 3*Trs.

- Proposal 4: RAN4 shall define the related requirements for less than 5MHz in NTN scenario :

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) for intra-frequency without measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x max(MGRP, SMTC period))x Klayer1_measurement x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Klayer1_measurement x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x Kmulti_SMTC x CSSFintra_less_than_5Mhz |
| NOTE 1: FFS When highSpeedMeasInterFreq-r17 NOTE 2: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1) for intra-frequency with measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz |
| Note 1: CSSFintra_less_than_5Mhz is 1 NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133