3GPP TSG-RAN WG4 Meeting # 113                                    R4-2418503

Orlando, US, 18th – 22nd November, 2024

Agenda item:	5.23.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Further NR mobility enhancements

Document for:	Approval

Introduction

In this contribution we discuss the remaining open issues for mobility core part and performance part requirements based on the RAN4#112 way forward [1].

Discussion

In this contribution, we will provide our views on some issues based on the WF [1] which agreed in RAN4#112 meeting.

2.1 L1/L2 based inter-cell mobility

Issue 1-2-1: Whether to consider early TCI state activation for multiple cells at the same time < Agreement>: - Interested companies can have further offline discussion. If no further progress, the following will be agreed in the Nov meeting. - No requirements of early TCI state activation delay are specified for the case that multiple LTM TCI activation commands are received at the same time.

- For this issue, we should discuss whether to consider early TCI state activation for multiple cells at the same time. RAN2 has reached consensus that early TCI state activation for multiple candidate cells, but each TCI state activation via MAC-CE is executed independently in previous meeting. That’s to say, one MAC-CE activates TCI states only for a single candidate cell. With this information, early TCI state activation for multiple cells at the same time only comes from the scenario with multiple time-overlapping MAC CE commands. As mentioned by other company, it is not a typical configuration that network simultaneously indicates multiple LTM TCI activation MAC CE for multiple cells. Based on the above analysis, we prefer not to specify requirements for the scenario with multiple time-overlapping MAC CE commands.

- Proposal 1: Early TCI state activation delay requirements to be defined for one or more TCI states for a single candidate cell, because one MAC-CE activates TCI states only for a single candidate cell.

Issue 1-2-2: The definition of Tfirst-SSB in early candidate cell’s TCI state activation delay for inter-frequency with gap for known TCI state case RAN4#111 < Agreement>: - In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG. - FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG. - Same agreement applies to the definition of Tfirst-SSB in PDCCH order RACH delay requirement.. < Way Forward > FFS the following options: - Option 1 (Apple, MTK, Ericsson): - remove “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG” in definition of TSSB in PDCCH ordered RACH and early TCI activation requirements. - Option 2 (ZTE, vivo): - Confirm the FFS.

- In the current specification, the description of Tfirst-SSB in MAC-CE based TCI state switch delay can be used as baseline, that is, Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE. Different from legacy, we should consider the cases of SSB being within and outside active BWP for L1-RSRP measurement. Therefore, the value of Tfirst-SSB can be divided into two parts. It’s common understamding that Tfirst-SSB is the time to first SSB occasion after slot n + THARQ +3Nslotsubframe,µ when SSB is within the active BWP, and TSSB is the time to first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ when SSB is outside the active BWP. In previous meeting, some companies proposed to add additional time to execute some adjustments related to MG. If there is no misunderstanding of the intention, we think that [2ms] only applies to UE supporting inter-frequency measurement with MG is reasonable.

- For this issue, there remains a question that the first bullet is only applicable to UE supporting inter-frequency L1 measurement with MG or not. In our understanding, RAN4 has agreed to decouple LTM and L1 measurement. That’s to say, LTM can be excuted without L1 measurement. Therefore, the definiation of Tfirst-SSB should be applied to both inter-frequency L1 measurement with MG and inter-frequency L3 measurement with MG.

Proposal 2: In early candidate cell’s TCI state activation delay for known TCI state case, the definiation of Tfirst-SSB should be applied to both inter-frequency L1 measurement with MG and inter-frequency L3 measurement with MG as LTM can be triggered by L1 or L3 measurement.

Issue 1-4-3-1: Which cell(s) TLTM-RRC-processing = 0 apply to when candidate cells configured are more than UE capability? Online agreement <Agreement>: - If the total number of cells to be fast RRC decoded does not exceed UE capability maxNumberStoredConfigCells-r18 and the number of LTMCandidateConfigs fast decoded does not exceed maxNumberConfigs-r18, TLTM_RRC-processing =0 applies to the LTM candidates with early TCI activation or early PDCCH order RACH, - Further discuss: - FFS: Otherwise, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) [and/or] PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18. - FFS: This is at least applicable to the case that NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion. - FFS: Whether to support and further optimize the case that NW triggers TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

This issue is about the condition on time gap which can guarantee TLTM_RRC-processing =0. In previous meeting, we have consensus that both TCI state activation comand and PDCCH-order RACH can trigger early ASN.1 decoding and validity check on the candidate cell. And there is an agreement that TLTM_RRC-processing in TS38.133 is zero if the time gap is larger than a certain value.

In this issue, we should handle the case how to choose cells to do early decoding and validity check when candidate cells configured are more than UE capability. it’s the common understanding that there is no need to consider  early TCI state activation or PDCCH order RACH if number of cells to perform early RRC decoding and validity check does not exceed UE capability. Conversely, if candidate cells configured are more than UE capability, we need to do early RRC decoding and validity check for the cells which have higher propability to be target cell. Consequently, we agreed TCI state activation comand and PDCCH-order RACH can trigger early ASN.1 decoding and validity check on the candidate cell.

- In our understanding, if the UE capability is still exceeded under the screening of these two conditions, i.e.,TCI state activation comand and PDCCH-order RACH, then further screening will be conducted based on the principle of the most recent occurrence.

- Whether to support and further optimize the case that NW triggers TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion, it depends on the issue whether to consider early TCI state activation for multiple cells at the same time.

- Proposal 3: When TCI state activation MAC-CE or PDCCH order is sent for more cells than UE capability for fast processing, the cells for which the UE received TCI state activation MAC-CE or PDCCH order the most recently before cell switch command are the ones that are pre-processed.

Issue 1-4-5-1: Whether are cell switch delay requirements applicable to FR2 without L1 measurement report? < Way Forward > FFS the following options: - Option 1 (OPPO): - Yes when SNR of the TCI state≥ -3dB and TCI state has been activated, it is also required that the RS associated to the target TCI state is available at least every 1280ms after TCI state activation command. - Option 2 (Nokia): Yes with the following conditions: - The target TCI state in the cell switch command is activated not more than 1280 ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or - The target TCI state in the cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every 1280 ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB - Option 3 (Ericsson, QC): Yes with the following conditions: - [- The TCI state is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state is ≥ -3dB; where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same; or] - [- The TCI state is activated before the reception of the cell switch command (where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same) and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or] - Option 4 (MTK): - Not to extend cell switch delay requirements to FR2 without L1 measurement report

- In RAN4#111 meeting, we have discussed whether it is flexible to decouple LTM and L1 measurement and reached the agreement that the cell switch delay requirement is applicable for the case without L1 measurement and report under certain conditions in FR1. As to FR2, due to the benefit is less without L1 measurement, the RAN4 Rel-18 related LTM requirements are not applicable if L1 measurement of LTM candidate cell is not supported or configured. However, some companies proposed that L1 report may be not necessary for cell switch delay especially when TCI state is activated. If TCI state is activated, cell switch delay will be reduced compared to legacy L3 handover.

In our understanding, it’s resaonable to consider the case that TCI state is activated before the reception of the cell switch command. And the benefit can be observed in FR2 without L1 measurement report.

Observation 1: for FR2 without L1 measurement report, cell switch delay could be reduced compared with legacy handover when TCI state is activated.

Proposal 4: cell switch delay requirements applicable to FR2 without L1 measurement report with the following conditions

- The target TCI state in the cell switch command is activated not more than [1280] ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or

- The target TCI state in the cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every [1280] ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB

2.2 Other

We have agreemrnt in RAN4#11bis meeting that UE is not required to report carriers which are not configured by NW [2] and capture it in the spec.

Agreement in RAN4#110bis - It is RAN4 understanding that UE only needs to follow NW configuration for measurement and validity check. - FG 39-x2 does not apply to any measurements during IDLE/INACTIVE but no included in configuration of R18 CA/DC setup improvement. - UE is not required to report carriers which are not configured by NW.

TS 38.133 clause 4.7.1 UE is not required to report for fast CA/DC setup from any measurements during IDLE mode for carriers which are not configured by higher layers.

However, RAN2 maybe have different understand based on their design. We can observe from the agreement blew. In RAN2’s view, measReselectionCarrierListNR is optional and UE should report any measurement it has if  measReselectionCarrierListNR is not configured [2].

Agreement in RAN2#125bis (R2-2404006) For reselection measurements: Confirm that the freq is optionally configured by the network, applicable to UE availability indication and UE reporting (doesn’t impact the actual measurements). If not configured, the UE reports what is has.

TS 38.331 clause 5.7.10.3 1> if the reselectionMeasurementReq is included in the UEInformationRequest: 2> if validatedMeasurementsReq is included in the UEInformationRequest and measReselectionValidityDuration is included in VarMeasReselectionConfig; 3> if measReselectionCarrierListNR is present in VarMeasReselectionConfig: 4> set the measResultReselectionNR in the UEInformationResponse message the valid NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig for each reported measurement; 3> else: 4> set the measResultReselectionNR in the UEInformationResponse message to any valid NR measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig; 2> else: 3> if measReselectionCarrierListNR is present in VarMeasReselectionConfig: 4> set the measResultReselectionNR in the UEInformationResponse message the NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig; 3> else: 4> set the measResultReselectionNR in the UEInformationResponse message to any NR measurement results, if available;

Based on the above analysis, we have some doubt that how to understand RAN4’s agreement on UE is not required to report carriers which are not configured by NW.

- Understanding 1: it only uses for the case that measReselectionCarrierListNR is configured and UE is not required to report carriers which are not in measReselectionCarrierListNR. If measReselectionCarrierListNR is not configured, UE will repoet any measurement results, if available.

- Understanding 2: if measReselectionCarrierListNR is not configured, UE will not repoet any measurement results.

Understanding 1 is algin with RAN2 and understanding2 is conflict with RAN2. therefore, we should confirm what  the UE behavior is if measReselectionCarrierListNR is not configured.

Observation 2: when measReselectionCarrierListNR is not present, there maybe disalignment between RAN2 and RAN4.

Proposal 5: RAN4 should confirm what the UE behavior is if measReselectionCarrierListNR is not present. UE will repoet any measurement results if available or UE will not repoet any measurement results.

2.3 Performance part

In RAN4#112 meeting, there was an issue that defined more test case for improvement on SCell/SCG setup delay and no consensus [3].

Issue 3-2-1: More test for Improvement on SCell/SCG setup delay < Way Forward > FFS the following option: - Proposal 1 (Nokia): - Introduce test cases with valid reporting for FR1 and FR2 also for the case when the UE is configured with EMR measurements.

For Improvement on SCell/SCG setup delay, we discussed in previous meetings and reached consensus that  defined three test cases to verify the UE behavior, one for EMR and two for non-EMR [4].

Issue 2-2: test purpose for solution based on existing measurement for validity check

Agreement:

- For EMR, TC will verify UE does not send measurement report which is measured exceeds X second before paging reception.

- For non-EMR, TC will verify

  - UE send measurement report which is measured within X second before [paging reception].

  - UE does not send measurement report which is measured exceeds X second before [paging reception] if it is feasible for the testing.

In our understanding, existing test cases are enough. For EMR, UE should pass the test case for reporting defined in Rel-16 and valid check defined in Rel-18. and similar cases also defined for non-EMR. As to the test case on reporting for non-EMR in Rel-18, it is because the consideration of cell reselection measurement results is a newly introduced feature in Rel-18 and has not been verified before. Differently, EMR has already completed testing on reporting in Rel-16, and we think it is not necessary to introduce testing on reporting for EMR.

Observation 3: different from non-EMR, EMR has already completed testing on reporting in Rel-16 and UE need to pass the test case defined in Rel-16 first.

Proposal 6: the test cases with valid reporting for FR1 and FR2 maybe not needed for the case when the UE is configured with EMR measurements.

Conclusions

- In this contribution, we put forward the following proposals and observations on L1/L2 triggered mobility and  improvement on SCell/SCG setup delay.

- Proposal 1: Early TCI state activation delay requirements to be defined for one or more TCI states for a single candidate cell, because one MAC-CE activates TCI states only for a single candidate cell.

Proposal 2: In early candidate cell’s TCI state activation delay for known TCI state case, the definiation of Tfirst-SSB should be applied to both inter-frequency L1 measurement with MG and inter-frequency L3 measurement with MG as LTM can be triggered by L1 or L3 measurement.

- Proposal 3: When TCI state activation MAC-CE or PDCCH order is sent for more cells than UE capability for fast processing, the cells for which the UE received TCI state activation MAC-CE or PDCCH order the most recently before cell switch command are the ones that are pre-processed.

Observation 1: for FR2 without L1 measurement report, cell switch delay could be reduced compared with legacy handover when TCI state is activated.

Proposal 4: cell switch delay requirements applicable to FR2 without L1 measurement report with the following conditions

- The target TCI state in the cell switch command is activated not more than [1280] ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or

- The target TCI state in the cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every [1280] ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB

Observation 2: when measReselectionCarrierListNR is not present, there maybe disalignment between RAN2 and RAN4.

Proposal 5: RAN4 should confirm what the UE behavior is if measReselectionCarrierListNR is not present. UE will repoet any measurement results if available or UE will not repoet any measurement results.

Observation 3: different from non-EMR, EMR has already completed testing on reporting in Rel-16 and UE need to pass the test case defined in Rel-16 first.

Proposal 6: the test cases with valid reporting for FR1 and FR2 maybe not needed for the case when the UE is configured with EMR measurements.

References

- R4-2413960, WF on NR mobility enhancements, MediaTek Inc.

- R2-2404006, [Post125bis][513][R18Mob] Idle/Inactive and Reselection Meas Reporting CR, Nokia

- R4-2413960, WF on Further NR mobility enhancements, MediaTek

- R4-2410280, WF on R18 Further NR mobility enhancement - part 2, Apple