3GPP TSG-RAN WG4 Meeting # 113                                R4-2418504

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.24.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Event triggered L1 measurement reporting

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to layer3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we will propose our views on RRM requirement for event triggered L1 reporting.

Discussion

According to the WID, RAN4 should discuss the impact on RRM requirements depends on the RAN1/2 specification introduced for Event triggered L1 measurement reporting.

The RRM impact due to event triggered L1 measurement reporting could include the followings:

- Capabilities for Support of Event Triggering and Reporting Criteria

- Event-triggered L1 reporting delay requirement and accuracy requirement

- Other potential impact of event-triggered L1 reporting

For the first bullet, some companies proposed that requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell in pervious meeting. In our understanding, it is necessary to update the requirements of Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS38.133 section 9.1.4 need to be updated to cover event triggered L1 measurement reporting.

Proposal 1: The requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS38.133 section 9.1.4 need to be updated to cover event triggered L1 measurement reporting.

For the second bullet, we should defined requirements for event triggered L1 measurement, including SSB based and CSI-RS based measurements. Since there has already been a sufficient foundation for SSB measurement in R18, we can start by discussing this part.

- Issue 1-9: L1 event triggered reporting delay - Candidate solutions: - Option 1: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point. (CTC, Samsung, ZTE) - Option 2: RAN4 to discuss how to consider the total measurement time if multiple cells are measured with different measurement period. (Xiaomi) - It’s possible that the periodicity of RS is different from each other and then the measurement period is different. How to define total measurement time for multiple cells is FFS. - Option 3: (CMCC) - for SSB based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements. - The delay is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index, which are the L1-RSRP measurement period defined for Rel-18 LTM. - for CSI-RS based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements, and L1 measurement reporting accuracy requirements - Option 4: Reporting delay is defined as one L1-RSRP measurement period on target cell. (Apple) - Option 4a: The event triggered measurement reporting delay, shall be less than LTM L1-RSRP measurement period based on SSB or CSI-RS. (Samsung) - Option 5: SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (Nokia) - Option 6: RAN4 to discuss and define the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion). (Ericsson)

In legacy L3 event triggered reporting, the measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. The event triggered measurement reporting delay for intra-frequency L3 measurement is defined as Tidentify intra with index, T identify intra without index, Tidentify_inter_without_index and Tidentify_inter_with_index。Take intra-frequency for an example, we can observed from current spec that the requirement for event triggered measurement reporting delay is applied for measurement without L3 filtering and an additional delay can be expected when L3 filtering is used. Event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra ms or T identify intra without index = TPSS/SSS_sync_ntra + T SSB_measurement_period_intra + TSSB_time_index_intra ms. Meanwhile, all reports shall meet accuracy defined in section 10.

9.2.4.3 Event Triggered Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled. The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

For accuracy requirements, we have reached consensus last meeting that the reported L1-RSRP measurement result shall meet R18 LTM L1-RSRP accuracy requirements for SSB based L1 RSRP event triggered reporting. And for CSI-RS based L1-RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R19 CSI-RS based L1-RSRP accuracy requirements for LTM. In this meeting, we should discuss how to define the requirement for event triggered measurement reporting delay.

Proposal 2: The principle of L3 event triggered reporting RRM requirements can be used as baseline to cover L1 event triggered reporting.

In Rel-18 LTM, we defined SSB based L1-RSRP measurement delay in clause 9.14 and 9.15. For L1 event triggered reporting, the reporting conditions are evaluated based on L1 measurement, so L3 cell identification period should be replaced by L1 measurement period in event triggered measurement reporting delay. That’s to say, TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap can be used as the starting point for the event triggered L1 measurement reporting delay.

Observation 1: For L1 event triggered reporting, the reporting conditions are evaluated based on L1 measurement, and L3 cell identification period should be replaced by L1 measurement period in event triggered measurement reporting delay.

Proposal 3: for the event triggered L1 measurement reporting delay, TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap can be used as the starting point for SSB based.

- Issue 1-5: filtering assumption in event evaluation - Candidate solutions: - Option 1: Wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report (MTK, Apple, HW, Nokia, E///, Qualcomm, CATT). - Option 1a: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT) - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. - Otherwise, there is no impact on RAN4. - Option 2: For filtering, we think it is better to wait the conclusion of RAN1. If it is up to UE implementation, it is unnecessary to define the related requirements. (CTC) - Event triggered reporting delay is up to whether the filtering is needed, when filtering is used an additional delay can be expected. - In case without filtering, we think current LTM L1-RSRP measurement delay can be as baseline. - Option 3: no need to define additional SSB based L1 measurement delay requirements with filtering, since the agreement about filtering in RAN2 is about filtering with filter coefficient as defined in Clause 5.5.3.2, TS38.331 or about Time-to-Trigger (TTT), neither of them have impact on L1 measurement delay requirements. (CMCC)

In previous meeting, RAN2 assumes filtering of the L1 measure results is needed for LTM event and it’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. For event triggered reporting for L3 measurement, an additional delay can be expected in event triggered measurement reporting delay when L3 filtering is used. If L1 filtering is used, there maybe an impact on report delay requirement. Based on this, RAN4 should follow up on other details of the event evaluation and report triggering. Therefore, we should wait for RAN1/RAN2’s progress before defining requirements.

Proposal 4: Wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report.

- Issue 1-7: beam level or cell level measurement result for new LTM event evaluation - Candidate solutions: - Option 1: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation. (Xiaomi, CMCC, Apple, Samsung) - Issue 1-10: where to capture event triggered L1 report requirement - Candidate solutions: - Option 1: new requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency. (CATT, Apple, vivo, HW)

In RAN2 #126 meeting, it was agreed that for event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. In RAN2 #127 meeting, it was agreed that beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. It is proposed to follow RAN2 agreements to only use beam level measurement result for LTM event evaluations.

Proposal 5: follow RAN2 agreements, only use beam level measurement result for LTM event evaluations.

RAN4 needs to define measurement reporting requirements for event triggered L1 measurement reporting. For intra-frequency, new requirements shall be introduced in clause 9.14.3, while for inter-frequency new requirements shall be introduced in 9.15.3.

Proposal 6: new requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency.

Conclusions

In this contribution, we put forward the following proposals on R19 L1/L2 triggered mobility.

Proposal 1: The requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS38.133 section 9.1.4 need to be updated to cover event triggered L1 measurement reporting.

Proposal 2: The principle of L3 event triggered reporting RRM requirements can be used as baseline to cover L1 event triggered reporting.

Observation 1: For L1 event triggered reporting, the reporting conditions are evaluated based on L1 measurement, and L3 cell identification period should be replaced by L1 measurement period in event triggered measurement reporting delay.

Proposal 3: for the event triggered L1 measurement reporting delay, TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap can be used as the starting point for SSB based.

Proposal 4: Wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report.

Proposal 5: follow RAN2 agreements, only use beam level measurement result for LTM event evaluations.

Proposal 6: new requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple