3GPP TSG-RAN WG4 Meeting # 113                                R4-2418505

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.24.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on CSI-RS based L1-RSRP measurement

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to layer3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we will propose our views on RRM requirement for CSI-RS based L1-RSRP measurement.

Discussion

In previous meeting, we have discussed the RRM scope of CSI-RS based L1-RSRP measurement and reached consensus that RAN4 should discuss at least measurement delay, measurement reporting requirements, measurement restrictions and scheduling availability for CSI-RS based L1-RSRP measurement on LTM candidate cells, which is considered in high level and same as the procedure of study on SSB based L1-RSRP measurement on candidate cells.

Similar as SSB based L1-RSRP measurement on neighbor cells, RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells. And we discussed the known cell definition in CSI-RS based L1 measurement last meeting.

For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition

For the first bullet, we need to clarify whether the L3 measurement referred to here pertains only to SSB-based measurements or also encompasses CSI-RS based measurements. Logically, our understanding is that both are included. However, from the perspective of the measurement procedure, if a CSI-RS L3 measurement is carried out, it necessarily follows that an SSB-based L3 measurement has already been performed, because the CSI-RS L3 measurement is directed towards the associated SSB. At its core, it still amounts to conducting an SSB-based L3 measurement. In other words, whether it's restricted to SSB-based L3 measurement only or applicable to both SSB-based L3 measurement and CSI-RS based L3 measurement, we understand that it makes no difference to the first bullet.

Observation 1: whether it's restricted to SSB-based L3 measurement only or applicable to both SSB-based L3 measurement and CSI-RS based L3 measurement, it makes no difference to the first bullet.

For the second bullet, whether to keep L1 measurement depends on whether RAN1/RAN2 has established QCL relation between CSI-RS and SSB. If yes, it is reasonable to consider SSB from the target cell configured for L1 measurement. After checking, there is no conclusion on QCL relation between CSI-RS and SSB in RAN1/RAN2. we prefer to keep L1 measurement for FR2-1 with bracket and wait for RAN1/RAN2’s further progress.

Observation 2: for measurement resource, there is no conclusion on QCL relation between CSI-RS and SSB in RAN1/RAN2.

For the third bullet, we prefer to use detectable which is used for R18 LTM.

The cell is considered as known if the following conditions are met in this requirement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2 remains detectable according to the cell identification requirements specified in clause 9.2. Otherwise, the cell is unknown.

Based on the aforementioned analysis, we propose that the definition of a known cell be as follows:

  - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

  - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2.

    - L1 measurement in the 2nd bullet is not needed for FR1.

    - keep L1 measurement for FR2-1 with bracket in the 2nd bullet and wait for RAN1/RAN2’s progress.

  - The CSI-RS from the target cell configured for L1 measurement remains detectable.

Proposal 1: The cell is considered as known if the following conditions are met in this requirement:

  - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

  - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2.

    - L1 measurement in the 2nd bullet is not needed for FR1.

    - keep L1 measurement for FR2-1 with bracket in the 2nd bullet and wait for RAN1/RAN2’s progress.

  - The CSI-RS from the target cell configured for L1 measurement remains detectable.

In R18 LTM, periodic reporting, semi-persistent reporting and aperiodic reporting are considered. For CSI-RS based measurement, RAN1 has reached consensus that CSI-RS on periodic measurement is supported in August, and aperiodic and semi-persistent measurement are supported in October. Therefore, we should define the requirements of CSI-RS on periodic measurements, aperiodic and semi-persistent measurement.

Agreement For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported

Proposal 2: RAN4 should define CSI-RS based L1 measurement requirements for periodic, aperiodic and semi-persistent CSI-RS.

In the last meeting, we have discussed the definiation of intra-f and inter-f measurement for CSI-RS based L1 RSRP measurement. And there is no consensus.

Issue 2-14: definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement (already discussed during online session) - Candidate solutions: - Option 1: Following CSI-RS based L3 measurement (CTC, CMCC, Apple, Samsung, OPPO, [Nokia?]) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: For definition of intra-frequency and inter-frequency, suggest to wait for more progress in other WGs. (Xiaomi) - Option 3: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy. (CATT) - One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell. - Option 4: Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency. (vivo) - Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules - No additional definition of intra-F/inter-F for CSI-RS based L1 measurements. - Option 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell. (HW) - It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

There are some options on whether to define intra-frequency and inter-frequency for CSI-RS based L1 measurement. For option1, it is same as CSI-RS based L3 measurement, which is aglin with the previous discussion for measurement. For option 2, proponent suggests to wait for other groups. RAN1 has the agreement in last meeting that there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement from RAN1 perspective. In our understanding, RAN1 prefers not to distinguish the inter-f and intra-f measurement.

Option 3 is same as option1, only requiring some application conditions. For option 4, there is no need to discuss the definiation of intra-frequency measurement and inter-frequency measurement, just follow the associated SSB.

In our understanding, we prefer to following CSI-RS based L3 measurement.

Proposal 3: Prefer to define the intra-frequency and inter-frequency for CSI-RS based L1 measurement and a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

      - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

      - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

        - It is applied for SCS = 60KHz

      - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

Meanwhile, for CSI-RS based L1-RSRP measurement, we discussed the measurement type, mainly study whether to cover with and without gap.

Issue 2-3: CSI-RS based L1 measurement outside active BWP or with measurement gap - Candidate solutions - Option 1: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. (MTK, QC, vivo, OPPO, Nokia) - Option 1a: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported. (Xiaomi) - Option 1b: RAN4 to define intra-frequency CSI-RS measurement requirements first. Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap. (Nokia) - Option 2: RAN4 at least support the following measurement types for CSI-RS based L1 measurement. (CATT) - Without gap - With gap - Option 2a: for CSI-RS based LTM, it is proposed to consider intra-frequency measurement, inter-frequency measurement without gap and inter-frequency measurement with gap, which are aligned with Rel-18 SSB based LTM. (CMCC) - Option 2b: prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. (Apple, Samsung, HW, OPPO, CATT) - Option 2c: CSI-RS based L1 measurements for candidate cells shall support - Measurement within active BWP, Measurements outside active BWP. (E///, ZTE)

In our views, both with and without gap should be covered due to this is for neighbor cell measurement and measurement with gap is a typical scenario. Some companies proposed the concern. For serving cell, there is no specific restriction in time domain on configurations of CSI-RS resources for L1-RSRP measurement. So, it’s difficult to ensure all the CSI-RS of the same frequency layer are within a single gap window.

For this issue, we think it can be solved by using the same principle for CSI-RS based L3 measurement that all CSI-RS resources in the same MO are confined in the same 5ms window. However, if we consider perform SSB based L3 measurement and CSI-RS based L1 measurement in FR2, it is difficult to use one gap to cover both SSB based L3 or L1 measurement and CSI-RS based L1 measurement, so concurrent gapshall be used.

Proposal 4: Support CSI-RS based L1-RSRP measurement within active BWP and outside active BWP, and same principle can be used as start point, e.g., defined a window to restrict the CSI-RS within gap which is used for CSI-RS based L3 measurement.

Issue 2-12: RTD assumption - Further discuss based on the following assumption: - RTD is the received time difference among CSI-RS and SSB from different cells. - Alt 1: define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP. - Alt 2: only define requirements for RTD<CP. - NW shall only enable this when RTD<CP is guaranteed.

As to RTD assumption, RTD>CP is a typical case for neighbor cell measurement. It’s reasonable to consider this case in CSI-RS based L1-RSRP measurement. We prefer to define requirements for both RTD<CP and RTD>CP. Meanwhile, the same method as SSB based L1-RSRP measurement can be used, i.e., introduce an optional UE capability for RTD>CP. However, as mentioned by UE vendor, it’s complicated to use different FFT for CSI-RS based measurement. Consider UE implementation and RTD>CP has been supported in SSB based measurement, it’s ok for us to deporitize the discussion on RTD>CP.

Proposal 5: Prefer to define requirements for both RTD<CP and RTD>CP.

Issue 2-17: CSI acquisition on candidate cell(s) before or during LTM cell switch - Candidate solutions: - Option 1: Wait for more RAN1 progress to start the discussion on CSI acquisition in RAN4. (MTK, QC) - Issue 2-18: accuracy and side condition for CSI-RS based L1-RSRP measurement - Candidate solutions: - Option 1: The existing side condition for CSI-RS based L1-RSRP measurement for serving cell is used as the starting point. (Apple) - Option 1a: CSI-RS based L1-RSRP accuracy of the serving cell can be applied for CSI-RS based L1-RSRP of the candidate neighbour cells. (HW) - Option 2: RAN4 to discuss the CSI-RS accuracy requirements during performance part. (E///) - Issue 2-19: CSI-RS based L1-SINR measurement - Candidate solutions: - Option 1: RAN4 need to discuss whether to define the requirements for CSI-RS based L1-SINR measurement. (CATT) - RAN4 to wait for more RAN1 input. - Option 2: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement (Apple, Nokia)

For the issues about CSI acquisition on candidate cell(s) before or during LTM cell switch and CSI-RS based L1-SINR measurement, they are remain open in RAN1. Therefore, we should wait for more progress from other group before defining requirements.

Conclusion There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells

Proposal 6: Wait for more RAN1 progress to start the discussion on CSI acquisition in RAN4.

Proposal 7: focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

As to the accuracy and side condition for CSI-RS based L1-RSRP measurement, we prefer to reuse CSI-RS based L1-RSRP accuracy of the serving cell. In our understanding, one CSI-RS resource is used for both serving and neighboring cell measurements, e.g., CSI-RS resources for L1-RSRP measurement on neighbor cell for UE#1 are in fact the resources for L1 measurement of serving cell for UE#2. Meanwhile, measurement accuracy is performance requirement, we can discuss it in performance part.

Proposal 8: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline.

Conclusions

In this contribution, we put forward the following proposals on R19 L1/L2 triggered mobility.

Observation 1: whether it's restricted to SSB-based L3 measurement only or applicable to both SSB-based L3 measurement and CSI-RS based L3 measurement, it makes no difference to the first bullet.

Observation 2: for measurement resource, there is no conclusion on QCL relation between CSI-RS and SSB in RAN1/RAN2.

Proposal 1: The cell is considered as known if the following conditions are met in this requirement:

  - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

  - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2.

    - L1 measurement in the 2nd bullet is not needed for FR1.

    - keep L1 measurement for FR2-1 with bracket in the 2nd bullet and wait for RAN1/RAN2’s progress.

  - The CSI-RS from the target cell configured for L1 measurement remains detectable.

Proposal 2: RAN4 should define CSI-RS based L1 measurement requirements for periodic, aperiodic and semi-persistent CSI-RS.

Proposal 3: Prefer to define the intra-frequency and inter-frequency for CSI-RS based L1 measurement and a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

      - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

      - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

        - It is applied for SCS = 60KHz

      - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

Proposal 4: Support CSI-RS based L1-RSRP measurement within active BWP and outside active BWP, and same principle can be used as start point, e.g., defined a window to restrict the CSI-RS within gap which is used for CSI-RS based L3 measurement.

Proposal 5: Prefer to define requirements for both RTD<CP and RTD>CP.

Proposal 6: Wait for more RAN1 progress to start the discussion on CSI acquisition in RAN4.

Proposal 7: focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

Proposal 8: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple