3GPP TSG-RAN WG4 Meeting # 113                                R4-2418507

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.5.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on intra-band non-collocated EN-DC and NR-CA

Document for:	Approval

Introduction

For intra-band non-collocated EN-DC/NR-CA deployment, RAN4 has diasussed and achieved agreements on MRTD/MTTD requirements, interruption requirements and scheduling restriction requirements for Type 2 UE in Rel-18. In Rel-19, we should focus on Type 4 UE accroding to WID [1] to better support the intra-band non-collocated EN-DC/NR-CA deployment scenarios.

The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

In this contribution, we will provide the discussion on RRM impacts for intra-band non-collocated EN-DC/NR-CA, including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption.

Discussion

Type 4 UE is assumed to have separate Rx chains for different CCs, which is similar as Type 2 UE. Different from Type 2 UE, Type 4 UE is up to 4-layer DL MIMO per CC. Therefore, the main difference between Type 2 and Type 4 is the supported MIMO layers per CC.

In the previous meeting, we have disscussed the requirements of Type 4 capable UE, including MRTD and MTTD, Interruption requirements, SCell activation delay requirements, Scheduling availability and SCell BFD/CBD requirements. And there is no distingush between Type 2 UE and Type 4 UE from RRM requirements.

In the last meeting, we have the agreement on some requirements with note that the specific UE capability and network signalling can wait for the RF session conclusion.

<Agreement> Issue 1-4: Interruption requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition Agreement: - When Type 4 capable UE operates in collocated condition, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied: - Interruption at SCell addition/release - Interruptions at SCell activation/deactivation - Interruptions during measurements on deactivated SCC Note: Some clarifications are needed with regard to the UE capability and network signalling. The specific UE capability and network signalling can wait for the RF session conclusion. <Agreement> Issue 1-5: SCell activation delay requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition Agreement: - When Type 4 capable UE operates in collocated condition, SCell activation delay defined for type 2 UE in collocated scenario in R18 can be applied. Note: Some clarifications are needed with regard to the UE capability and network signalling. The specific UE capability and network signalling can wait for the RF session conclusion. <Agreement> Issue 1-6: Scheduling availability for Type 4 capable UE when UE operates with shared RF chain in collocated condition Agreement: - When type 4 capable UE operates in collocated scenario, scheduling availability defined for type 2 UE in collocated scenario in R18 can be applied. Note: Some clarifications are needed with regard to the UE capability and network signalling. The specific UE capability and network signalling can wait for the RF session conclusion.

In Rel-18, RAN4 has the agreement on the the impacted RRM requirement due to new BS signalling, we think it also can apply to Type 4 capable UE.

RAN4#109 Issue 1-3: the impacted RRM requirement for UE supporting [intraBandNRCA-NonCollocated-r18] due to new BS signalling Agreement: The impacted RRM requirement applicability for UE supporting [intraBandNRCA-NonCollocated-r18] due to new BS signalling - MRTD/MTTD, interruption, [SCell activation delay, BFD/CBD, scheduling restrictions and measurement restrictions requirements.] Issue 1-4: the impacted RRM requirement for R18 UE supporting interBandMRDC-WithOverlapDL-Bands-r16 due to new BS signalling Agreement: The impacted RRM requirement applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 due to new BS signalling - MRTD/MTTD and interruption requirements.

Proposal 1: If RF makes progress in network signaling and UE capability, the following RRM requirements may need to be modified for Type 4 capable UE:

- MRTD/MTTD and interruption requirements for EN-DC

- MRTD/MTTD, interruption, [SCell activation delay, BFD/CBD, scheduling restrictions and measurement restrictions requirements] for NR CA.

As to network signaling and UE capability, discussions are currently underway in RF section. In R18 non-collocated WI, two new BS signalling are introduced, i.e., nonCollocatedTypeMRDC or nonCollocatedTypeNR-CA. When indicated, it is a Type 1 UE, and when not indicated, it is a Type 2 UE. We think the similar principle can be inherited for R19.

In our understanding, there are different ways to complete the design, such as designing new signaling or extending previous signaling to cover the current required capabilities. However, it should discuss in RF and we need to wait for the RF session conclusion.

Observation 1: For the design of BS signalling, there are different ways to complete the design, such as designing new signaling or extending previous signaling to cover the current required capabilities.

Proposal 2: How to define requirements based on BS signaling in the protocol depends on the specific design of RF and RAN2.

Conclusions

In this contribution, we put forward the following proposals:

Proposal 1: If RF makes progress in network signaling and UE capability, the following RRM requirements may need to be modified for Type 4 capable UE:

- MRTD/MTTD and interruption requirements for EN-DC

- MRTD/MTTD, interruption, [SCell activation delay, BFD/CBD, scheduling restrictions and measurement restrictions requirements] for NR CA.

Observation 1: For the design of BS signalling, there are different ways to complete the design, such as designing new signaling or extending previous signaling to cover the current required capabilities.

Proposal 2: How to define requirements based on BS signaling in the protocol depends on the specific design of RF and RAN2.

References

- RP-240838, New WID on Support of intra-band non-collocated EN-DC/NR-CA, Moderator (Apple)

- R4-2416850, WF on RRM requirements for NonCol_intraB_ENDC_NR_CA_Ph2, Huawei