3GPP TSG-RAN WG4 Meeting # 113                                R4-2418509

Orlando, US, 18th – 22nd November, 2024

Agenda item:	5.18.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RRM requirements for multi-carrier enhancements

Document for:	Approval

Introduction

In the last meeting, we discussed whether the UE shall report the same value between RAN4 features 6-3 and 38-9 and there were no consensus. In this contribution, we want to provide our view on this issue.

Discussion

According to the agreed WF[1], there is a leftover issue should be discussed. This issue is about UE capability for switching between non-dormant and dormant BWPs triggered by the new DCI format 0-3 and 1-3 was introduced.

Based on the LS from RAN2, RAN4 should discuss whether the UE shall report the same value between RAN4 feature 6-3 and 38-9.

RAN2 discussed the two options and concluded to down-select approach 2 because Approach 1 is a non-backward compatible change from RAN2 perspective. RAN2 introduced a field bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 corresponding to RAN4 feature 38-9, and a field scellDormancyWithinActiveTime-DCI-0-3-And-1-3-r18 corresponding to its prerequisite feature (RAN1 feature 49-9). It is up to RAN4 whether the UE shall report the same value between RAN4 features 6-3 and 38-9.

Based RAN4’s discussion in RAN4#111 meeting, RAN2 concluded to down-select approach 2 that introduce a new Rel-18 UE capability. And define prerequisite feature group as 49-9 and add “with DCI 0-3/1-3” in feature group.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-9 | Dormant BWP switching on multiple CCs RRM requirements with DCI 0-3/1-3 | Incremental delay for BWP switch processing on additional SCells in DCI based simultaneous dormant BWP switching on multiple SCells | 49-9 |

In legacy, we just have feature 6-3 and the value of D is same for each additional CC. Currently, Some companies think that it’s necessary to clarify whether different D values are allowed on additional CCs related to different UE capability reporting.

The definition of D has been copied as follows:

UE shall finish BWP switch within the time duration TMultipleBWPswitchDelay + Y, which is defined as: TMultipleBWPswitchDelay = TBWPswitchDelay + D*(N-1) Where: - TBWPswitchDelay is the BWP switching delay on single CC defined in Table 8.6.2-1 depending on UE capability bwp-SwitchingDelay [2]. TBWPswitchDelay shall be based on the smallest SCS among SCS of all involved CCs before and after BWP switch. If the BWP switch on multiple CCs results in the change of the SCS on any CC among involved CCs, TBWPswitchDelay shall be based on the smallest SCS among all SCS values of all involved CCs. - D is the incremental delay for each additional CC involved in simultaneous BWP switch and depends on UE capability bwp-SwitchingMultiCCs-r16 [TS 38.306, 14] for switching between non-dormant BWPs, and bwp-SwitchingMultiDormancyCCs-r16 or bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 for switching between non-dormant and dormant BWPs.

Legacy capability can be observed blew, e.g., feature 6-3.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 6. LTE_NR_DC_CA_enh | 6-3 | Dormant BWP switching on multiple CCs RRM requirements | Incremental delay for BWP switch processing on additional SCells in DCI based simultaneous dormant BWP switching on multiple SCells | RAN1 feature 18-4 or 18-4a |
| 18. MR-DC/CA enhancement | 18-4 | SCell dormancy indication within active time | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_1/1_1 | 6-5 |
| 18. MR-DC/CA enhancement | 18-4a | SCell dormancy indication outside active time | Support for SCell dormancy indication sent outside the active time on PCell with DCI format 2_6 | 19-1 |

In our understanding, this is not to introduce additional delay, but the legacy delay is also applicable to the new DCI format scenario. For TS 38.133, just to add the new IE if go with approach 2. If it is up to UE whether to report the same or different values for the Rel-18 capability and the legacy capability, maybe there are some modifications on requirements. We perfer not to do any change in the existing requirements and the same vlaue of D for the Rel-18 capability and the legacy capability.

Proposal: perfer not to do any change in the existing requirements and the same vlaue of D for the Rel-18 capability and the legacy capability.

Conclusions

In this contribution, we discuss the RRM requirements for Rel-18 Tx switching and propose that:

Proposal: perfer not to do any change in the existing requirement and the same vlaue of D for the Rel-18 capability and the legacy capability.

References

- R4-2416878, WF on UE report of incremental BWP switch delay for multi-carrier enhancements, NTT DOCOMO