3GPP TSG-RAN4 Meeting  #113	R4-2418532

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.6.3

Source:	Apple

Title:	On UE RF requirements for fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

At the last RAN4 meeting, a WF was agreed that captures the agreements and open issues [2]. In this contribution, we share our views on the open issues.

# 2	Discussion

Requirements for UE supporting fragmented carriers with shared RF chains

In the WF, the point on how to specify requirements for UE supporting fragmented carriers with shared RF chains is in brackets, as shown below

- [Adjusted requirements for fragmented carriers, if any, shall be separated from the existing requirements]

It is important to have a common understanding on it. As is well understood, the current RF requirements for intra-band non-contiguous CA were derived based on a partially shared RF chain architecture captured in TR 36.823. Given the blocker in the gap, it is evident that a shared RF chain architecture cannot meet such requirements. That is the reason why the SI scope explicitly contains the bullet “Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking.”

To be clear, whatever the outcome of the SI is, the existing requirements should still apply to UEs using partially shared RF chain architecture to support fragmented carriers (or intra-band non-contiguous CA). With that, we think it is reasonable to say the adjusted requirements for UE supporting fragmented carriers with shared RF chains should be separated from existing requirement. Of course, there should be a UE capability or other means to indicate such UEs.

Proposal 1: It is proposed to agree that adjusted requirements for UEs supporting fragmented carriers with common RF chain architecture, if any, shall be separated from the existing requirements. A TP is needed to capture this agreement in the TR.

Receiver sensitivity

In current CA receiver sensitivity requirement, there is ΔRIBNC to allow relaxation due to TX noise at RX band, i.e., TX noise at SCC, which assumes UE architecture of separate RF chains.

Fig. 1 below (copied from [3]) shows for CA_n25(2A), PCC UL is closer to SCC in the CA receiver sensitivity requirement setting than the usual TX and RX in a single carrier case.

Figure 1. CA receiver sensitivity setting for band n25

With common RF chain architecture, the TX noise at SCC from the PCC remains the same as the analog BB filter cannot provide any help. However, besides TX noise, there is TX leakage at TX band that may benefit from analog filtering before it goes into digital BB. In this case, since common RF chain architecture uses a wide analog BB filter to cover the overall frequency span from the lower edge of CC1 to the upper edge of CC2, as compared to the separate RF chain architecture where two separate narrow BB filters to cover CC1 and CC2, there is less filter rejection. As a result, the SCC performance may take some hit. This performance hit is expected to be less severe than the TX noise at SCC. More detailed analysis is needed to quantize the SCC performance degradation.

In addition, we would like to propose to apply some relaxation like ΔRIBNC for PCC receiver sensitivity. This is because after the down-conversion, the mirror frequency of PCC UL would be close to PCC DL and become a blocker. With the wider analog BB filtering covering PCC and SCC, the filter rejection is reduced compared to single-CC narrowband filtering.

Proposal 2: ΔRIBNC for SCC may need to be updated to reflect the filter rejection reduction of common RF chain compared to separate RF chains for each intra-band non-contiguous CA band combination. Furthermore, ΔRIBNC needs to apply to PCC too, although the value of ΔRIBNC can be different for PCC and SCC.

In-gap blocker power level

The following is captured in the WF:

#### Issue 2-2: Study on power spectral density difference between carriers of co-located adjacent channel operators Agreement: To evaluate acceptable [PSD] difference from UE side between wanted signal and un-wanted signal based on the deployment where BSs from different operators are collocated but their radio units (RUs) are not shared. - The [PSD] difference means the difference between unwanted signal and the wanted carrier with the lower PSD - Start evaluation with wanted signal level currently existing ACS requirements as baseline and to see the performance difference

Because of the impact on both ACS and in-band blocking requirement, it is justified to limit the blocker power level. Furthermore, the blocker would impact RX dynamic range. For example, consider the extreme case where the gap is 13 times wide than a 5MHz channel, like CA_n3(2A). If we assume the blocker is 65MHz wide and there is 6dB PSD difference between the blocker and one of the two non-contiguous CCs, the blocker power is 17dB higher than the CC power, which would affect the ADC dynamic range requirement and the receiver performance. Another aspect is that due to the wide analog BB filter, the rejection to the TX signal is lower, leading to a stronger TX leakage that will further impact the ADC dynamic range. Therefore, it is necessary to limit not only the PSD difference between the blocker and CCs, but also the gap width between the two CCs. That said, if we follow the ACS and in-band blocking test configurations, in which the blocker BW is either 5MHz or the same as wanted signal BW, we may not need to limit the gap width. Some UE capability reporting could also be considered for UE to report its capability in handling blocker level and the maximum frequency span per shared RF chain. Note also that it was agreed in the SID that there is up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs.

Proposal 3: It is proposed to limit the PSD difference between the blocker and the CC with the lower PSD to [6dB].

Another issue is depending on the gap width, there could be more than one blocker in the gap. An alternative to limiting PSD difference and gap width is to consider if it is feasible to define a total aggregated power of all blockers in the gap. This is because there could be quite a few combinations of blocker power and gap width. As a result, it may be complicated to specify in standards under which case the UE is expected to enable common RF chain, with all possible gap width and number of blockers.

Proposal 4: It is proposed to consider as an alternative if it is feasible to define a total aggregated power of all blockers in the gap between CC1 and CC2.

Image rejection requirement

Currently there is no requirement on image rejection for NR. On the need of a rejection level of the image caused by the in-gap interferer, our current view is if PSD of blocker is limited, the image rejection requirement can be indirectly ensured. Since any new requirement would incur testing overhead, it is not preferred to specify a new image rejection requirement.

Proposal 5: It is not preferred to specify a new image rejection requirement. Instead, the blocker PSD relative to PSD of CCs should be limited, as in Proposal 3.

Other constraints

Besides the necessary adjustments to requirements, we propose to clearly specify the constraints:

- To ease the receive design when common RF chain is used, it is preferred to have the same SCS for both CCs. We think this can be reasonable since both CCs are in the same band.

- The two CCs are co-located which ensures good time alignment of the two CCs at the UE receiver. This is already assumed in the SID.

Proposal 6: It is proposed to consider only same SCS for both CCs to ease receiver design for the common RF chain.

# 3	Conclusions

In this contribution, we make the following proposals and observations.

Proposal 1: It is proposed to agree that adjusted requirements for UEs supporting fragmented carriers with common RF chain architecture, if any, shall be separated from the existing requirements. A TP is needed to capture this agreement in the TR.

Proposal 2: ΔRIBNC for SCC may need to be updated to reflect the filter rejection reduction of common RF chain compared to separate RF chains for each intra-band non-contiguous CA band combination. Furthermore, ΔRIBNC needs to apply to PCC too, although the value of ΔRIBNC can be different for PCC and SCC.

Proposal 3: It is proposed to limit the PSD difference between the blocker and the CC with the lower PSD to [6dB].

Proposal 4: It is proposed to consider as an alternative if it is feasible to define a total aggregated power of all blockers in the gap between CC1 and CC2.

Proposal 5: It is not preferred to specify a new image rejection requirement. Instead, the blocker PSD relative to PSD of CCs should be limited, as in Proposal 3.

Proposal 6: It is proposed to consider only same SCS for both CCs to ease receiver design for the common RF chain.

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

R4-2417201, "WF on fragmented DL carriers study," MediaTek

RP‑241257, "Views on SI of Fragmented Carriers in Rel-19," Huawei, HiSilicon