# 3GPP TSG-RAN WG4 Meeting #113	R4-2418536

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.15.2.1

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#112, which includes open issues for further discussion. In this contribution, we continue the discussion for FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor.

# Discussion

Assumption on searchers

Issue 1-1-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656. - Option 1 (CATT, Apple, Samsung, ZTE, QC, NTT DCM): For Multi-Rx based L3 measurement, 1 searcher is assumed for single carrier. - Option 2 (vivo, LGE, OPPO): For Multi-Rx based L3 measurement, 2 searcher is assumed for single carrier. - Option 3 (CTC): Searcher occupied by single carrier is depended on UE implementation, 1 searcher or 2 searchers can be occupied by the single carrier. - Option 4 (HW): RAN4 to skip the discussion about the searcher assumption to support FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. And whether UE can support this feature can be considered with UE capability discussion. [Moderator]: FFS on option 1 and option 2.

To derive the requirements, it is necessary to make an assumption on the number of searchers. Therefore, it is better to focus on the first two options, as recommended by Moderator. We can see the rationale behind Option 1 or Option 2. While Option 2 is feasible, we slightly prefer Option 1 as it is forward compatible.

Proposal 1: To consider forward compatibility where the specified requirement can apply to CA case, Option 1, i.e., one searcher assumption, is slightly preferred.

SSB processing delay/time

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC - Option 1 (CATT, Apple, CTC, Nokia): if option 1 is adopted in issue 1-1-3, no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity - Option 1a (CTC, Intel): UE baseband processing capabilities need to be considered. - Option 1b (Nokia): - If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms - Option 2 (vivo): No need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity. - Option 3 (NTT DCM):SSB processing delay/time for this BSF optimization should be same as the legacy requirements because the BSF optimization should be achieved by simultaneously measurement of two spatial signal quality for one SSB. - Option 4 (QC): Proposal 3: In the handover delay equation, SSB processing time (2 ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - With above proposal, FR2 – FR2 HO timeline takes the following form (changes from legacy HO timeline is shown in blue): - Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + M * Tmargin ms - Where, - M is the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Tmargin is time for SSB post-processing. Tmargin can be up to 2ms. - Tsearch is the time required to search the target cell when the handover command is received by the UE. If the target cell is a known cell, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = ceil(N/M)* Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = ceil(N/M)*3* Trs ms. N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. Proposal 4: SSB processing time does not need to be introduced to handle optimized Rx BSF in following scenarios. - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection. - Option 5 (Ericsson): - Reflecting different assumptions of searcher number, processing time is specified differently for such two configurations. - For single carrier configuration in multi-Rx reception, extra processing time=0. - For CA/DC configuration in multi-Rx reception, if applicable, extra processing time could be reviewed after completing single carrier requirements. - For single carrier configuration, no extra processing time. For CA/DC configuration, if applicable, RAN4 to check the necessity of extra processing time for handover interruption time. - No extra processing time for cell identification delay. [Moderator]: FFS the following summarized options. (CA/DC is low priority, and therefore CA/DC based options are not reflected below): - For scenario 1/2/3/4 - Option 1: no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms - Option 2: no need to add additional processing time in the delay requirement. - For scenario 5 - Option 1: no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity - Option 2: In the handover delay equation, SSB processing time (2 ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - For scenario 7 - Option 1: no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity - Option 2: no need to add additional processing time in the delay requirement.

As pointed out in our previous contribution, when both panels are activated to make measurements, the measurement samples from each panel can be processed sequentially if only one searcher is assumed. Since the SMTC periodicity is 5ms or longer, it is possible for the UE to process both samples (one from each panel) within one SMTC periodicity. Therefore, it is OK to only consider some additional processing time after the last SMTC periodicity.

In our understanding, Option 4 is based on the fact that in current requirements, only handover delay requirement has explicitly considered time for SSB post-processing in the term Tmargin. However, we still hold the view that some additional processing time after the last SMTC periodicity is needed to ensure UE does have the needed time to process the measurement samples.

Proposal 2: It is possible for the UE to process both samples (one from each panel) within one SMTC periodicity. Therefore, it is OK to only consider some additional processing time after the last SMTC periodicity, i.e., Option 1.

Power consumption issue (including conditions to trigger UE using FBS for L3 measurement)

Issue 1-2-5: FFS: Power consumption issue (including conditions to trigger UE using FBS For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details.

It is reasonable to consider UE location and/or mobility state as a trigger for activation/deactivation of FBS. Besides these two factors, UE QoS should be considered too. For example, if a serving cell cannot provide good QoS such as data rates or latency to a UE for a certain service, the network may need to hand over the UE to another cell even though there is no urgent need to trigger handover due to mobility or signal quality degradation. Meanwhile, UE power consumption should also be considered.

Proposal 3: The triggering conditions should include UE power consumption, UE location, UE mobility and UE QoS requirements.

Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

### Issue 1-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF - Option 1 (Xiaomi): - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Option 2 (CATT, vivo, CMCC, Apple, ZTE, CTC, NTT DCM, Nokia, OPPO, MTK): For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability - Option 2a (CATT, CMCC, ZTE, CTC): For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. - Option 2b (vivo): Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4. - Option 2c (Apple, CTC): For UE supporting R19 multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline. - Option 2d (Nokia): At least support reduction of N from 8 to 4 in Rel-19. Support of other values such as N = 2, 6 to be discussed, and UE capability signalling to be introduced, if more than one value for reduced N is supported. - Option 2e (OPPO): Discuss the reduced scaling factors due to Rx BSF, considering both UE capabilities and applicable conditions. - Option 3 (vivo, Samsung, LGE): Introduce the UE capability to indicate the reduced M value (M=sample*BSF), - Option 3a (vivo): e.g., M=12, 14, 16, etc. - Option 3b (Samsung): suggest to take N=4, M=20 as the baseline. Whether to/How to define the other values can be based on SLS results (if necessary). - Option 3c (LGE): 3 or 4 Rx BSF value or additional scaling factor < 1 to reduce M value - Option 4 (vivo, LGE): - Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc. - Option 5 (Ericsson): - In order to cover all possible directions of neighbor cells, RX beam sweep doesn’t dynamically change, the factor can be defined as a single number, e.g., [4]. - Option 6 (Intel): - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Beam sweeping factor/total sample number of measurements can be cut half as a baseline while a margin can be added to the cut factor according to different SSB/SMTC configurations. - Recommended WF - [Moderator]: discuss option 2 and option 3 together with the scenarios 1/2/3/4/5/7.

Option 2 and Option 3 are similar in the sense that both consider the time needed for RX beam sweeping. The difference is Option 2 explicitly specifies the RX beam sweeping factor while Option 3 implicitly does so. Since RAN4 agreed at the last meeting to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19, it is natural to reflect that capability in the requirement. In other words, Option 2 is preferred.

Proposal 4: Option 2 is preferred, i.e., for UE supporting multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability.

Requirement at transition between the normal L3 measurement and FBS L3 measurement

Issue 1-4-1: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement - Proposal (ZTE): - The requirement at transition between the normal L3 measurement and FBS L3 measurement should be identified. There are two options: - Option 1: Reuse similar solution as the requirements at transition between measurement within gap and measurement outside gap, i.e. the longer delay is the period when transition happens. - Option 2: Once the transition happens, apply the requirements after the transition, no matter it is the normal L3 measurement or the FBS L3 measurement. - Recommended WF - TBA

Similar issues were discussed in R18 multi-RX WI. In the end, such a requirement were not pursued due to the associated complexity such as the needed conditions to define the start and end of transition. For L3 measurement, similar complexity exists and thus such a requirement should not be specified.

Proposal 5: Similar to the handling of R18 L1 measurements, no requirement is specified at transition between the normal L3 measurement and FBS L3 measurement.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: To consider forward compatibility where the specified requirement can apply to CA case, Option 1, i.e., one searcher assumption, is slightly preferred.

Proposal 2: It is possible for the UE to process both samples (one from each panel) within one SMTC periodicity. Therefore, it is OK to only consider some additional processing time after the last SMTC periodicity, i.e., Option 1.

Proposal 3: The triggering conditions should include UE power consumption, UE location, UE mobility and UE QoS requirements.

Proposal 4: Option 2 is preferred, i.e., for UE supporting multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability.

Proposal 5: Similar to the handling of R18 L1 measurements, no requirement is specified at transition between the normal L3 measurement and FBS L3 measurement.

# References

[1] R4-2414026, WF for NR_RRM_Ph5_Part1, Apple, RAN4#112