3GPP TSG-RAN WG4 Meeting #113		R4-2418555

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.19.3.2

Source:	Apple

Title:	On RRM Requirements for MIMO Phase 5

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#112bis the other RRM requirements impact for MIMO phase 5 was discussed and WF [1] was approved. In this contribution we discuss RRM requirements impact for objectives other than UE initiated beam management.

# 2. Discussion

UE Reporting enhancement for CJT calibration

Issue 2-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration? Agreement: - RAN4 to simulate the performance of offsets for phase, time delay and frequency, based on one-shot measurement and multiple-sample measurement, and discuss the impact on RRM core requirement later. - Further discuss the simulation assumptions offline during this meeting.

It was encouraged to bring simulation assumptions for further evaluation of performance of delay and frequency offset based on one shot measurement. The simulation assumptions are proposed in Table 1.

| Parameter | Value |
| --- | --- |
| Duplex | FDD |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC30-10 TDLC300-100 |
| Antenna configuration | 1X2 |
| Number of TRP | 2 |
| TRS Bandwidth | 48RB |
| SCS | 30kHz |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm. |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples (TRS resource sets) for measurement | 1, 2, 3, 4, 5 |
| CSI-RS periodicity | 20ms / 40 slots |
| CSI-RS offset | Slot 20, 21 |

Simulation assumptions for evaluation of frequency and delay offset measurement:

| Parameter | Value |
| --- | --- |
| Duplex | FDD |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC30-10 TDLC300-100 |
| Antenna configuration | 1X2 |
| Number of TRP | 2 |
| TRS Bandwidth | 48RB |
| SCS | 30kHz |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm. |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples (TRS resource sets) for measurement | 1, 2, 3, 4, 5 |
| CSI-RS periodicity | 20ms / 40 slots |
| CSI-RS offset | Slot 20, 21 |

The evaluation for delay and frequency offset shall separate and not combined.

Evaluate the frequency offset and delay offset measurement separately.

RAN4 could potentially define mapping tables for the delay quantities of not covered in RAN1 spec in performance part.

RAN4 to consider defining mapping tables for frequency/delay/phase offset reporting in performance part of WI.

Asymmetric DL sTRP/UL mTRP

In RAN4112bis the following agreements were made:

Issue 4-1: Whether RRM impacts exist by 2TA enhancement? Agreement: - RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP] - As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID. - Further discuss the details in the next meeting.

In Rel-18 RAN4 introduced timing requirements for 2TA for multi-DCI multi-TRP transmission scheme. Impacts for timing requirements for 2 TA with asymmetric DL sTRP/ UL mTRP needs to be further discussed in RAN4. In Rel-18 with 2 TA there was a reference point for each TAG associated with the DLorJoint TCI state with same TAG as UL signal. With asymmetric DL sTRP/ UL mTRP there is only 1 DL TRP, hence only a common reference point is possible for both TAGs.

- With asymmetric DL sTRP/ UL mTRP only 1 DL TRP and only common reference point is feasible for this case.

RAN4 to update the reference point definition for 2 TA with asymmetric DL sTRP/ UL mTRP.

Issue 4-2: RRM core impacts of Active uplink TCI state switching delay for unified TCI?

Way forward:

- Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

- Option 2: RAN4 to specify UL TCI state switching requirements.

- Option 3: FFS.

For asymmetric Dl sTRP/ UL mTRP, one of the TRPs is UL only TRP. The UL only TRP has no DL-RS for TCI state. Hence any TCI state for the UL channels to that TRP can only be SRS.

- With asymmetric DL sTRP/ UL mTRP the UL only TRP TCI state can only be SRS.

The TCI state switch/ UL spatial relation switch requirements are only defined when the target TCI state / spatial relation info is DL-RS, not SRS. For the UL only TRP, RAN4 should further discuss if TCI state switch requirements are introduced. For UL only TRP the options need to be discussed:

Option1: Define requirements for SRS bases TCI state switch

Option 2: Do not define requirements for UL TCI state switch for UL only TRP

- Until Rel-18 the UL TCI state switch requirements are only defined when target TCI state is DL-RS.

RAN4 to further discuss how to define requirements for TCI state switch for UL only TRP and further discuss the following options:
Option 1: Define requirements for SRS bases TCI state switch
Option 2: Do not define requirements for UL TCI state switch for UL only TRP

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from UE initiated beam management. Our observations and proposals are captured below:

UE Reporting enhancement for CJT calibration

- Simulation assumptions for evaluation of frequency and delay offset measurement:

| Parameter | Value |
| --- | --- |
| Duplex | FDD |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC30-10 TDLC300-100 |
| Antenna configuration | 1X2 |
| Number of TRP | 2 |
| TRS Bandwidth | 48RB |
| SCS | 30kHz |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm. |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples (TRS resource sets) for measurement | 1, 2, 3, 4, 5 |
| CSI-RS periodicity | 20ms / 40 slots |
| CSI-RS offset | Slot 20, 21 |

Evaluate the frequency offset and delay offset measurement separately.

RAN4 to consider defining mapping tables for frequency/delay/phase offset reporting in performance part of WI.

Asymmetric DL sTRP/UL mTRP

- With asymmetric DL sTRP/ UL mTRP only 1 DL TRP and only common reference point is feasible for this case.

RAN4 to update the reference point definition for 2 TA with asymmetric DL sTRP/ UL mTRP.

- With asymmetric DL sTRP/ UL mTRP the UL only TRP TCI state can only be SRS.

- Until Rel-18 the UL TCI state switch requirements are only defined when target TCI state is DL-RS.

RAN4 to further discuss how to define requirements for TCI state switch for UL only TRP and further discuss the following options:
Option 1: Define requirements for SRS bases TCI state switch
Option 2: Do not define requirements for UL TCI state switch for UL only TRP

# Reference

- R4-2416921, “WF on RRM requirements for NR_MIMO_Ph5_Part2”, MediaTek.