3GPP TSG-RAN WG4 Meeting #113                              			      				 R4-2418576

Orlando, US, 18th – 22nd November 2024

Source: 	Apple

Title: 	On type 4 UE RF requirement

Agenda Item:	7.5.2.1

Document for:	Discussion

# Introduction

RF requirement for Intra-band non-collocated NR CA/EN-DC has been discussed in recent RAN4 meetings and a WF was agreed [1] in RAN4#112 meeting with the following issues are remaining open for further study. There are some discussions in Hefei meeting while it was not concluded.

< Issue 2-1-2:  How to capture the power imbalance and REFSENS requirements >

Agreement:

- Define the Type 4 receiver inmalande requirements in the same table as for Type 2.

  - FFS on the details of modification

This contribution presents our views and propose to conclude this issue.

# Discussion

## How to capture the power imbalance requirements

In our view, there are 2 potential ways to capture these agreements in the power imbalance table [2]. In general, we are fine with both options. While it should be noted that a definition for type 2 and type 4 UE will be needed if we go option 2. While RAN4 agreed to give a definition for type 2/4 capability rather than type 2/4 UE [3]. In this regard, we would prefer Option 1.

Option 1: Adding NOTE 6 in Table 7.10A.2-1.

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: For UE indicating intraBandNR-CA-non-collocated-r18, REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. For UE indicating [intraBandNR-CA-non-collocatedType4-r19, REFSENS is the reference sensitivity level for four antenna port which is specified as the value in Table 7.3.2-1b modified by the amount ΔRIB,4R in Table 7.3.2-2. NOTE 5: Void. |  |  |  |  |

Option 2: Specified different Rx power for type 2 and type 4 UE.

- Note: For this option, it’s better to introduce definition for type 2 UE and type 4 UE in the specification.

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier for type 2 UE | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Wanted carrier for type 4 UE | REFSENS NOTE 4 + ΔRIB,4RNOTE 6 1 |  |  |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier for type 2 UE | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Wanted carrier for type 4 UE | REFSENS NOTE 4 + ΔRIB,4RNOTE 6 1 |  |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier for type 2 UE | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Wanted carrier for type 4 UE | REFSENS NOTE 4 + ΔRIB,4RNOTE 6 1 |  |  |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: Void. NOTE 6: ΔRIB,4R is the reference sensitivity allowance as specified in Table 7.3.2-2. |  |  |  |  |

Proposal 1: It is proposed to decide between Option 1 and Option 2 on how to capture power imbalance requirement for type 4 UE. We slightly prefer option 1.

## Requirement applicability

The following has been agreed for type 4 UE

- Type 4 should support the following capabilities (Set 1) when operating in non-collocated scenarios.

  - Maximum 4 MIMO layers on each CC

  - Maximum power imbalance of 25dB

- Type 4 UE should support the following capabilities (Set 2=Type 1 capability) when operating in collocated scenarios.

  - Maximum 8 MIMO layers on each CC

    - It may be optionally supported

  - Maximum PSD imbalance of 6dB

.

With this agreement, it means there are 2 kind of type 4 UE. One kind is Capability set 1 capable UE. The other one is Capability set 1&2 capable UE, which is different from type 2 UE.

For capability set 1 capable UE, it should be verified with power imbalance requirement for type 4 UE in 7.10A and 4Rx RF requirements for intra-band non-contiguous CA in 7.4A.2~7.8A.2.

For capability set 1&2 capable UE, it should be verified with power imbalance requirement for type 4 UE in 7.10A and 8Rx RF requirements for intra-band non-contiguous CA in 7.4A.2~7.8A.2.

Proposal 2: Requirement applicability rules for type 4 UE,

- For capability set 1 capable UE, it should be verified with power imbalance requirement for type 4 UE in 7.10A and 4Rx RF requirements for intra-band non-contiguous CA in 7.4A.2~7.8A.2.

- For capability set 1&2 capable UE, it should be verified with power imbalance requirement for type 4 UE in 7.10A and 8Rx RF requirements for intra-band non-contiguous CA in 7.4A.2~7.8A.2.

# Summary

This contribution presented our consideration on the open issues for type 4 UE RF requirements. The following proposals are concluded.

Proposal 1: It is proposed to decide between Option 1 and Option 2 on how to capture power imbalance requirement for type 4 UE. We slightly prefer option 1.

Proposal 2: Requirement applicability rules for type 4 UE,

- For capability set 1 capable UE, it should be verified with power imbalance requirement for type 4 UE in 7.10A and 4Rx RF requirements for intra-band non-contiguous CA in 7.4A.2~7.8A.2.

- For capability set 1&2 capable UE, it should be verified with power imbalance requirement for type 4 UE in 7.10A and 8Rx RF requirements for intra-band non-contiguous CA in 7.4A.2~7.8A.2.

# References

[1] R4-2410686, WF on Rel-19 non-collocated UE RF requirements, KDDI, RAN4#111

[2] R4-2411414, On remaining issue for type 4 UE RF requirement, Apple, RAN4#112

[3] R4-2417115, WF on rest issues for intra-band non-collocated EN-DC/NR-CA, KDDI, RAN4#112bis