3GPP TSG-RAN WG4 Meeting #113	 R4-2418586

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	7.22.2.2

Source: 	Apple

Title:	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion

# Introduction

RAN4 requirements for adaptation of common signal/channel transmission was widely discussed in the previous RAN4 meeting. The latest agreements can be found in [1]. There are quite many open issues as captured in [1].  In this contribution, we will continue discussion on all open issues listed in RAN4 WF [1]. After discussion, corresponding proposals are provided.

# Discussion

## SSB Adaptation requirements

In RAN4#112bis, supported scenario for this work item in RAN4 as agreed:

Issue 1-1-1: Scenario clarification based on RAN1 progress

Agreement:

- Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion:

  - CONNECTED mode, and

  - SSB adaptation for SCell, and

  - SSB adaptation for NCD-SSB, and

  - SSB periodicity adaptation in time-domain

Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism)

Besides, RAN4 also agreed to discuss whether there are any impacts existing RRM requirements due to SSB adaption:

Issue 1-2-1: Requirements need updating

Agreement:

- RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption:

  - L1-RSRP for serving cell

  - L1-SINR

  - BFD

  - CBD

  - SCell activation

  - TCI state switching

  - NR intra-frequency L3 measurement

  - NR inter-frequency L3 measurement

Note: Above list could be further updated based on RAN4 discussion.

For L1 operation like L1-RSRP, BFD and so on, RAN4 requirements are defined based on TSSB (periodicity of SSB configured for L1 operation). If SSB adaptation results in periodicity change, then naturally RRM requirements would change accordingly. One thing we would like to point out is that minimum requirements upon SSB periodicity adaptation is needed. Take TCI state switching measurement for example, sometimes UE needs multiple samples to finish TCI switching from RAN4 requirements point of view. If UE doesn’t have enough samples to finish TCI switching when SSB periodicity adaptation occurs, UE needs to follow new SSB periodicity to for measurement. This is similar with other transition procedures.

Proposal 1: minimum requirements for L1 operations upon SSB periodicity adaptation are needed:

- When SSB periodicity changes upon SSB periodicity adaptation, the L1 requirements apply based on the longer delay before or after the adaptation.

Proposal 2: after transition period, existing requirements still apply with clarification that TSSB is the updated SSB periodicity after adaptation.

As for L3 measurement, since RAN4 requirements are defined based on SMTC configuration, rather than SSB configuration, whether there is impact on L3 measurement depends on whether there is impact on SMTC when SSB adaptation occurs. So far we couldn’t find any conclusion on this from RAN1/2.

Observation 1: impact on L3 measurement due to SSB adaptation depends on whether there is impact on SMTC configuration upon SSB adaptation.

Proposal 3: more RAN1/2 inputs are expected before RAN4 can conclude on impact from SSB adaptation on L3 measurement.

## Impact on Paging Adaptation requirements

Issue 3-2-1: Paging adaptation

Agreement:

- Deprioritize Paging adaptation until the RAN1 concrete conclusion is available.

We haven’t identified any RAN4 impact yet.

## Impact on PRACH Adaptation requirements

Issue 3-3-1: PRACH adaptation

Agreement:

- Deprioritize PRACH adaptation until the RAN1 concrete conclusion is available.

We haven’t identified any RAN4 impact yet.

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for adaptation of common signal/channel transmission. After discussion, the following conclusions are provided:

Proposal 1: minimum requirements for L1 operations upon SSB periodicity adaptation are needed:

Proposal 2: after transition period, existing requirements still apply with clarification that TSSB is the updated SSB periodicity after adaptation.

Observation 1: impact on L3 measurement due to SSB adaptation depends on whether there is impact on SMTC configuration upon SSB adaptation.

Proposal 3: more RAN1/2 inputs are expected before RAN4 can conclude on impact from SSB adaptation on L3 measurement.

# Reference

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson