3GPP TSG-RAN WG4 Meeting #113							R4-2418603

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.20.2.3

Source: 	Qualcomm Incorporated

Title: 	Views on existing Rx BS RF requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#111, a WF was agreed in [3] based on some of the topics listed in the moderator summary, that is summarized in [2]. In this contribution, we share our views on existing Rx BS RF requirements for Rel-19 SBFD.

# Impact on existing BS RF requirements

## Rx Requirements

### Reference sensitivity level and OTA sensitivity

For the reference sensitivity, it was agreed in Rel-18 SI that the existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols for BS type 1-H. For the OTA sensitivity, RAN4 is expected to further discuss the allowed sensitivity degradation at the SBFD BS due to the self-interference. During the SI, value ranges within [0.5-1] dB were considered. The definition of the OTA sensitivity has not been finalized during the SI, it is our view that OTA sensitivity should be defined considering self-interference, inter-site interference, and inter-sector interference.

Proposal 1: RAN4 to agree on 1 dB degradation due to self-interference, inter-site interference, and inter-sector interference when defining the OTA reference sensitivity.

### In-band selectivity and blocking

As agreed, in [2], the in-band blocking requirements are expected to be derived based on RAN4 adjacent channel coexistence as agreed below:

- FFS on the followings: - For FR1 WA BS, RAN4 need to discuss whether a unified in-band blocking requirement is defined? - Option 1: a unified blocking requirement to consider both scenarios: coexistence with legacy TDD system in adjacent channel, and coexistence with new SBFD system in adjacent channel. - Option 2: separate requirements are defined, - One set of requirements for coexisting with legacy TDD system in adjacent channel - One set of requirements for coexisting with new SBFD system in adjacent channel - For FR1 WA BS co-existence study, FFS grid-shift values should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - In-band blocking requirement for FR1 MR SBFD BS - In-band blocking requirement for FR2-1 WA and MR SBFD BS - For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements.

In our previous contribution (R4-2415468), we have provided our in-band blocking requirement for 50% GS around -26 dB, considering AAS configuration considering two panels with 46 dBm conducted power per polarization. Given that there are difference in the underlying assumptions among companies within RAN4 and given that the main source of higher values for the in-band blocking stem from the inter-gNB CLI. It is preferable that RAN4 decouples the in-band requirements for coexistence with legacy TDD and coexistence with SBFD deployments. Such a decoupling of requirements can be implemented via defining different in-band blocking requirements and different declarations for the different levels.

Proposal 2: RAN4 to consider decoupled requirements for in-band blocking, where legacy requirements can be reused for coexistence with legacy TDD, and new set of requirements defined for new SBFD deployment in the adjacent channel.

Regarding the coexistence with SBFD deployments, we believe it is important to account for realistic deployments and scenarios. Based on [1], we have the following

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3]: - Support gNB-to-gNB CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117)

Thus, it is within the scope of the Rel-19 normative work that techniques for inter-gNB CLI will be developed for both co-channel and adjacent channel. Since it might not be straightforward how to account for such CLI handling techniques in RAN4 when defining the different RAN4 requirements, it is preferable to take into consideration that values based on the adjacent channel coexistence alone might be pessimistic. In addition, considering low grid shift values and looking at the 99% percentile point might lead to overestimation of the inter-gNB CLI.

Proposal 3: RAN4 to account for the fact that inter-gNB CLI handling techniques will be considered in the Rel-19 normative work and thus, low grid shift values and very high percentiles of the Rx power CDF (99%) might cause overestimation of the inter-gNB CLI.

# Conclusion

In this contribution, we provided our views on BS RF requirements for SBFD with the following observations and proposals:

Proposal 1: RAN4 to agree on 1 dB degradation due to self-interference, inter-site interference, and inter-sector interference when defining the OTA reference sensitivity.

Proposal 2: RAN4 to consider decoupled requirements for in-band blocking, where legacy requirements can be reused for coexistence with legacy TDD, and new set of requirements defined for new SBFD deployment in the adjacent channel.

Proposal 3: RAN4 to account for the fact that inter-gNB CLI handling techniques will be considered in the Rel-19 normative work and thus, low grid shift values and very high percentiles of the Rx power CDF (99%) might cause overestimation of the inter-gNB CLI.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2416585, “Way Forward for [112][308] NR_duplex_evo_General”, RAN4#112bis, Huawei, HiSilicon.