3GPP TSG-RAN4 Meeting #113	R4-2418608

Orlando, US, 18th – 22nd November, 2024

Title:	Views on Additional AAS aspects related to spatial beamforming

Source:	Qualcomm Incorporated

Agenda item:	7.2.5

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1]. One of the objectives of the SI is as follows:

- Considering the system-level simulation work typically conducted in ITU-R WP 5D for sharing and compatibility studies using 3D modelling of antenna pattern, ITU-R WP 5D is seeking information on whether for implementation of IMT BS multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference and the accuracy of studies. If considered necessary, guidance on the process of deriving the necessary beamforming weights for the IMT AAS BS to compute its radiation pattern.

Since it is expected that RAN4 provide its technical view on the above matter, independent of the frequency range, it is important that RAN4 have enough time to converge on the technical response to be provided. In this contribution, we provide our views on the impact of multi-user spatial beamforming techniques on the system level simulation, and we propose a way forward to be shared with WP5D.

# 2 	Discussion

## 2.1	Background

In ITU’s WP5D LS sent to RAN4 [2], questions on the impact of IMT BS multi-user spatial beamforming such as ZF, MRT, and MMSE on the system level studies outcome were included, independent of the frequency range under investigation. Since RAN4#110, RAN4 has been working on drafting its response to WP5D LS [2] for the different frequency ranges. Among those generic questions is the impact of spatial beamforming on the studies outcome. Specifically, WP5D is currently trying to converge whether it is needed or not to consider multi-user beamforming techniques impact on top of the AAS model (i.e., ITU-R M.2101). Some administrations are concerned that such techniques might lead to additional unwanted aggregated emissions above the horizon, whereas the legacy model might not sufficiently account for that (see Document 5D/158-E for more information). On the other hand, during the June WP5D meeting, there were other views from the industry that incorporating ZF beamforming techniques into the legacy M-2101 model will have a quite minimal impact on the aggregate emissions above the horizon (see Document 5D/197-E for more information).

Observation 1: WP5D is currently investigating whether it is needed or not to consider multi-user beamforming techniques impact on top of the AAS model (i.e., ITU-R M.2101).

In our contribution we investigate the impact of ZF beamforming at the IMT BS, serving UEs in the DL transmission, with specific focus on the aggregate unwanted emissions above the horizon for a given IMT BS. Our IMT BS AAS modelling follows closely the framework on IMT AAS radiation pattern characterization in Recommendation ITU-R M.2101.

Finally, RAN4 should investigate the impact of such techniques on system level performance, converge on a technical response that addresses the concerns of WP5D members, and document its findings in TR 38.922.

Observation 2: RAN4 is expected to provide its technical response on the impact of multi-UE beamforming techniques impact on system level studies to WP5D and capture its findings in TR 38.922.

## 2.2	ZF impact in system level simulations

To investigate the impact of ZF on the emissions above the horizon, proper modelling of the multi-user spatial beamforming needs first to be carried out. In this regard, we follow the methodology considered in Annex 1 to evaluate the composite radiation pattern in system level studies. Our objective is to investigate whether incorporating ZF technique in a system level framework would lead to additional unwanted aggregated emissions above the horizon for an IMT BS serving its scheduled UEs.

The nulling capability of the adopted methodology and the impact of ZF on the AAS BS radiation pattern is investigated in this section, where we consider as an example the co-scheduling of two UEs by the IMT BS. We assume that UE#1 is located at the BS boresight while UE#2 is located at either 5, 10, and 20 degrees from UE#1. Figure 1 shows the radiation patterns towards UE #1 of a 16x8 AAS array with spacing of 0.5 wavelengths in the vertical and horizontal planes.

As observed, when there is sufficient angular (i.e., spatial) separation between the scheduled UEs (e.g., at 10 and 20 degrees from UE#1), the antenna gain is maximized at UE#1 and minimized at UE#2, compared without ZF case (figure on the left). However, when such angular separation decreases (e.g., at 5 degrees), degradation of performance at UE#1 is expected compared to the case without ZF. From the figures, we can observe a gain degradation of 4.8, 0.7, and 0.2 dB for the 5-, 10-, and 20-degrees angular separation, respectively. As a consequence, as also shown in section 3, a 10 degrees angular separation is sufficient since the antenna gain reduction is within 0.7 dB.

Figure 1 Radiation pattern at UE#1 assuming UE2 at 5, 10 or 20 degrees from UE1 respectively.

## 2.3 	Simulation assumptions and Results

BS AAS parameters and configurations that are considered in our Monte-Carlo simulation campaign are provided in Table 1, Table 2, and Table 3, respectively. In terms of scenario/deployment, we consider a single sector where within each snapshot, 3 UEs are randomly deployed within the coverage region of that sector with a given minimum angular/spatial separation of 10 degrees. The location of those UEs is assumed to be known at the BS and a dedicated beam is generated to serve each UE. In order to investigate the impact of BS emissions above the horizon, probing points in the upper hemisphere have been placed, as visualized in Figure 2. Black points represent the total probing points, while blue points represent the 3 UEs dropped in each snapshot, taking into account the minimum angular separation among those UEs (i.e., 10 degrees as assumed in this contribution). The probing points are equidistantly distributed on a hemisphere above the horizon with angular separation between them approximately 18 degrees. It is worth mentioning that the objective of our study is to investigate the experienced emissions at the different probing points, since they represent possible incumbent services that will suffer from to the IMT BS unwanted emissions above the horizon.

TABLE 1 IMT deployment-related parameters

|  | Urban macro |
| --- | --- |
| Base station |  |
| Carrier frequency | 6 GHz-band |
| Channel bandwidth | 100 MHz |
| BS Antenna height | 18 m |
| Cell size | 300 m |
| Sectorization | 1 sector |
| Frequency reuse | 1 |
| User terminal |  |
| UE height | 1.5 m |
| UE density for terminals that are transmitting simultaneously | 3 UEs per sector |

TABLE 2

IMT base station beamforming antenna characteristics for IMT

|  | Urban Macro |
| --- | --- |
| Antenna pattern | Refer to Recommendation ITU-R M.2101 Annex 1 |
| Element gain (incl. Ohmic loss) (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-120 (Note 1) |
| Mechanical downtilt (degrees) | 10 |
| Note 1: The vertical coverage range includes the mechanical downtilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

TABLE 3

IMT base station beamforming antenna characteristics for IMT with subarray configuration

|  | Urban Macro |
| --- | --- |
| Antenna pattern | Refer to the extended AAS model in 5D/716 (Annex 4.4 - WRC23) |
| Element gain (incl. Ohmic loss) (dBi) | 6.4 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 16 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H 1.4 of wavelength for V |
| Number of element rows in sub-array | 2 |
| Vertical radiating element spacing in sub-array | 0.7 of wavelength |
| Pre-set sub-array downtilt (degrees) | 3 |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-100 (Note 1) |
| Mechanical downtilt (degrees) | 6 |
| Note 1: The vertical coverage range includes the mechanical downtilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

.

Figure 2 System layout for a given sector

### 2.3.1	Results Based on methodology given in Annex A

The CDF curves of the IMT BS gain towards the black probing points are shown in Figure 3, considering the AAS without subarray structure (left figure) and with subarray structure (right figure). It can be observed the performance degradation observed when ZF is applied is negligible. Such behaviour shows that when considering large number of snapshots in a Monte-Carlo framework, the impact of the spatial beamforming above the horizon has minimal impact on the emissions above the horizon.

Figure 3 Composite AAS gain from IMT BS towards probing points considering AAS with (left) and without subarrays (right)

In Table 4 we provide a summary of the average base station antenna gain difference when using ZF beamforming and ITU-R M.2101 model, comparing both AAS with and without subarray configurations for long-term (80th percentile) and short-term (99th percentile) metrics.

TABLE 4 Difference in average BS antenna gain between the ITU-R M.2101 beamforming model and ZF beamforming

| Scenario | Long-term (80th percentile) | Short-term (99th percentile) |
| --- | --- | --- |
| AAS without subarrays | 0.48 dB | 0.17 dB |
| AAS with subarrays | 0.12 dB | 0 dB |

### 2.3.1 	Observations with modified methodology

In Annex A, the beamforming matrix WkZF for the k-th user is built based on Ukn,m.Based on the latest ITU-R WP 5D discussions and updates (i.e., October meeting), an alternative methodology to construct WkZF has been discussed. In contrast to the methodology given in Annex A, the alternative methodology considers the construction of the channel matrix Hk based on wn,m and νn,m from ITU-R M.2101 that is then used to derive the zero-forcing beamforming (precoding) matrix WkZF as

WkZF=HkHHkHkH-1.

Note that νn,m is the superposition vector as given in the Rec. ITU-R M.2101. Additionally, for this updated methodology, the beam pattern following ITU-R M.2101 is evaluated as

Akθ,ϕ=AEθ,ϕ+10log10(|m=1NHn=1NVwkn,m​2),

And for AAS with sub-arrays, AEθ,ϕ is updated by Asubθ,ϕ:

Akθ,ϕ=Asubθ,ϕ+10log10(|m=1NHn=1NVwkn,m​2).

It is noteworthy to emphasize that for this modified methodology, the beam pattern evaluation is based only on WkZF and not the multiplication of WkZF and νn,m as the methodology given in Annex A.

To validate the impact of this modified methodology, the BS IMT gain CDF curves towards the black probing points are shown in Figure 4, based on the methodology given in Annex A (i.e., solid red curves) and the modified methodology given in this section that considers both  wn,m and νn,m  when constructing the precoding matrix (i.e., dashed cyan curves). It can be observed that the solid red and dashed cyan curves are nearly overlapping, which implies that both methodologies to construct the beamforming matrix and composite AAS beam pattern led to nearly identical results. This is because both the beamforming direction and the superposition vector are both accounted for when the composite IMT BS AAS beam pattern is evaluated.

Figure 4 Composite AAS gain from IMT BS towards probing points considering AAS with (left) and without subarrays (right)

## 2.4	Simulation results conclusions

In order to provide RAN4 feedback to WP5D on the impact of spatial beamforming in system level studies, it is proposed based on our observation to consider the legacy AAS model without ZF beamforming (or similar techniques), since the performance is quite comparable compared to the case where ZF beamforming is considered. Thus, with no impactful difference and additional complexity for the system level studies, legacy AAS model should be sufficient for WP5D sharing studies.

Proposal 1: RAN4 to reply to WP5D that it is sufficient to adopt the legacy AAS model based on ITU-R Recommendation M.2101 since the performance when ZF beamforming is considered is similar to the performance without ZF beamforming technique. Thus, the difference between M.2101 model and the ZF precoder scheme is minor, and therefore M.2101 should be used for WRC-27 study cycle.

# Conclusion

In this contribution we provide our views on the impact of multi-user spatial beamforming techniques on the system level simulation. Our observations and proposals can be summarized for that frequency range as follows:

Observation 1: WP5D is currently investigating whether it is needed or not to consider multi-user beamforming techniques impact on top of the AAS model (i.e., ITU-R M.2101).

Observation 2: RAN4 is expected to provide its technical response on the impact of multi-UE beamforming techniques impact on system level studies to WP5D and capture its findings in TR 38.922.

Observation 3: From Monte-Carlo studies, similar IMT BS gain above the horizon is observed when zero forcing is considered at the BS IMT when compared to the case without zero forcing beamforming.

Observation 4: The IMT BS radiation pattern is nearly identical when considering the methodology given in Annex A and the modified methodology based wn,m and νn,m from ITU-UR M.2101 when constructing the channel matrix Hk.

Proposal 1: RAN4 to reply to WP5D that it is sufficient to adopt the legacy AAS model based on ITU-R Recommendation M.2101 since the performance when ZF beamforming is considered is similar to the performance without ZF beamforming technique. Thus, the difference between M.2101 model and the ZF precoder scheme is minor, and therefore M.2101 should be used for WRC-27 study cycle.

# Reference

[1]		RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson, RAN #103, Maastricht, Netherlands.

[2]	R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, WP5D, RAN4 #110, Athens, Greece.

[2] 	R4-2406615, “WF on IMT parameters for other frequency ranges”, CATT, RAN4#110-bis, Changsha, China.

[3]	TR 38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz”, 3GPP V17.1.0.

# Annex A: Methodology for ZF formulation

The following steps represent the procedure to evaluate the impact of ZF at the IMT BS on the emissions above the horizon.

- For the k-th UE, generate the uniform planar array (UPA) coefficients, Ukn,m, using the weighting formulation in Rec. ITU-R M.2101 – Table 4, to reflect the UE angles from the BS antenna array in both azimuth and elevation:

Ukn,m=expj2πn-1dvλcosθk+m-1dhλsinθksinφk

where, for the k-th UE, θk and φk are the angles of elevation and azimuth, respectively. The ranges of θk and φk are 0,π and [-π,π] radians, respectively, as defined in Rec. ITU-R M.2101. The size of the matrix Ukn,m is N×M as defined in M.2101, where N is the number of rows and M is the number of columns of the antenna array.

- Assess the angular separation between the K UEs in the cell (ITU-R assumption, K = 3).

- For each UE, if there is at least another UE which with the minimum angular separation requirement is fulfilled, create a concatenated P×MN channel matrix,  Hk:

Hk=hkT …hpTT

where T is the transpose operator and hk corresponds to the vectorization of Ukn,m resulting in a vector of size MN×1. Note that, for simplicity, hkT is always the first row of Hk, and hpT represent the UE(s) that can be paired with the k-th UE.

- For each UE, the zero-forcing beamforming (precoding) matrix WkZF (MN x P) is:

WkZF=HkHHkHkH-1

where Hk represents the channel matrix with its number of columns corresponding to the number of Rx antennas (i.e., UEs) and its number of rows corresponding to the number of Tx antennas (BS antenna ports), and HkH is the conjugate transpose of Hk.

- The entire WkZF matrix can be normalised by diving it by its Frobenius norm, or, doing a column wise normalisation by diving each column with its norm.

- The actual precoding weights for each UE (wk) are obtained by selecting the first column of WkZF. The size of wk is MN×1.

- Obtain the individual Zero Forcing beam pattern: The precoding weights for each beam wk of size MNx1 may be used to obtain the k-th zero forcing beam pattern by reordering the column vector wk of size MNx1 into a matrix Wk of size NxM which can be used as element weight in the formulation in Rec. ITU-R M.2101:

Akθ,ϕ=AEθ,ϕ+10log10(|m=1NHn=1NVwkn,m⋅νn,m​2)

where wkn,m refers to the entry in the n-th row and m-th column of weighting matrix Wk and νn,m is the superposition vector as given in the Rec. ITU-R M.2101.

Note that for AAS with sub-arrays, AEθ,ϕ is updated by Asubθ,ϕ:

Akθ,ϕ=Asubθ,ϕ+10log10(|m=1NHn=1NVwkn,m⋅νn,m​2)

where Asubθ,ϕ calculation follows the method defined in the extended AAS modelling in Annex 4.4 to Working Party 5D Chairman’s Report ANNEX 3.

- The resulting composite ZF beam pattern may be obtained by summing up the individual Zero Forcing beam patterns in linear scale (and transforming them back to dB):

AZFθ,ϕ=10log10p=1P10Akθ, ϕ10

where AZFθ,ϕ refers to the composite ZF beam pattern and Akθ,ϕ refers to the k-th individual zero forcing beam pattern.

# TP to TR 38.922 on MIMO Modelling

# 7.3	MIMO modelling

## 7.3.1	Simulation methodologies

## 7.3.1.1	Methodology 1

The following steps represent the procedure to evaluate the impact of ZF at the IMT BS on the emissions above the horizon.

- For the k-th UE, generate the uniform planar array (UPA) coefficients, Ukn,m, using the weighting formulation in Rec. ITU-R M.2101 – Table 4, to reflect the UE angles from the BS antenna array in both azimuth and elevation:

Ukn,m=expj2πn-1dvλcosθk+m-1dhλsinθksinφk

where, for the k-th UE, θk and φk are the angles of elevation and azimuth, respectively. The ranges of θk and φk are 0,π and [-π,π] radians, respectively, as defined in Rec. ITU-R M.2101. The size of the matrix Ukn,m is N×M as defined in M.2101, where N is the number of rows and M is the number of columns of the antenna array.

- Assess the angular separation between the K UEs in the cell (ITU-R assumption, K = 3).

- For each UE, if there is at least another UE which with the minimum angular separation requirement is fulfilled, create a concatenated P×MN channel matrix,  Hk:

Hk=hkT …hpTT

where T is the transpose operator and hk corresponds to the vectorization of Ukn,m resulting in a vector of size MN×1. Note that, for simplicity, hkT is always the first row of Hk, and hpT represent the UE(s) that can be paired with the k-th UE.

- For each UE, the zero-forcing beamforming (precoding) matrix WkZF (MN x P) is:

WkZF=HkHHkHkH-1

where Hk represents the channel matrix with its number of columns corresponding to the number of Rx antennas (i.e., UEs) and its number of rows corresponding to the number of Tx antennas (BS antenna ports), and HkH is the conjugate transpose of Hk.

- The entire WkZF matrix can be normalised by diving it by its Frobenius norm, or, doing a column wise normalisation by diving each column with its norm.

- The actual precoding weights for each UE (wk) are obtained by selecting the first column of WkZF. The size of wk is MN×1.

- Obtain the individual Zero Forcing beam pattern: The precoding weights for each beam wk of size MNx1 may be used to obtain the k-th zero forcing beam pattern by reordering the column vector wk of size MNx1 into a matrix Wk of size NxM which can be used as element weight in the formulation in Rec. ITU-R M.2101:

Akθ,ϕ=AEθ,ϕ+10log10(|m=1NHn=1NVwkn,m⋅νn,m​2)

where wkn,m refers to the entry in the n-th row and m-th column of weighting matrix Wk and νn,m is the superposition vector as given in the Rec. ITU-R M.2101.

Note that for AAS with sub-arrays, AEθ,ϕ is updated by Asubθ,ϕ:

Akθ,ϕ=Asubθ,ϕ+10log10(|m=1NHn=1NVwkn,m⋅νn,m​2)

where Asubθ,ϕ calculation follows the method defined in the extended AAS modelling in Annex 4.4 to Working Party 5D Chairman’s Report ANNEX 3.

- The resulting composite ZF beam pattern may be obtained by summing up the individual Zero Forcing beam patterns in linear scale (and transforming them back to dB):

AZFθ,ϕ=10log10p=1P10Akθ, ϕ10

## where AZFθ,ϕ refers to the composite ZF beam pattern and Akθ,ϕ refers to the k-th individual zero forcing beam pattern.

## 7.3.2	Simulation results

BS AAS parameters and configurations that are considered in our Monte-Carlo simulation campaign are provided in Table 1, Table 2, and Table 3, respectively. In terms of scenario/deployment, we consider a single sector where within each snapshot, 3 UEs are randomly deployed within the coverage region of that sector with a given minimum angular/spatial separation of 10 degrees. The location of those UEs is assumed to be known at the BS and a dedicated beam is generated to serve each UE. In order to investigate the impact of BS emissions above the horizon, probing points in the upper hemisphere have been placed, as visualized in Figure 2. Black points represent the total probing points, while blue points represent the 3 UEs dropped in each snapshot, taking into account the minimum angular separation among those UEs (i.e., 10 degrees as assumed in this contribution). The probing points are equidistantly distributed on a hemisphere above the horizon with angular separation between them approximately 18 degrees. It is worth mentioning that the objective of our study is to investigate the experienced emissions at the different probing points, since they represent possible incumbent services that will suffer from to the IMT BS unwanted emissions above the horizon.

TABLE 1 IMT deployment-related parameters

|  | Urban macro |
| --- | --- |
| Base station |  |
| Carrier frequency | 6 GHz-band |
| Channel bandwidth | 100 MHz |
| BS Antenna height | 18 m |
| Cell size | 300 m |
| Sectorization | 1 sector |
| Frequency reuse | 1 |
| User terminal |  |
| UE height | 1.5 m |
| UE density for terminals that are transmitting simultaneously | 3 UEs per sector |

TABLE 2

IMT base station beamforming antenna characteristics for IMT

|  | Urban Macro |
| --- | --- |
| Antenna pattern | Refer to Recommendation ITU-R M.2101 Annex 1 |
| Element gain (incl. Ohmic loss) (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-120 (Note 1) |
| Mechanical downtilt (degrees) | 10 |
| Note 1: The vertical coverage range includes the mechanical downtilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

TABLE 3

IMT base station beamforming antenna characteristics for IMT with subarray configuration

|  | Urban Macro |
| --- | --- |
| Antenna pattern | Refer to the extended AAS model in 5D/716 (Annex 4.4 - WRC23) |
| Element gain (incl. Ohmic loss) (dBi) | 6.4 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 16 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H 1.4 of wavelength for V |
| Number of element rows in sub-array | 2 |
| Vertical radiating element spacing in sub-array | 0.7 of wavelength |
| Pre-set sub-array downtilt (degrees) | 3 |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-100 (Note 1) |
| Mechanical downtilt (degrees) | 6 |
| Note 1: The vertical coverage range includes the mechanical downtilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

.

Figure 2 System layout for a given sector

The CDF curves of the IMT BS gain towards the black probing points are shown in Figure 3, considering the AAS without subarray structure (left figure) and with subarray structure (right figure). It can be observed the performance degradation observed when ZF is applied is negligible. Such behaviour shows that when considering large number of snapshots in a Monte-Carlo framework, the impact of the spatial beamforming above the horizon has minimal impact on the emissions above the horizon.

Figure 3 Composite AAS gain from IMT BS towards probing points considering AAS with (left) and without subarrays (right)