3GPP TSG-RAN4 Meeting #113	R4-2418609

Orlando, US, 18th – 22nd November, 2024

Title:	Views on Radio parameters for 14800 - 15350 MHz

Source:	Qualcomm Incorporated

Agenda item:	7.2.4.2

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1]. In RAN4#112bis, a WF was agreed in [2] and [3] regarding BS and UE parameters for the 14800 – 15350 MHz frequency range, respectively. In this contribution, we provide our views on the open items regarding system parameters and RF parameters for BS and UE items for the 14800 – 15350 MHz frequency range.

# 2 	Discussion

## 2.1	System parameters

### 2.1.1	Duplexing

Since there is no 14800 – 15350 MHz frequency range, it is expected that the discussion regarding duplexing would be similar to the 7125 – 8400 MHz frequency range. Accordingly, RAN4 is expected to consider TDD as a candidate duplexing for the 14800 – 15350 MHz Frequency range in its LS response to WP5D, while capturing additional information on SBFD in TR 38.922.

Proposal 1: RAN4 to agree on TDD as a baseline duplexing for 14800 – 15350 MHz frequency range and capture the following text in TR 38.922.

“Even though FDD is not precluded, most likely TDD will be used in this frequency range. In addition, SBFD is considered as a candidate duplexing method. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [8].”

### 2.1.2	Channel bandwidth

In general, wider channel bandwidths should be enabled with increasing frequency following same approach as FR1 (3 to 100 MHz) to FR2 (50 to 400 MHz). This comes with increasing SCS and increasing RB width and availability of the spectrum. Since the 14800 - 15350 MHz frequency range is not in the FR1 frequency range, higher channel bandwidths compared to 100MHz should be the baseline. Such channel bandwidths can be achieved via higher SCS or intra-band CA employment. Thus, we propose to consider 200MHz as a baseline for this frequency range, while mentioning that other channel bandwidths are not precluded.

Proposal 2: RAN4 to adopt 200MHz is a typical channel bandwidth. Additionally, it should be stated clearly in TR 38.922 that higher channel bandwidths should not be precluded for this frequency range.

### 2.1.3	Signal bandwidth

Signal bandwidth depends on the channel bandwidth and the considered SCS. The transmission bandwidth configuration defined in RAN4 core spec for BS and UE can be reused to derive the necessary signal bandwidth, where the signal bandwidth is calculated based on the NR spectrum utilization (number of PRBs and SCS) as follows:

Signal bandwidth = NPRB x SCS x 12

For the targeted bandwidth, NPRB can be assumed the number of PRBs for a 200MHz maximum channel bandwidth and 60 KHz is a possible candidate for this frequency range. Since it will be difficult to agree within RAN4 on a given SCS till RAN4#112 (i.e., RAN4 target deadline for 7GHz parameters), it is proposed to mention the dependency of signal bandwidth on the SCS and number of RBs, while mention typical values for NPRB and SCS in the LS reply.

Proposal 3a: RAN4 to not specify a fixed signal bandwidth but rather mention its dependency on SCS and number of RBs, similar to what was done to 7125-8400 MHz frequency range.

Proposal 3b: RAN4 to consider typical 60KHz as typical SCS for this frequency range and NPRB as given for 200MHz is TS 38.104.

## 2.2	BS parameters

### 2.2.1	Array size

From a feasibility point of view and with the expectation to provide wide-area coverage for this frequency range, considering at least 1k elements at the BS side is a viable option. When considering large antenna elements at this frequency range, there has been some discussion within RAN4 on the near-field impacts. This is justified under the fact that in the context of large antenna arrays and short UE-gNB distances where the near-far field region can be of the same order as the UE-BS separation, such large panels may potentially test the limits of planar wavefront modelling and may necessitate spherical wavefront modelling where the exact distance between any pair of transmit and receive antennas needs to be taken into account. Spherical wavefronts occur naturally when the antennas are modelled as point sources. Such a phenomena is currently being under investigation as part of the Rel -19 SI on FR3 channel modelling (see RP-234018  for more details). The impact of modelling near-by users using spherical waves on system level evaluations is still not clear and thus, it is recommended that RAN4 postpones the near-field aspects for this frequency range in the normative phase and not address it within its reply to WP5D until RAN1 concludes its study

Observation 1: The impact of modelling near-by users using spherical waves on system level evaluations is not clear in RAN4 and currently being studied in RAN1.

Proposal 4: RAN4 to not consider near-field impacts in this frequency range in its correspondence to WP5D.

Regarding the number of antenna elements, in [2], the following was captured:

- Make assumption that reply can say that array size is [3072 (16*24, SA size 4)], and the TR will document that other arrays are possible. - Double check the coverage and [3072] before making final decision on reply

Based on our simulation work and accompanying contribution, we have observed that BS array size of 3072 dual polarized antenna elements is sufficient to provide urban macro deployments for this frequency range. Our considered array configuration is based on [3], i.e., 16*24 URA with 4 elements per subarray.

Proposal 5: RAN4 to consider 1536 dual polarized antenna array based on 16*24 with 4 elements per subarray.

### 2.2.2 Spectral mask

For 10.0 - 10.5 GHz range derivation of the spectral mask in TR 38.921, RAN4 considered that channel bandwidths less than 50MHz are not foreseeable, thus, the basic limits for the SEM were based on 50MHz. RAN4 could follow the same approach considering scaling the basic limits for OOB emission mask with an appropriate minimum channel bandwidth value. This is in alignment with the proposed WF given in [2].

Proposal 6: RAN4 to adopt Category A and B SEM masks given in TR 38.921.

### 2.2.3	BS ACLR/ACS

Generally, the BS ACLR and ACS are based on the outcome of the adjacent channel coexistence. Since this frequency range falls between FR1 and FR2-1, RAN4 should make sure that the defined ACLR/ ACS are consistent with the defined ACLR/ACS for FR1 and FR2-1.

Proposal 7: For BS ACLR and ACS will be based on the outcome of the adjacent channel coexistence. In addition, RAN4 should make sure that the defined ACLR/ ACS are consistent with the defined ACLR/ACS for FR1 and FR2-1.

Regarding the coexistence results, based on our simulation work and accompanying contribution summarizing the outcome of the adjacent channel coexistence, we have observed values of 21 and 13 dB for downlink and uplink ACIR, respectively.

Observation 2: Based on the agreed simulation parameters for the adjacent channel coexistence work, 21 and 13 dB ACIR were observed for the downlink and uplink ACIR, respectively.

### 2.2.4	Blocking response

By definition, the in-band blocking requirement should apply from FUL,low - ΔfOOB to FUL,high + ΔfOOB, excluding the downlink frequency range of the operating band. In TR 38.921, ΔfOOB = 100 MHz was assumed for 10GHz frequency range. Adopting ΔfOOB = 100 MHz for the 14800-15300 MHz frequency range should be further discussed. On top of that, it is proposed that in-band and out-of-band blocking requirements follow what was agreed in TR 38.921.

Proposal 8: RAN4 to adopt if ΔfOOB = 100 MHz for the definition of in-band and out-of-band blocking.

## 2.3	UE parameters

### 2.3.1	Maximum Output power

PC3 was assumed during Rel-17 work on 7GHz and 10GHz as captured in [3]. On the other hand, n104 considers both PC2 and PC3. Additionally, 3GPP has studied high power UEs during Rel-17 covering high-power UE (power class 1.5) operation in NR bands n77 and n78 (RP-202912), high power UE for NR TDD intra-band Carrier Aggregation in frequency range FR1 (RP-221774), and high-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands (RP-221795). To capture the trend of higher UE power classes, we propose to communicate with WP5D PC3 (i.e., 23 dBm) as the typical UE output power, while capture in the TR that higher UE maximum output power values (26 dBm or 29 dBm) are not precluded in the future.

Proposal 9: RAN4 to agree on 23 dBm as UE typical maximum output power in the LS reply and to include in TR that higher power classes are not precluded in the normative phase of this frequency range.

### 2.3.2	UE power dynamic range

To derive the power dynamic range, RAN4 needs to agree on the UE minimum and maximum output power. We propose to reuse FR1 UE minimum output power of -33 dBm while 23 dBm as maximum output power. Accordingly, we propose 59 dBm for the UE power dynamic range.

Proposal 10: RAN4 to agree on 56 dBm as UE power dynamic range.

### 2.3.5	UE Noise figure

Based on [3], UE noise figure of 13 dB was tentatively agreed. In order to ensure consistency of the response with the carried out adjacent channel coexistence work, it is preferable to consider UE noise figure of 13 dB for both LS reply (i.e., TR) and simulation work.

Proposal 11: RAN4 to consider 13 dB as UE noise figure for the LS reply.

### 2.3.6	Spectral mask

For 10.0 - 10.5 GHz range derivation of the UE SEM in TR 38.921, is relaxed compared to NR general NR FR1 SEM at the FOOB edge ± 0 – 5 MHz by 3 dB. For the 14800 – 15300 MHz frequency range, the adoption of high SCS is not precluded (e.g., 60KHz). In order to ensure that the region of the SEM close to the CHBW is compliant with the SEM future requirements, it is viable to consider similar relaxation at the FOOB edge ± 0 – 5 MHz by 3 dB.

Proposal 12: RAN4 to consider UE SEM given in TR 38.921, which is relaxed compared to NR general NR FR1 SEM at the FOOB edge ± 0 – 5 MHz by 3 dB.

### 2.3.3	UE ACLR/ACS

Typically, the UE ACLR and ACS should be derived based on the outcome of the adjacent channel coexistence. However, additional consideration should be considered in order to make sure that the ACIR ( BS/UE ACLR and ACS) value is reasonable. As an example, for FR2-1 UE, the uplink ACIR resulting from the adjacent channel coexistence work, captured in TR 38.803, resulted in 16 dB ACIR, which is evaluated based on 17 dB UE ACLR and 23.5 dB BS ACS. In reality, the UE need to also meet the occupied bandwidth requirement, which is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. Considering the occupied bandwidth requirement, the actual UE ACLR is found to be equivalent to 24 dB. Accordingly, RAN4 should make sure that the BS and UE ACLR/ACS values communicated to WP5D are reasonable considering other RF requirements.

Proposal 13: RAN4 to consider the UE occupied bandwidth requirement when defining the UE ACLR value, thus, consider UE ACLR equal to 24 dB.

### 2.3.7	 Spurious emissions

Since it will be difficult to finalize if this frequency range will be delt as FR1 or FR2-1 band during the study item period, our preference is to follow what was done for the 10GHz during Rel-17 study item. Thus, we propose that RAN4 would adopt the general spurious emissions defined in TS 38.101-1 for the 14800-15300 MHz frequency range.

Proposal 14: RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1.

### 2.3.8	Blocking response

Given that no 3GPP band is yet specified for this frequency range, it is viable to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied for 14800 – 15300  MHz frequency range.

Proposal 15: For UE blocking response, RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied.

# 3	Conclusion

In this paper we have shared our views on the RF parameters for 14800 – 15350 MHz Frequency range. Our observations and proposals can be summarized for that frequency range as follows:

Proposal 1: RAN4 to agree on TDD as a baseline duplexing for 14800 – 15350 MHz frequency range and capture the following text in TR 38.922.

“Even though FDD is not precluded, most likely TDD will be used in this frequency range. In addition, SBFD is considered as a candidate duplexing method. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [8].”

Proposal 2: RAN4 to adopt 200MHz is a typical channel bandwidth. Additionally, it should be stated clearly in TR 38.922 that higher channel bandwidths should not be precluded for this frequency range.

Proposal 3a: RAN4 to not specify a fixed signal bandwidth but rather mention its dependency on SCS and number of RBs, similar to what was done to 7125-8400 MHz frequency range.

Proposal 3b: RAN4 to consider typical 60KHz as typical SCS for this frequency range and NPRB as given for 200MHz is TS 38.104.

Observation 1: The impact of modelling near-by users using spherical waves on system level evaluations is not clear in RAN4 and currently being studied in RAN1.

Proposal 4: RAN4 to not consider near-field impacts in this frequency range in its correspondence to WP5D.

Proposal 5: RAN4 to consider 1536 dual polarized antenna array based on 16*24 with 4 elements per subarray.

Proposal 6: RAN4 to adopt Category A and B SEM masks given in TR 38.921.

Proposal 7: For BS ACLR and ACS will be based on the outcome of the adjacent channel coexistence. In addition, RAN4 should make sure that the defined ACLR/ ACS are consistent with the defined ACLR/ACS for FR1 and FR2-1.

Observation 2: Based on the agreed simulation parameters for the adjacent channel coexistence work, 21 and 13 dB ACIR were observed for the downlink and uplink ACIR, respectively.

Proposal 8: RAN4 to adopt if ΔfOOB = 100 MHz for the definition of in-band and out-of-band blocking.

Proposal 9: RAN4 to agree on 23 dBm as UE typical maximum output power in the LS reply and to include in TR that higher power classes are not precluded in the normative phase of this frequency range.

Proposal 10: RAN4 to agree on 56 dBm as UE power dynamic range.

Proposal 10: RAN4 to consider 13 dB as UE noise figure for the LS reply.

Proposal 12: RAN4 to consider UE SEM given in TR 38.921, which is relaxed compared to NR general NR FR1 SEM at the FOOB edge ± 0 – 5 MHz by 3 dB.

Proposal 13: RAN4 to consider the UE occupied bandwidth requirement when defining the UE ACLR value, thus, consider UE ACLR equal to 24 dB.

Proposal 14: RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1.

Proposal 15: For UE blocking response, RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied.

# Reference

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson, RAN #103, Maastricht, Netherlands.

[2] R4-2417108, “WF on 15GHz BS parameters”, ZTE, RAN4#112bis, Hefei, China.

[3] R4-2417108, “WF on 15GHz UE parameters”, Qualcomm, RAN4#112bis, Hefei, China.

[4]	TR 38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz”, 3GPP V17.1.0.

# TP to TR 38.922 on 14800-15350 MHz Frequency Range

## 6	14800 - 15350 MHz frequency range

### 6.2.1	Duplex mode

### Even though FDD is not precluded, most likely TDD will be used in this frequency range. In addition, SBFD is considered as a candidate duplexing method. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [8].

### 6.2.2	Channel Bandwidth

### While a number of channel bandwidth will be specified for this frequency range, 200 MHz is considered as a typical channel bandwidth. Other values for channel bandwidths are not precluded for this range.

### 6.2.3	Signal Bandwidth

The signal bandwidth for a 200 MHz channel bandwidth signal is calculated based on the NR spectrum utilization

Signal bandwidth = NRB  x SCS x 12.

with typical values for NRB: Number of Resource block for 200 MHz channel bandwidth and 60kHz SCS, can be assumed as given in TS 38.104 [9], clause 5.3.2.

### 6.4 UE parameters

### 6.4.1.1	Power dynamic range

### The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. The power dynamic range could be 56 dB for 200 MHz channel bandwidth assuming -33 dBm as minimum UE output power.

### 6.4.1.5	Maximum output power

The typical UE maximum output power for the considered frequency ranges could be 23 dBm. Other UE power classes, are not precluded (corresponding ACLR limit will be adapted accordingly to avoid additional interference).