3GPP TSG-RAN WG4 Meeting #113	R4-2418621

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:	Discussion on UE demodulation and CSI performance requirements for 8Rx with MMSE-IRC

Agenda item:	7.16.2

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the revised WID [1] for “NR demodulation performance: Phase 5” has been approved. According the plan, this WI is targeting to be finished in RAN#109 meeting on Sep 2025.

Generally, the enhancements for UE demodulation performance focus on two scenarios, the first one is the inter-cell interference with MMSE-IRC receiver scenario, the second one is the intra-cell inter-user interference with MMSE-IRC receiver scenario.

RAN4#112 meeting is the first meeting for this topic, several high-level agreements are achieved [2]. Cover CQI reporting requirements under ICI for 8Rx is agreed meeting and most parameters for simulation assumptions are agreed in RAN4#112bis [3].

In this contribution, we provide our further analysis and views on the open issues for PDSCH demodulation performance parts and simulation assumptions for CQI part.

- Discussion

  - General

8Rx UE MMSE-IRC receiver coverage

In last RAN4#112bis meeting, agreement for initial simulation analysis is achieved to cover both baseline and simplified 8Rx MMSE IRC receiver. However, the criteria for one set of requirements need further discussion.

8Rx UE MMSE-IRC receiver coverage Agreement: - Cover both Baseline 8Rx UE MMSE-IRC receiver and simplified 8Rx UE MMSE-IRC Receiver for the initial simulation analysis. - TBD: Criteria used to decide if one set of requirements can be introduced covering both receiver types. - Note: Test coverage shall cover both receivers

Per our understanding, RAN4 should try our best to define only one set of test requirements to cover both receivers, no matter the UE receiver is baseline or simplified which should depends on UE implementation.

Proposal 1: For the criteria of both inter-cell interference scenario and intra-cell inter-user interference scenario, RAN4 should try our best to define only one set of test requirements to cover both receivers, no matter the UE receiver is baseline or simplified which should depends on UE implementation.

  - PDSCH requirements for inter-cell interference with MMSE-IRC receiver scenario

Rank

For the Rank of serving cell, companies agreed to evaluate rank 1,2 & 4 in initial simulation analysis and further discuss the down-selection for requirement definition. However, considering this is the rank discussion for inter-cell interference with MMSE-IRC receiver scenario, which means UE is supposed to be distributed at the cell edge where usually not suitable for high rank.

Rank of serving cell Agreement - Evaluate rank 1,2 & 4 in initial simulation analysis. TBD: Down-selection for requirement definition.

Observation 1: Considering this is the rank discussion for inter-cell interference with MMSE-IRC receiver scenario, which means UE is supposed to be distributed at the cell edge where usually not suitable for high rank.

Proposal 2: For the rank of serving cell for PDSCH demodulation performance with 8Rx inter-cell interference scenario, prefer only consider rank 1 or rank 2 for requirements definition.

Other parameters

For other parameters, there are two options proposed in last RAN4#112bis meeting as below:

Other parameters Agreement - Use 2Tx for interreference cell. - Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2 - SSB configuration as Rel-17 - Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells Remaining proposals - Option 1: - For PDSCH configuration, DMRS configuration, TRS configuration, HARQ, precoding, Tx EVM, Test metric and other parameters, reuse the configuration from 8Rx requirement as the baseline. - For PCI and SSB configuration of serving cell and interference cell(s), Rank/MCS, precoding, timing and frequency offset, and other interference cell modeling parameters, reuse the configuration from R17 2/4Rx inter-cell interference suppressing requirement as the baseline. - Option 2: For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point for initial evaluation.

Since the only difference between 8Rx MMSE-IRC receiver and Rel-17 2/4Rx MMSE-IRC receiver for inter-cell interference scenario is the number of Rx antennas, it seems no problem to reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point for initial evaluation.

Proposal 3: For PDSCH demodulation performance of 8Rx inter-cell interference scenario, for not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point for initial evaluation.

  - Wideband CQI reporting requirements for inter-cell interference with MMSE-IRC receiver scenario

Propagation condition

For the propagation condition, reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell should be a suitable starting point.

Proposal 4: For 8 wideband CQI reporting requirements with Rx inter-cell interference scenario, reuse the same propagation configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

CQI calculation algorithm

For the CQI calculation algorithm, since the test metric is the ratio γ of the throughput instead of absolute throughput value, it seems it’s not necessary to define two CQI calculation assumptions to match two 8Rx receiver assumptions.

Proposal 5: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, it’s not necessary to define two CQI calculation assumptions to match two 8Rx receiver assumptions.

Test metric

For the CQI test metric, below legacy test metric should be workable.

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Proposal 6: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, reuse legacy CQI test metric of Rel-17 2/4Rx.

Tested SINR value, throughput ratio and BLER X%

For the tested SINR value, throughput ratio and BLER X%, we suppose they should be decided based on further simulation results.

Proposal 7: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, the tested SINR value, throughput ratio and BLER X% should be decided based on further simulation results.

Rank

For the rank value, we assume reuse the same configuration of Rel-17 2/4Rx is enough.

Proposal 8: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, reuse the same rank configuration for Rel-17 2/4Rx, i.e., rank 1.

Antenna configuration and MIMO correlation for the serving cell

For the antenna configuration and MIMO correlation for the serving cell, we propose the similar configuration as legacy 2Rx and 4Rx, i.e., 2T8R with ULA Low.

Proposal 9: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, similar antenna and MIMO correlation configuration for the serving cell as legacy Rel-17 2/4Rx could be used, i.e., 2T8R with ULA Low.

Antenna configuration the interference cell

For the antenna configuration for the interference cell, we propose the similar configuration as legacy 2Rx and 4Rx, i.e., 1T8R.

Proposal 10: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, similar antenna configuration for the interference cell as legacy Rel-17 2/4Rx could be used, i.e., 1T8R.

CSI-IM and NZP CSI-RS configurations

For the CSI-IM and NZP CSI-RS configurations, we suppose the same configuration as Rel-17 2/4Rx should be used.

Proposal 11: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, the CSI-IM and NZP CSI-RS configurations should be configured as same as Rel-17 2/4Rx cases.

Interference power level

For the interference power level, we assume reuse the same configuration of Rel-17 2/4Rx is enough.

Proposal 12: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, reuse the same interference power level for Rel-17 2/4Rx, i.e., INR=10.04dB with rank 1.

Other parameters

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC (Option 1) as a starting point for initial evaluation.

Proposal 13: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, the CSI-IM and NZP CSI-RS configurations should be configured as same as Rel-17 2/4Rx cases.

  - Applicability rule and release independent

Test applicability

In order to balance the test coverage and test workload, for both demodulation performance tests and CQI reporting tests, we prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 14: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Furthermore, we also prefer to introduce similar applicability rules as below in Rel-17 for UE supporting both duplex mode TDD and FDD as legacy 2Rx/4Rx demodulation performance tests, for the purpose to reduce the workload.

- 5.1.1.10 Applicability of requirements for PDSCH with inter cell interference Tests; Applicability notes. All tests in Clause 5.2.2.1.15, 5.2.3.1.15, 5.2.2.2.16 and 5.2.3.2.16; If UE supporting both duplex mode TDD and FDD with 2RX, only test 1-1 in clause 5.2.2.1.15 and test 1-2 in clause 5.2.2.2.16 will be applied. If UE supporting both duplex mode TDD and FDD with 4RX, only test 1-1 in clause 5.2.3.1.15 and test 1-2 in clause 5.2.3.2.16 will be applied..

Proposal 15: For 8Rx inter-cell interference scenario, for demodulation performance tests, prefer to introduce similar applicability rules defined in section 5.1.1.10 for UE supporting both duplex mode TDD and FDD as legacy 2Rx/4Rx.

Release independent

Since 8Rx is introduced in Rel-18, we think it is reasonable to define the new requirements for 8Rx should be release independent from Rel-18.

Proposal 16: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18.

- Conclusion

In this contribution, we provide analysis and views on NR demodulation performance: Phase 5.

The observations and proposals could be summarized as:

Proposal 1: For the criteria of both inter-cell interference scenario and intra-cell inter-user interference scenario, RAN4 should try our best to define only one set of test requirements to cover both receivers, no matter the UE receiver is baseline or simplified which should depends on UE implementation.

Observation 1: Considering this is the rank discussion for inter-cell interference with MMSE-IRC receiver scenario, which means UE is supposed to be distributed at the cell edge where usually not suitable for high rank.

Proposal 2: For the rank of serving cell for PDSCH demodulation performance with 8Rx inter-cell interference scenario, prefer only consider rank 1 or rank 2 for requirements definition.

Proposal 3: For PDSCH demodulation performance of 8Rx inter-cell interference scenario, for not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point for initial evaluation.

Proposal 4: For 8 wideband CQI reporting requirements with Rx inter-cell interference scenario, reuse the same propagation configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

Proposal 5: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, it’s not necessary to define two CQI calculation assumptions to match two 8Rx receiver assumptions.

Proposal 6: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, reuse legacy CQI test metric of Rel-17 2/4Rx.

Proposal 7: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, the tested SINR value, throughput ratio and BLER X% should be decided based on further simulation results.

Proposal 8: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, reuse the same rank configuration for Rel-17 2/4Rx, i.e., rank 1.

Proposal 9: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, similar antenna and MIMO correlation configuration for the serving cell as legacy Rel-17 2/4Rx could be used, i.e., 2T8R with ULA Low.

Proposal 10: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, similar antenna configuration for the interference cell as legacy Rel-17 2/4Rx could be used, i.e., 1T8R.

Proposal 11: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, the CSI-IM and NZP CSI-RS configurations should be configured as same as Rel-17 2/4Rx cases.

Proposal 12: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, reuse the same interference power level for Rel-17 2/4Rx, i.e., INR=10.04dB with rank 1.

Proposal 13: For wideband CQI reporting requirements with 8Rx inter-cell interference scenario, the CSI-IM and NZP CSI-RS configurations should be configured as same as Rel-17 2/4Rx cases.

Proposal 14: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 15: For 8Rx inter-cell interference scenario, for demodulation performance tests, prefer to introduce similar applicability rules defined in section 5.1.1.10 for UE supporting both duplex mode TDD and FDD as legacy 2Rx/4Rx.

Proposal 16: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18.

- Reference

[1] RP-241297, Revised WID on NR demodulation performance: Phase 5, RAN#104 meeting, China Telecom, NTT DOCOMO

[2] R4-2413570, WF on NR demodulation performance: Phase 5, RAN4#112 meeting, China Telcom

[3] R4-2416555, WF for [112bis][320] NR_demod_Ph5_Part2_UE, RAN4#112bis meeting, Nokia