3GPP TSG-RAN WG4 Meeting #113	R4-2418622

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:	Discussion on RRM requirements for on-demand SSB SCell operation

Agenda item:	7.22.2.1

Document for:	Discussion

- Introduction

The SID [1] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved in the RAN#104 meeting [2] and updated in the RAN#105 meeting [3], which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#108 in June 2025. The objectives are as below:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In last RAN4#112 and RAN4#112bis meeting, the work plan, requirements scopes, on-demand SSB impact on neighbour cell measurement, and several issues on on-demand SSB based SCell L3 measurements have been agreed [8][9].

In this contribution, we share our views and analysis on the open issues for on-demand SSB based measurements and SCell activation impact.

- Discussion

  - LS from RAN1

RAN1 agreements about the timeline for on-demand SSB operation on SCell in RAN1#118 meeting are as below [6]. In RAN1#118 meeting, they agreed the working assumption that the value of T_min for MAC-CE based on-demand SSB indication is the same value as UE’s MAC-CE processing time for SCell activation command, and need our RAN4’s confirmation about this working assumption.

Agreement The previous RAN1 agreement made in RAN1#117 is revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ m+3Nslotsubframe,μ+1m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ m+3Nslotsubframe,μ+1 - FFS: Whether the value of T is predefined or indicated/configured by gNB - (Working assumption) T=T_min - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

In RAN1#118bis meeting, the agreement has been updated in [7] as below.

Agreement The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

From this description, we know if UE receives the MAC CE signalling which indicates the on-demand SSB transmission in slot#n, then slot#(n+m) is for PUCCH transmission with HARQ-ACK information, and at least until slot#(n+m+3Nslotsubframe,μ+1) is the slot for on-demand SSB burst(s) transmission with the first actually transmitted SSB index as Figure 2-1, which is as same as the timeline required for UE’s MAC CE processing for SCell activation. In other words, UE should be able to monitor on-demand SSB transmission from slot#(n+m+3Nslotsubframe,μ+1) based on on-demand SSB configurations. Furthermore, according latest agreement in RAN1, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB. The typical scenario for the time of on-demand SSB indication and SSB burst is Scenario A (there is a time interval between the first actually transmitted SSB and the slot#(n+m+3Nslotsubframe,μ+1)) in Figure 2-2. However, Scenario B (The first actually transmitted SSB is transmitted on the slot#(n+m+3Nslotsubframe,μ+1)) may also exist although it is rare.

Figure 2-1. Minimum requirements for Time instant A

Figure 2-2. Two scenarios for time of on-demand SSB indication and SSB burst

Per our understanding, the proposed slot#(n+m+3Nslotsubframe,μ+1) in RAN1 LS is the time for UE could receive on-demand SSB. And for typical scenario A in Figure 2-2, we think the working assumption in LS is enough, but for rare scenario B, we suppose further discussion is needed.

Proposal 1: RAN4 further discuss if the working assumption in LS is workable for the rare scenario B (The first actually transmitted SSB is transmitted on the slot#(n+m+3Nslotsubframe,μ+1)), if confirmed, RAN4 reply LS to RAN1 to confirm the working assumption of T_min= m+3Nslotsubframe,μ+1 can be as an agreement.

  - On-demand SSB based deactivated SCell L3 measurement (Case 1)

In last RAN4#112bis meeting, below agreements are achieved for on-demand SSB based deactivation Scell measurement (case 1).

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

Ending point of the fast L3 measurement time window

According above agreements, we know fast L3 measument should be supported within a time window, and the starting point of the time window is slot #n for UE receiving on-demand SSB activation indication. Although the ending point of the time window could be configured by gNB or predefined, but it’s more efficient to use predefined mode. Similar as the cell identification, the ending point of the time window could be defined as the time when UE first report the on-demand SSB based L3 SCell measurement results.

Proposal 2: RAN4 define the ending point of the fast L3 measurement time window could as the time when UE first report the on-demand SSB based L3 SCell measurement results.

For the relation between the fast L3 measurement time window and OD-SSB deactivation command in the time domain, which also should be related with the RAN1 agreement for deactivation of on-demand SSB transmission as below issue. Hence, we prefer to defer the discussion for relation between the fast L3 measurement time window and OD-SSB deactivation command in the time domain issue.

Proposal 3: RAN4 defer the discussion for the relation between the fast L3 measurement time window and on-demand SSB deactivation command in the time domain to the time when RAN1 get conclusion for deactivation of on-demand SSB transmission issue.

Agreement - For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options

Fast L3 measurement requirement

The fast L3 measurement only should be applied within a time window which has on-demand SSB transmission. For the deactivated SCell measurement, currently, the time periods are defined based on measCycleSCell which is ENUMERATED from {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}. On the one hand, the duration of on-demand SSB should be long enough to guarantee the know cell condition, otherwise current definition is not suitable anymore. On the other hand, long on-demand SSB duration may lose the gain of network energy saving. Therefore, it’s better to define the deactivated SCell measurement requirements based on the periodicity of on-demand SSB instead of measCycleSCell and/or DRX cycle. Latest RAN1 agreements mentioned the periodicity of on-demand SSB can be configured and indicated. Hence the fast L3 measurement should be based on the periodicity of on-demand SSB instead of measCycleSCell and DRX cycles.

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

Proposal 4: Fast L3 measurement should be based on the periodicity of on-demand SSB instead of measCycleSCell and DRX cycle.

Slow mode L3 measurement after the fast L3 measurement time window

After the fast L3 measurement time window, three options are proposed in last meeting. Per our understanding, fallback to legacy requirements is the most appropriate selection to balance the performance for both network and UE.

Propose 5: After the time window for fast L3 measurement, UE can perform deactivated SCell measurement with a slow mode which fallback to legacy requirements.

  - On-demand SSB based deactivated SCell L3 measurement (Case 2)

Always-on SSB and on-demand SSB time/frequency domain relation

For the always-on SSB and on-demand SSB overlapping discussion, we have below conclusion in last RAN4 meeting.

Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement.

Per our understanding, Scenario 1 with on-demand SSB and always-on SSB with same SSB carrier frequency, same offset but different periodicities, which means on-demand SSB may contain always-on SSB during the transmission of on-demand SSB (assume the typical configuration as on-demand SSB periodicity is smaller than of always-on SSB periodicity), and when OD-SSB is terminated, always-on SSB is still transmitted. In other words, Scenario 1 is the scenario which may has both time domain overlap. Scenario 2 may have frequency domain overlap. Scenario 3 do not have the overlapping issues but need to consider on-demand SSB and always-on SSB with different carrier frequencies problem. Lastest RAN1 agreements including Mux-case #1 and Mux-Case #2 in RAN1#118bis meeting [7] are as below

Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

Observation 1: On-demand SSB and always-on SSB may overlap at both time and frequency domain based on RAN1 latest agreement.

Since we have sent out LS [10] to RAN1 to query the periodicity, time domain location, frequency domain location and spatial relation for on-demand SSB and always-on SSB configurations, it’s reasonable to first consider the Scenario 1.

Propose 6: Wait for the reply from RAN1 on the query of the periodicity, time domain location, frequency domain location and spatial relation for on-demand SSB and always-on SSB configurations, and at the same time continue the discussion for Scenario 1.

On-demand SSB based deactivated SCell L3 measurement (Case 2)

For both L3 and L1 measurements based on Scenario 1, measurements based on on-demand SSB or always-on SSB only, or measurements based on both on-demand SSB and always-on SSB need to be considered. For L1 measurement, RAN1 has below agreements.

Agreement - For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not

Similar as L1 measurement, L3 measurement also need to consider above option 1 and option 2.

Propose 7: For L3 measurement, consider both option 1 (measurement based on both on-demand SSB and always-on SSB) and option 2 (measurement based on one of on-demand SSB and always-on SSB) similar as RAN1 agreement for L1 measurement, and further clarify how to judge or distinguish the measurement results of option 1 and option 2.

OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered (Case 2)

For Case 2, the fast L3 measurement time window mechanism seems still workable when both on-demand SSB and always-on SSB exist. Therefore, for on-demand SSB based deactivated SCell L3 measurement for Case 2, when on-demand SSB transmission is triggered, a fast L3 measurement within the time window should be applied based on the periodicity of on-demand SSB, and after the fast L3 measurement time window, the slow mode measurement which fallback to legacy requirement following the measCycleSCell and/or DRX.

Observation 2: For Case 2, the fast L3 measurement time window mechanism seems still workable when both on-demand SSB and always-on SSB exist.-

Propose 8: For on-demand SSB based deactivated SCell L3 measurement for Case 2, when on-demand SSB transmission is triggered, a fast L3 measurement within the time window should be applied based on the periodicity of on-demand SSB, and after the fast L3 measurement time window, the slow mode measurement should be applied which fallback to legacy requirement following the measCycleSCell and/or DRX.

  - On-demand SSB based SCell activation

Known/Unknown SCell

There is different SCell activation time requirements for known SCell and unknown SCell due to different synchronization time. For the SCell is known or unknown, we have below definition based on valid measurement reports and detectable conditions during a time period. Considering the potential new time periods defined for on-demand SSB and the short duration character of on-demand SSB, the definition of known/unknown SCell for on-demand SSB should be updated.

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown.

Generally speaking, it seems the principle for the known cell definition “During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command, the UE has sent a valid measurement report for the SCell being activated and the SSB measured remains detectable according to the cell identification conditions” is still workable. However, for Case 1, only on-demand SSB could be measured during the defined period, such as max(5*measCycleSCell,  5*DRX cycles), and on-demand SSB may not exist anymore after the defined period since it’s a kind of temporary signal. In this case, a long-defined period for the know cell definition may not appropriate. For Case 2, if the known cell conclusion is get based on always-on SSB, it may be unapplicable for on-demand SSB, and vice versa. Therefore, the updated definition of known/unknown cell should be defined carefully to avoid problematic situations.

Observation 3: Generally speaking, it seems current known/unknown cell definition is still workable.  However, for Case 1, only on-demand SSB could be measured during the defined period, such as max(5*measCycleSCell,  5*DRX cycles), and on-demand SSB may not exist anymore after the defined period since it’s a kind of temporary signal. In this case, a long-defined period for the know cell definition may not appropriate. For Case 2, if the known cell conclusion is get based on always-on SSB, it may be unapplicable for on-demand SSB, and vice versa.

Proposal 9: The definition of known/unknown SCell should be updated for on-demand SSB, and should be discussed carefully to avoid problematic situations.

- Conclusion

In this contribution, we provide analysis and views on on-demand SSB related topic.

The observations and proposals could be summarized as:

Proposal 1: RAN4 further discuss if the working assumption in LS is workable for the rare scenario B (The first actually transmitted SSB is transmitted on the slot#(n+m+3Nslotsubframe,μ+1)), if confirmed, RAN4 reply LS to RAN1 to confirm the working assumption of T_min= m+3Nslotsubframe,μ+1 can be as an agreement.

Proposal 2: RAN4 define the ending point of the fast L3 measurement time window could as the time when UE first report the on-demand SSB based L3 SCell measurement results.

Proposal 3: RAN4 defer the discussion for the relation between the fast L3 measurement time window and on-demand SSB deactivation command in the time domain to the time when RAN1 get conclusion for deactivation of on-demand SSB transmission issue.

Proposal 4: Fast L3 measurement should be based on the periodicity of on-demand SSB instead of measCycleSCell and DRX cycle.

Propose 5: After the time window for fast L3 measurement, UE can perform deactivated SCell measurement with a slow mode which fallback to legacy requirements.

Observation 1: On-demand SSB and always-on SSB may overlap at both time and frequency domain based on RAN1 latest agreement.

Propose 6: Wait for the reply from RAN1 on the query of the periodicity, time domain location, frequency domain location and spatial relation for on-demand SSB and always-on SSB configurations, and at the same time continue the discussion for Scenario 1.

Propose 7: For L3 measurement, consider both option 1 (measurement based on both on-demand SSB and always-on SSB) and option 2 (measurement based on one of on-demand SSB and always-on SSB) similar as RAN1 agreement for L1 measurement, and further clarify how to judge or distinguish the measurement results of option 1 and option 2.

Observation 2: For Case 2, the fast L3 measurement time window mechanism seems still workable when both on-demand SSB and always-on SSB exist.-

Propose 8: For on-demand SSB based deactivated SCell L3 measurement for Case 2, when on-demand SSB transmission is triggered, a fast L3 measurement within the time window should be applied based on the periodicity of on-demand SSB, and after the fast L3 measurement time window, the slow mode measurement should be applied which fallback to legacy requirement following the measCycleSCell and/or DRX.

Observation 3: Generally speaking, it seems current known/unknown cell definition is still workable.  However, for Case 1, only on-demand SSB could be measured during the defined period, such as max(5*measCycleSCell,  5*DRX cycles), and on-demand SSB may not exist anymore after the defined period since it’s a kind of temporary signal. In this case, a long-defined period for the know cell definition may not appropriate. For Case 2, if the known cell conclusion is get based on always-on SSB, it may be unapplicable for on-demand SSB, and vice versa.

Proposal 9: The definition of known/unknown SCell should be updated for on-demand SSB, and should be discussed carefully to avoid problematic situations.

- Reference

[1] RP-234061, “New WID: Enhancements of network energy savings for NR”, Ericsson

[2] RP-241650, “Revised WID: Enhancements of network energy savings for NR”, RAN#104 meeting, Ericsson, Apple

[3] RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN#105 meeting, Ericsson

[4] RP-241182, “Status Report for WI – Enhancements of network energy saving for NR”, RAN#104 meeting, Ericsson, Apple

[5] RP-241866, “Status Report for WI – Enhancements of network energy saving for NR”, RAN#105 meeting, Ericsson, Apple

[6] R1-2407565, “LS on timeline for On-demand SSB operation on SCell”, RAN1#118 meeting

[7] RAN1 Chair’s Notes RAN1#118bis v20, RAN1#118bis meeting

[8] R4-2413885, “WF on Netw_Energy_NR_enh”, RAN4#112 meeting, Ericsson

[9] R4-2416916, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, RAN4#112bis meeting, Ericsson

[10] R4-2416913, “LS on SSB relation in On-demand SSB and SSB adaptation on SCell”, RAN4#112bis meeting