3GPP TSG-RAN WG4 Meeting #113		R4-2418626

Orlando, US, 18th – 22nd November 2024

Agenda item:	7.22.2.1

Source:	Intel Corporation

Title:	Discussion on RRM requirements for On-demand SSB SCell operations

Document for:	Discussion

# Introduction

The Rel-19 NR network energy saving enhancements work item was approved in [1]. The work item Core part objectives for on-demand SSB are provided below.

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this paper we provide analysis on the RRM requirements for on-demand SSB in SCell.

# On-demand SSB RRM requirements

## On-demand SSB in SCell

In general this part of the discussion is about the On-demand SSB that is used by UE for SCell time/frequency synchronization and L1 or L3 measurements during or outside SCell activation when the UE is configured with carrier aggregation operations.

In RAN4, it is natural that we look at the activation procedure and its corresponding RRM requirements first and then move to discuss the serving cell measurement requirements enhancements after CA is configured to the UE (the measurements enhancements based on On-demand SSB aim to serve the activation procedure eventually) or after SCell activation is complete.

Correspondingly we can identify the below aspects of RAN4 RRM specification impacts:

- Enhanced deactivated SCell measurement delay requirements based on On-demand SSB

- Enhanced SCell activation delay requirements based on On-demand SSB

  - Enhanced L3 measurement delay (reuse existing measurement requirements)

  - Known cell conditions based on on-demand SSB

  - Enhanced L1 measurement delay

The below figure helps to understand the RRM scope regarding On-demand SSB enhancements in RAN4.

Figure 1 Correspondence between RAN1 scenarios and RRM scope

## On-demand SSB and always-on SSB

No always-on SSB transmission

RAN1 identified the following two cases in their discussions.

- Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell

RAN4 will be looking at the scenarios where On-demand SSB is configured in the SCell due to energy saving purposes and seek to minimize the impact of less SSB transmissions and to enhance the performance by allowing the UE to measure on the On-demand SSB specifically transmitted for the SCell activation procedure.

From RAN4 perspective, it is possible that at least for some of the scenarios there is no always-on SSB on the target SCell. SSB-less operations are applicable to intra-band and inter-band carrier aggregations already in the existing specifications. With the help of On-demand SSB, we see great benefits as it completes the SSB-less SCell systematically.

In fact we should focus on the cases where there is no always-on SSB on the target cell since this is the major use case in Rel-19 for this work item to compensate for SSB-less operations 3GPP introduced in Rel-18.

In the last WG meeting, we had discussions on case #1 and case #2 and come to some agreements which are shown in the below box.

Issue 1-1-3: OD-SSB type Agreement: - RAN4 to first take NCD-SSB as a start point for OD-SSB requirements. Issue 1-3-1: Always-on SSB and OD-SSB time/frequency domain relation Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement.

Proposal 1: RAN4 focuses on Case #1 where there is no always-on SSB transmitted on the target cell.

Regarding issues 1-3-1, our views are that the OD-SSB should not be limited to any biased configuration just to align the frequency and time domain positions with the always-on SSB. In the meanwhile, RAN1 is also having this discussion hence we don’t need to duplicate discussions between working groups on this matter. Our opinion on this is that if there is always-on SSB transmitted, on-demand SSB needs to be flexible enough to accommodate different UE-s in the SCell hence limiting the configurations does not help on the system performance.

Proposal 2: No limitation is applied to the time and frequency allocation between OD-SSB and always-on SSB in RAN4 spec.

For UE supporting Rel-19 OD-SSB features for case #1 where there is no always-on SSB, the UE should also support SSB-less SCell operations. And the UE is required to meet the requirements RAN4 introduces in Rel-19.

For UE additionally supporting case #2 where there is both OD-SSB and always-on SSB transmitted in the network, the UE should meet higher expectations so that it can cope with all different kinds of configurations between the two sets of SSB-s.

Proposal 3: For UE supporting Rel-19 OD-SSB features for case #1 where there is no always-on SSB, the UE should also support SSB-less SCell operations. And the UE is required to meet the requirements RAN4 introduces in Rel-19.

## Deactivated SCell measurements based on on-demand SSB

Deactivated SCell measurements based on On-demand SSB

Regarding scenario #2 shown in fig.1, the idea is to allow On-demand SSB transmissions before the activation command where the SCell is a deactivated SCell. Meanwhile the UE carries out deactivated SCell measurements based on On-demand SSB. RAN4 specifies the enhanced deactivated SCell measurement requirements based on On-demand SSB for this case.

For Case #1 where there is no always-on SSB, RAN4 agrees in the last meeting that:

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

Meanwhile RAN1 #118bis reached agreements on the activation and deactivation of the OD-SSB transmissions:

Agreement For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options

Putting the agreements together we think the best way to define deactivated SCell measurements for case #1 where no always-on SSB is transmitted is that we allow the UE to measure the OD-SSB after activation of the OD-SSB and allow the UE to seize measurements after the deactivation of the OD-SSB transmissions.

The network should have the privilege to configure to the UE the measurement cycle in a similar way as in the legacy deactivated SCell measurements. This is because the OD-SSB is a cell-specific configuration, and the network needs to configure to the UE a UE-specific measurement cycle. By this, the UE follows the configured [MeasCycleODSSB] to carry out OD-SSB based deactivated SCell measurements.

Proposal 4: In Case #1, UE starts deactivated SCell measurements on the SSB-less SCell after OD-SSB transmission is activated by the network and the measurement period is according to the configured measurement cycle dedicated for OD-SSB from the network.

Proposal 5: In Case #1, UE stops deactivated SCell measurements after the OD-SSB transmission is deactivated.

Case #2 where always-on SSB is transmitted

In the last meeting RAN4 agreed that for Case #2 if the OD-SSB transmission is not activated, the UE follows legacy measurement requirements and measurement cycle for deactivated SCell measurements.

RAN4 also agreed to further define the requirements when OD-SSB transmission is activated together with always-on SSB-s. The agreements are copied in the below box.

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreements: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE) - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO) - Option 3: RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE) - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm) - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

In our view, for capable UE the performance enhancements by sending both sets of SSB-s have to be guaranteed. This means that the UE supporting Case #2 should be capable of utilizing the OD-SSB for better mobility performance on the deactivated SCell. It is required that the capable UE should measure the OD-SSB when the transmission is activated, with a measurement cycle that is configured by the network. Note that this measurement cycle is necessary since the OD-SSB is cell level but the UE needs a UE level configuration.

Proposal 6: In Case 2, RAN4 requirement is defined based on the network indicated measurement cycle dedicated for OD-SSB when OD-SSB transmission is activated.

## SCell activation based on on-demand SSB

Known cell conditions based on On-demand SSB during SCell activation

In the last meeting RAN4 agreed the principles for known SCell criteria as in the below box.

Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded.

Regarding Case #1 where there is no always-on SSB transmitted on the target SCell, the known cell conditions are based on on-demand SSB only.

We propose that RAN4 specifies the known cell conditions based on OD-SSB in a similar way as in the legacy requirements:

Proposal 7: In Case #1, SCell in FR1 with OD-SSB is known if it has been meeting the following conditions:

- During the period equals max(5 * [measCycleODSSB], 5 * DRX cycle) for FR1 before the reception of the SCell activation command:

  - The UE has sent a valid measurement report for the SCell being activated and

  - The OD-SSB measured remains detectable according to the cell identification conditions.

- The OD-SSB measured during the period equals max(5 * measCycleODSSB, 5 * DRX cycle) also remains detectable during the SCell activation delay according to the cell identification conditions.

Otherwise the OD-SSB SCell in FR1 is unknown.

For Case #2 when there are both always-on SSB and OD-SSB transmissions, the SCell is considered a known SCell when measurement report is sent either based on always-on SSB or OD-SSB during max(5 * measCycleSCell, 5 * measCycleODSSB, 5 * DRX cycle) period. The corresponding side conditions should also be fulfilled that the target SSB of the measurement report should remain detectable all the time during the period. Note that mostly measCycleSCell is probably larger than measCycleODSSB, and since measCycleSCell is the legacy/existing implementation, we would still like to propose this method.

Proposal 8: For Case #2, the SCell is considered a known SCell when measurement report is sent either based on always-on SSB or OD-SSB during max(5 * measCycleSCell, 5 * measCycleODSSB, 5 * DRX cycle) period before the activation command is received at the UE.

L1 and L3 measurements based on On-demand SSB during SCell activation

During a typical activation procedure, the UE needs to get the L3 results on the target SCell first and L1 measurements are carried out on the SCell for better beam information. One of the typical applications of On-demand SSB is to trigger transmissions together with SCell activation command and to seize transmissions after receiving UE activation CSI report. RAN1 also discusses about other deactivation procedures in the meantime. It is possible that during the whole process we could allow the UE to measure on On-demand SSB-s for either L3 or L1 measurements or both.

It is likely that RAN1 confirms reusing physically layer designs for the On-demand SSB-s for activation procedure. RAN4 specifies the L3 and L1 measurements requirements during the activation based on the On-demand SSB configurations.

In the last meeting, RAN4 agreed on which type of SSB is used during SCell activation:

Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG

So we propose:

Proposal 9: Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot (n + THARQ + T activation_time + T CSI_Reporting ) / NR slot length.

If the SCell is known and belongs to FR1, T activation_time is based on OD-SSB: T FirstODSSB + Trs + 5ms.

If the SCell is unknown and belongs to FR1, L3 measurements are based on OD-SSB for both Case #1 and Case #2, where T activation_time is:

- 6ms + T FirstODSSB + T SMTC_MAX + Trs + T L1-RSRP, measure + T L1-RSRP, report + T HARQ + max(T uncertainty_MAC + T FineTiming + 2ms, T uncertainty_RRC + T RRC_delay) if SP CSI-RS is used,

- 3ms + T FirstODSSB + T SMTC_MAX + Trs + T L1-RSRP, measure + T L1-RSRP, report + max(T HARQ + T uncertainty_MAC + 5ms + T FineTiming, T uncertainty_RRC + T RRC_delay) if periodic CSI-RS is used.

In the above functions, T L1-RSRP, measure is based on the OD-SSB for both case #1 and case #2.

In the previous release we specified fast SCell activation delay requirements for UE capable of fast SCell activations on intra-band contiguous CA and inter-band CA where the target SCell has similar spatial characteristics as the PCell. The below requirements can be referred to.

If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided - The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and - The difference of the reception power with the contiguous active serving cell is <= 6dB, and - The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band. For a UE supporting [scellWithoutSSB-interband], if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and - The [EPRE] difference at the UE is smaller than or equal to [9] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], and - The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell. where the reference serving cell can be indicated by higherlayer parameter [SSB-less-Referencecell]. If UE is not indicated with [SSB-less-Referencecell], the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured.

Thus, the mentioned Rel-19 SCell activation requirements which are based on OD-SSB apply only when the UE indicates support of Rel-19 NES UE features but do not support the fast SCell activation features or the conditions of the target SCell do not meet the specified conditions. When the UE supports fast SCell activation for intra-band contiguous CA or inter-band CA and indicating support of scellWithoutSSB or scellWithoutSSB-InterBandCA-r18 and the conditions meet fast SCell activation conditions, the Rel-19 requirements do not apply.

Proposal 10: When the UE supports fast SCell activation for intra-band contiguous CA or inter-band CA and indicating support of scellWithoutSSB or scellWithoutSSB-InterBandCA-r18 and the conditions meet fast SCell activation conditions, the Rel-19 requirements do not apply.

# Conclusion

In this paper we provide analysis on the RRM requirements for on-demand SSB in SCell.

Proposal 1: RAN4 focuses on Case #1 where there is no always-on SSB transmitted on the target cell.

Proposal 2: No limitation is applied to the time and frequency allocation between OD-SSB and always-on SSB in RAN4 spec.

Proposal 3: Different requirements apply between Case #1 and Case #2, where Case #1 has no always-on SSB on the target SCell, but Case #2 has both OD-SSB and always-on SSB on the target SCell.

Proposal 4: In Case #1, UE starts deactivated SCell measurements on the SSB-less SCell after OD-SSB transmission is activated by the network and the measurement period is according to the configured measurement cycle dedicated for OD-SSB from the network.

Proposal 5: In Case #1, UE seizes deactivated SCell measurements after the OD-SSB transmission is deactivated since there is no SSB transmission.

Proposal 6: In Case 2, RAN4 requirement is defined based on the network indicated measurement cycle dedicated for OD-SSB when OD-SSB transmission is activated.

Proposal 7: SCell in FR1 with OD-SSB is known if it has been meeting the following conditions:

- During the period equals max(5 * [measCycleODSSB], 5 * DRX cycle) for FR1 before the reception of the SCell activation command:

  - The UE has sent a valid measurement report for the SCell being activated and

  - The OD-SSB measured remains detectable according to the cell identification conditions.

- The OD-SSB measured during the period equals max(5 * measCycleODSSB, 5 * DRX cycle) also remains detectable during the SCell activation delay according to the cell identification conditions.

Otherwise the OD-SSB SCell in FR1 is unknown.

Proposal 8: For Case #2, the SCell is considered a known SCell when measurement report is sent either based on always-on SSB or OD-SSB during max(5 * measCycleSCell, 5 * measCycleODSSB, 5 * DRX cycle) period before the activation command is received at the UE.

Proposal 9: Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot (n + THARQ + T activation_time + T CSI_Reporting ) / NR slot length.

If the SCell is known and belongs to FR1, T activation_time is based on OD-SSB: T FirstODSSB + Trs + 5ms.

If the SCell is unknown and belongs to FR1, L3 measurements are based on OD-SSB for both Case #1 and Case #2, where T activation_time is:

- 6ms + T FirstODSSB + T SMTC_MAX + Trs + T L1-RSRP, measure + T L1-RSRP, report + T HARQ + max(T uncertainty_MAC + T FineTiming + 2ms, T uncertainty_RRC + T RRC_delay) if SP CSI-RS is used,

- 3ms + T FirstODSSB + T SMTC_MAX + Trs + T L1-RSRP, measure + T L1-RSRP, report + max(T HARQ + T uncertainty_MAC + 5ms + T FineTiming, T uncertainty_RRC + T RRC_delay) if periodic CSI-RS is used.

In the above functions, T L1-RSRP, measure is based on the OD-SSB for both case #1 and case #2.

Proposal 10: When the UE supports fast SCell activation for intra-band contiguous CA or inter-band CA and indicating support of scellWithoutSSB or scellWithoutSSB-InterBandCA-r18 and the conditions meet fast SCell activation conditions, the Rel-19 requirements do not apply.

# References

- RP-234065, New WID: Enhancements of network energy savings for NR, Moderator (Ericsson)