3GPP TSG-RAN WG4 Meeting # 113	R4-2418637

Orlando, US, Nov.18th – 22nd 2024

Title: 	(NR_2step_RACH-Perf) Discussion on test configuration in 2-step RACH RRM test cases

Source: 	Huawei, HiSilicon, Starpoint

Agenda item:	4.5.1

Document for:	Discussion

Introduction

Several issues are met during practical testing of 2-step RACH RRM test cases. This contribution discusses the solutions of these issues.

Discussion

Definition of first MsgA PUSCH transmit power

In 2-step RACH test cases, DUT is required to transmit the first MsgA PUSCH with a power of 0.6+3μ+2 dBm. E.g., TC A.4.3.2.2.3 sub-test 1. Such a power level is unreasonably high. The DUT is nearly impossible to achieve considering bandwidth of MsgA PUSCH is set to only 2RBs in MsgA RMCs.

A.4.3.2.2.3.2.1 MsgA Transmission To test the UE behaviour specified in Clause 6.2.2.3.1.1 the System Simulator shall receive the MsgA with a preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured msgA-RSRP-ThresholdSSB. In addition, the power applied to all MsgA transmission shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first MsgA preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

After checking WF [1] we realized that it’s actually a copy-and-paste mistake. In [1], it’s required that first MsgA PUSCH shall be transmitted with a power 3μ+2 dB higher than first MsgA PRACH power, rather than at the absolute power of 3*(2+μ) dBm.

R4-2017254: - Define 2-step RACH RRM performance requirements with MsgA PUSCH power calculated with a shift of 3.μ + 2 dB in comparison to the MsgA PRACH power. Definition of μ should be clarified, e.g., μ = 0 for SCS = 15kHz.

Observations 1: In WF [1], transmit power of first MsgA PUSCH is defined as the 3μ+2 dB power shift to first MsgA PRACH power, rather than the absolute power of 3μ+2 dBm.

It is recommended to be consistent with WF [1], i.e. defining transmit power of first MsgA PUSCH power by specifying an power offset relative to first MsgA PRACH.

Proposal 1: Defining transmit power of first MsgA PUSCH power by specifying its power offset relative to first MsgA PRACH.

Configuration of msgA-DeltaPramble

In [1], the power offset between MsgA PUSCH and MsgA PRACH in [1] is derived based on following test parameters. Which is also the case in current MsgA RMCs.

WF R4-2017254: - Define 2-step RACH RRM performance requirements with the parameters defined in the table below: Parameter; Value. MsgA number of PRBs; 2. msgA-DeltaPreamble; 3 dB. msgA-Alpha; alpha1. deltaMCS; Disabled.

Note that msgA-DeltaPramble is set to 3dB. However, it’s impossible to do so because msgA-DeltaPramble can only be an even value as specified in 38.331.

msgA-DeltaPreamble-r16 INTEGER (-1..6) OPTIONAL -- Need R msgA-DeltaPreamble Power offset of msgA PUSCH relative to the preamble received target power. Actual value = field value * 2 [dB] (see TS 38.213 [13], clause 7.1).

Observations 2: msgA-DeltaPramble can only be even value as specified in 38.331.

It’s suggested to change msgA-DeltaPramble to 2dB in MsgA RMCs.

Proposal 2: Set msgA-DeltaPramble = 2dB in MsgA RMCs.

Calculation of power offset between first MsgA PUSCH and first MsgA PRACH

The power offset between first MsgA PUSCH and MsgA PRACH needs to be updated after changing msgA-DeltaPramble. current power offset is derived in [2]. Our calculation also reuses the method in [2], but additionally corrects a mistake in [2].

R4-2014934 [1]: One additional element that is still not defined is the power of MsgA PUSCH. The power of MsgA PUSCH is specified in clause 7.1.1 of 38.211 which can be simplified as [5]: PPUSCH=minPCMAXPO_PUSCH+10log102μ.MRBPUSCH+α.PL+∆TF+f; Equation 1. When MsgA PUSCH is considered, the values in the equation above can be defined as: - PO_PUSCH=PO_PRE+∆MsgA_PUSCH, where PO_PRE is msgA-PreambleReceivedTargetPower, and ∆MsgA_PUSCH is msgADeltaPreamble - μ indicates the numerology - MRBPUSCH is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for PUSCH transmission occasion, - α is defined by msgA-Alpha, - PL – is the path loss, - ∆TF – is the transport format factor, and depends on deltaMCS, and - f – is the PUSCH power control adjustment state, which is zero for the first MsgA PUSCH transmission. NOTE 1: If some of the parameters above are not defined, they may be replaced by 4-step RACH parameters. Refer to 38.213 [5] for the complete description of each variable. Considering that the power for PRACH and MsgA PRACH is calculated as: PPRACH=minPCMAXPPRACH,target+PL; Equation 2. Equation 1 can simplified for the first transmission of MsgA PUSCH by considering the MsgA PRACH as: PMsgA PUSCH=minPCMAXPMsgA PRACH+ΔMsgA_PUSCH+10log102μ.MRBPUSCH+α-1 .PL+∆TF; Equation 3. Equation 3 can be further simplified by choosing the following parameters: - ΔMsgA_PUSCH = 3 dB, - MRBPUSCH= 2, for 2 PRBs, - α = 1 - And deltaMCS=disabled, which results in ∆TF=0 PMsgA PUSCH=minPCMAXPMsgA PRACH+3+10log102μ+1=minPCMAXPMsgA PRACH+3.μ+2; Equation 3.

Note that In Equation 3 PO_PRE (msgA-PreambleReceivedTargetPower) is substituted by PRACH received target power PPRACH,target. However, PPRACH,target for 1st MsgA PRACH should be calculated as the sum of PPRACH,target and DELTA_PREAMBLE as specified in 38.321 cl. 5.1.3a.

TS 38.321 cl.5.1.3a 1> select the value of DELTA_PREAMBLE according to clause 7.3; - set PREAMBLE_RECEIVED_TARGET_POWER to msgA-PreambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP; TS 38.321 cl.7.3 Table 7.3-2: DELTA_PREAMBLE values for short preamble formats. Preamble Format; DELTA_PREAMBLE values (dB). A1; 8 + 3 × μ. A2; 5 + 3 × μ. A3; 3 + 3 × μ. B1; 8 + 3 × μ. B2; 5 + 3 × μ. B3; 3 + 3 × μ. B4; 3 × μ. C0; 11 + 3 × μ. C2; 5 + 3 × μ.

So power offset should be calculated as:

PMsgA PUSCH-PMsgA PRACH=ΔMsgA_PUSCH2 dB-DELTA_PREAMBLE +3μ+1

PRACH preamble format A1/A2/C2 are used for FR1 FDD /FR1 TDD /FR2 TDD test configurations according to MsgA RMCs (prach-ConfigurationIndex = 102/190 for FR1/FR2 respectively) and 38.211 Table 6.3.3.2-2/3/4. Then we have

PMsgA PUSCH-PMsgA PRACH=-3for FR1 FDD0for FR1/FR2 TDD in [dB]

Proposal 3: Specify the power offset of first MsgA PUSCH to first MsgA PRACH in test requirements as follows:

- For FDD test configurations, power offset is -3dB;

- For TDD test configurations, power offset is 0dB;

Configuration of ra-PreambleIndex

In last meeting [3] is agreed to update ra-PreambleIndex to 40 in 4-step CFRA RMCs to solve the issue that DUT is unable to receive the RAR with RAPID = 50 correctly. Same issue also exists in 2-step CFRA TCs. So we suggest also change ra-PreambleIndex = 40 in 2-step RACH RMCs.

Proposal 3: Set ra-PreambleIndex = 40 in 2-step CFRA RMCs to align with 4-step CFRA RMCs.

Conclusions

This contribution provides further discussion on issues met during practical testing of 2-step RACH RRM test cases. The following observations and proposals are provided:

Observations 1: In WF [1], transmit power of first MsgA PUSCH is defined as the 3μ+2 dB power shift to first MsgA PRACH power, rather than the absolute power of 3μ+2 dBm.

Proposal 1: Defining transmit power of first MsgA PUSCH power by specifying its power offset relative to first MsgA PRACH.

Observations 2: msgA-DeltaPramble can only be even value as specified in 38.331.

Proposal 2: Set msgA-DeltaPramble = 2dB in MsgA RMCs.

Proposal 3: Specify the power offset of first MsgA PUSCH to first MsgA PRACH in test requirements as follows:

- For FDD test configurations, power offset is -3dB;

- For TDD test configurations, power offset is 0dB;

Proposal 3: Set ra-PreambleIndex = 40 in 2-step CFRA RMCs to align with 4-step CFRA RMCs.

Corresponding CR for 2-step RACH RMCs and Rel-16 2-step RACH test cases is given in [4], and CR for Rel-17 RedCap 2-step RACH test cases is given in [5].

Reference

- R4-2017254, WF on 2-step RACH RRM test cases, ZTE Corporation

- R4-2014934, 2-step RACH RRM performance requirements, Nokia, Nokia Shanghai Bell

- R4-2412162, (NR_newRAT-Perf) Correction to PRACH RMCs_R15, Huawei, Hisilicon

- R4-2418634, (NR_2step_RACH-Perf) Correction to 2-step RACH RRM RMCs and test cases_R16, Huawei, Hisilicon, Starpoint

- R4-2418641, (NR_redcap-Perf) Correction to RedCap 2-step RACH RRM test cases_R17, Huawei, Hisilicon, Starpoint