3GPP TSG-RAN WG4 Meeting # 113	R4-2418654

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.22.2.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES on adaptation

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN 4 discussed the RRM impacts of SSB adaption in last meeting with agreements and issues captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

The progress in RAN4 discussion in previous meetings are summarized as follows:

- RAN4#112 Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode - Agreement - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode. - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point. - Wayforward: - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress. - RAN4#112bis Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) Issue 1-2-1: Requirements need updating Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

Per RAN1 discussion, the progress are summarized as follows:

Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior). - No spec impact

Based on the discussion in last RAN1 meeting, there is not much progress compared with last meeting w.r.t SSB adaptation. In this contribution, we continue discussing the issues not discussed in last RAN4 meeting.

## SSB adaptation for L3 measurement

For SSB adaptation for L3 measurement, RAN4 send a LS to RAN2 to ask for the potential impacts on SMTC [2]. Based on the discussion in last meeting, RAN4’s common understanding is that the L3 measurement is still based on SMTC, which means UE is still expected to detect/measure Cells within SMTC. Regarding whether there is any impact on SMTC (i.e. new SMTC or SMTC adaptation as mentioned by companies) is up to RAN2 discussion. For instance, if gNB can guarantee that SSB can always be found in the configured SMTC, there is no any impacts on L3 measurement behaviour/requirements. If RAN2 decides to introduce SMTC adaptation, then RAN4 may needs to further discuss the potential transition requirements for SMTC adaptation.

Proposal 1: For SSB adaptation in L3 measurement, RAN4 assumes that UE is still expected to perform measurement based on SMTC. RAN4 to wait for RAN2 LS reply to decide whether there is RAN4 requirements impacts (e.g. whether to define requirements at transition).

## SSB adaptation for L1 measurement

To simplify the description, L1 measurement refers to L1-RSRP/L1-SINR/BFD/CBD as agreed in last meeting.

For L1 measurement, following two scenarios are discussed based on the contribution in last meeting:

- L1 measurements at transition for SSB adaptation

- L1 measurement after transition

For L1 measurement at transition, there is already similar requirements defined in the spec. Taking the requirements at transition for BFD as example, which is shown as follows:

### 8.5.10 Minimum requirement at transitions for beam failure detection When the UE transitions between DRX and no DRX or when DRX cycle periodicity changes, for each BFD-RS resource, for a duration of time equal to the evaluation period corresponding to the second mode after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation period corresponding to the first mode and the second mode. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second mode for each BFD-RS resource. When the UE transitions from a first configuration of BFD resources to a second configuration of BFD resources that is different from the first configuration, for each BFD resource present in the second configuration, for a duration of time equal to the evaluation period corresponding to the second configuration after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second configuration for each BFD resource present in the second configuration. When the UE transitions from a first configuration of active TCI state of the CORESET to a second configuration of active TCI state of the CORESET, for each CSI-RS for BFD present in the second configuration, the UE shall use an evaluation period corresponding to the second configuration from the time of transition.

However, existing requirements only define the minimum evaluation period but without maximum evaluation period. It is reasonable to also define the upper bound of the evaluation/measurement period to have a guaranteed performance when SSB adaptation happens. For the SSB adaptation, if only considering periodicity adaptation, we believe similar principles can apply.

Proposal 2: For L1 measurement (L1-RSRP/L1-SINR/BFD/CBD), RAN4 to define requirements at transmission at follows:

- UE shall use an evaluation/measurement period that is no less than the minimum of evaluation/measurement period corresponding to the SSB before and after the adaptation, and no more than the maximum of evaluation/measurement period corresponding to the SSB before and after the adaptation. Subsequent to this duration, the UE shall use the evaluation/measurement period corresponding to the SSB after the adaptation.

Base on proposal 2, after the SSB adaptation, it is straight forward that after the transition period, L1 requirements apply based on the SSB after adaptation.

Proposal 3: Based on proposal 2, after the transition period, L1 requirements apply based on the SSB after the adaptation.

## SSB adaptation for SCell activation

For SCell activation, one typical use case is that the SSB periodicity is changed (e.g. from 40 ms to 20 ms) before SCell activation or at the same time as SCell activation, and it is straightforward that UE just uses the new SSB periodicity for SCell activation. Because for this case, it is not clear how to configure the SMTC in servingCellConfig.

However, how to trigger the SSB adaptation is still under discussion in RAN1. It is not clear whether is triggered by MAC-CE as OD-SSB, or RRC signalling. Besides, similar as the discussion in OD-SSB, there is no clear motivation for SSB adaption during SCell activation.

Proposal 4: UE shall use SSB after adaption for SCell activation regardless of the SMTC configuration when SSB adaption happens before or at the same time as SCell activation. Deprioritize the requirements when SSB adaptation happens during SCell activation.

- Detailed triggering on SSB adaptation depends on further RAN1 discussion.

## Others

Based on the discussion in last meeting, companies also proposed to evaluate the impacts on TCI state switching. From our understanding, SSB is only used for timing tracking and L1-RSRP for an unknown TCI state. In realistic scenario, it is less likely that gNB trigger the TCI state switching simultaneously with SSB adaption. Thus, we think there is no RRM impacts on TCI state switching for SSB adaption.

Observation 1: In realistic scenario, it is less likely that gNB trigger the TCI state switching simultaneously with SSB adaption.

# Conclusions

Proposal 1: For SSB adaptation in L3 measurement, RAN4 assumes that UE is still expected to perform measurement based on SMTC. RAN4 to wait for RAN2 LS reply to decide whether there is RAN4 requirements impacts (e.g. whether to define requirements at transition).

Proposal 2: For L1 measurement (L1-RSRP/L1-SINR/BFD/CBD), RAN4 to define requirements at transmission at follows:

- UE shall use an evaluation/measurement period that is no less than the minimum of evaluation/measurement period corresponding to the SSB before and after the adaptation, and no more than the maximum of evaluation/measurement period corresponding to the SSB before and after the adaptation. Subsequent to this duration, the UE shall use the evaluation/measurement period corresponding to the SSB after the adaptation.

Proposal 3: Based on proposal 2, after the transition period, L1 requirements apply based on the SSB after the adaptation.

Proposal 4: UE shall use SSB after adaption for SCell activation regardless of the SMTC configuration when SSB adaption happens before or at the same time as SCell activation. Deprioritize the requirements when SSB adaptation happens during SCell activation.

- Detailed triggering on SSB adaptation depends on further RAN1 discussion.

Observation 1: In realistic scenario, it is less likely that gNB trigger the TCI state switching simultaneously with SSB adaption.

# References

[1] R4-2416860 WF on RRM requirements for Netw_Energy_NR_enh_Part2

[2] R4-2416911 LS on SSB adaptation

[3] R4-2415663 Topic summary for [112bis] [216] Netw_Energy_NR_enh_Part2