3GPP TSG-RAN WG4 Meeting # 113	R4-2418657

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.25.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM requirements for Rel-19 XR

Document for:	Discussion

# Introduction

The new WI about XR was approved in [1] and further revised in [2]. In last RAN4 meeting, RAN4 discussed the RRM impacts of the WI with agreements and issues captured in the WF [3]. In this contribution, we further provide our views on remaining issues.

# Discussion

## 2.1 Scenario

Regarding the applicable scenarios, following agreements are achieved in last meeting

## Issue 1-1: Deployment scenarios < Agreement>: - Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation <Way forward>: - Further discussion needed for - FR2-2 - NR-DC, EN-DC, NE-DC

Apart from the agreement scenarios (FR1/FR2-1 in NR SA and CA), for FR2-2, usually it is separately considered in previous release as it has unique SCS and beam sweeping factors. Thus, we are OK to deprioritize FR2-2 in this WI.

## Issue 1-2: Types of measurements to be considered <Way forward>: - Option 1: Agree at least on the following and further discuss other scenario - Intra-frequency - Inter-Frequency - LTE inter-RAT - Option 2: Agree at least on the following and further discuss other scenario - Intra-frequency - Inter-Frequency - LTE inter-RAT - XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC Option 3: include all related scenarios

For type of measurement to be considered, the majorities view is that at least NR intra-/inter-frequency and LTE inter-RAT measurement should be considered. For 2G/3G measurement, companies show concerns that it may impact previous implementation and the time offset to cancel the measurement may take much longer time. From our understanding, it is not a critical scenario to enhance XR service when UE is also configured to perform 2G/3G measurement.

Proposal 1: Deprioritize FR2-2 scenario for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

Proposal 2: RAN4 to considering following type of measurement:

  - Intra-frequency

  - Inter-Frequency

  - LTE inter-RAT

  - And the requirements do not apply when UE is required to perform 2G and/or 3G measurement.

For L1 measurement and also LTM measurement, from our understanding, the discussion about the feature is only about L3 measurement (in MG or with measurement restriction). It could be also found in the TR 38.835 which his shown as follows that only intra-frequency and inter-frequency are studied. Another technical reason is that L1 measurement is highly correlated to data scheduling. We can try to find additional scheduling opportunity in L3 measurement occasions since there could be the case that the mobility demand is not that urgent when there is data XR data. But for L1 measurement, which is to guarantee the link between the UE and the serving cell, we see less room for cancel L1 measurement for additional transmission opportunity. Based on analysis above, we suggest to only consider the Tx/Rx in occasions of L3 measurement.

Proposal 3: RAN4 to only consider Tx/Rx in occasions of L3 measurement.

For L3 measurement, RAN4 also discussed whether to consider both Measurement with gap and without gap. As commented by companies in last meetings, there benefits of skipping measurement without gap is not significant compared with measurement with gap. The scheduling restriction caused by measurement without gap comes from following three aspects:

- Different numerologies in FR1

- Beam sweeping in FR2

- Not support simultaneous Tx and Rx for certain BC

For the first aspect, as commented by many companies, it is not a typical case in real deployment, and most UE can support such capabilities. For the second aspect, even UE cancels the measurement, gNB also needs to transmit different SSB index, thus means gNB may not be able to schedule the UE during the whole SMTC. For the third aspects, it can bring more UL transmission opportunities. However, the UL transmission is not the bottleneck as there is no non-integer frame restriction for UL transmission. Thus, based on the analysis above, we are fine to deprioritize the L3 measurement without gap.

Proposal 4: Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction.

Another issue which is not discussed in last meeting is about the type of MGs, and RAN4 indeed spend quite a lot time in the first meeting. From our understanding, we see no significant difference of the solutions/RRM impacts for different MG types. Companies spend a lot time discussing which kind(s) MGs to be considered in the feature, but without very good justifications. This deliberate down selection on type of MGs does not help to figure out how this solution could work. Thus, we suggest RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether to the feature can apply to this MG type.

Proposal 5: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether to the feature can apply to this MG type.

## 2.2 Requirements impacts

Regarding how to capture impact of measurement skipping, following options are considered:

## Issue 3-1: How to capture impact of measurement skipping <Way forward>: - Option 1: - As minimum requirements, cell detection, index reading, and measurement period is extended. - FFS how to extend - Reuse existing measurement accuracy requirements. - Option 2: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - Reuse existing measurement accuracy requirements. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information.

As analyzed in last meeting, Option 2 is more reasonable and beneficial from both UE side and NW side. As commented by many companies, even we consider the minimum requirements as defined in the spec without reducing the number of samples for measurement, there is already unused Gap occasion as different frequency layers may have different SMTC configurations. If gNB take these unused gap occasion for XR service, there is no impact on mobility measurement performance, and has minor impacts on current UE implementations.

Observation 1: Even considering the minimum requirements in the spec without reducing the number of samples, there is already unused Gap occasion.

Observation 2: If gNB take these unused gap occasion for XR service, there is no impact on mobility measurement performance, and has minor impacts on current UE implementations

Thus, one feasible approach is to indicate the Gap occasions via UAI as asked in RAN1 LS [5]. For the case where there is no measurement impacts, the conditions are as follows:

- UE reports the UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply.

- The MGs indicated to be canceled by NW is within the MGs occasion indicated via UAI.

From gNB perspective, if gNB deems that the XR service is more urgent and can tolerance the performance degradation of mobility measurement in some degree, that gNB can cancel more MGs occasion or cancel different MGs occasion as indicated by UE, thus the measurement delay will be extended.

Based on the discussion in last meeting, companies shown concerns on indicated the detailed information of unused MG occasion to NW. From our understanding, it is up to UE to indicate the exact number/passion of MG occasions as long as UE can guarantee that UE can still meet the existing requirements. For instance, if there is 4 MG occasions are not used in every 10 MG occasions, UE can indicate via UAI about two MG occasions which can be skipped without impact mobility performance, which already leave some flexibility from UE side. Another compromised solution is to allow different UAI indication, UE can indicate following two types of UAI. UAI 1 is the same as mentioned above. UE can also only indicate UAI 2 about numbers and/or positions of MG can be skipped, when the delay will be extended. Thus, it is same as option 1 in the WF.

UAI 1: 	UE reports the UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply.

UAI 2:  UE reports the UAI about the numbers and/or positions of MG can be skipped, when measurement requirements delay will be extended.

Proposal 6: To enable different UE implementations, RAN4 to introduce UAI with different measurement requirements, and it is up to UE to report one of / or both UAI:

- UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply.

- UAI about the numbers and/or positions of MG can be skipped, when measurement delay will be extended accordingly.

Proposal 7:

To enable different UE implementations and give flexibility from NW perspective to skip MG with different urgency:

For the case where there is no measurement impact, the conditions are as follows:

- UE reports the UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply, and

- The MGs indicated to be canceled by NW is within the MGs occasion indicated via UAI.

For case when the measurement delay is extended, the conditions are as follows:

- UE reports the UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply, and

- The MGs indicated to be canceled by NW is more than/ or different from the MGs occasion indicated via UAI.

Or

- UAI about the numbers and/or positions of MG can be skipped, when measurement delay will be extended accordingly, and

- The MGs indicated to be canceled by NW is within the MGs occasion indicated via UAI.

## 2.3 LS on indication

RAN4 receive another LS [6] from RAN1 about indication to enable Tx/Rx in particular gap/restriction that are caused by RRM. It could be observed that there were two alternatives under discussion in RAN1 in previous meeting. One is the dynamic indication via DCI, and the other one is the Semi-static solution via RRC. And RAN1 also ask the question about the minimum time offset between the indication and the skipped measurement occasion if the dynamic approach is supported.

Agreement For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select one among the following options: Option 1: Support Alt. 1-1: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s); - Indication is included as part of scheduling DCI: - FFS: Bit-field size is one bit; - FFS: Bit-field size is >1 bit; - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. - FFS: DCI format, DCI content, DCI bit-field size; - FFS: Whether indication is for one or more occasions; - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped. Option 2: Support Alt. 3-1: - Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements. - Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions; - FFS: Details of pattern: - FFS: Pattern is based on periodicity, offset and duration; - FFS: Pattern is based on a bitmap; - FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). R1-2407289 Moderator summary #2 - Enabling TX/RX for XR during RRM measurements Moderator (Nokia) Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561.

RAN4 has controversial discussion in last meeting about whether to also support semi-static schemes (e.g. via RRC). We summarize the pros and cons for dynamic indication and semi-static schemes

Dynamic indication

Pros:

- Obviously, DCI-based dynamic indication can enable more prompt MG canceling for emergency data transmission. However, it depends on the value of time offset. If the time needed for MG canceling is very long which can approximate RRC processing time, this benefit is not that significant.

Cons:

- UE may need to re-arrange the measurement pattern occasion by occasion. It is complicated to UE implementation and hard to define delay requirements.

- If the occasion to be skipped is very flexible and dynamic, UE cannot guarantee that there is no impact on delay requirements even only one MG occasion is skipped, as this particular MG occasion could be the one that UE has to use as scheduled before.

- Per RAN1 agreement, the scheduling DCI is used to skip following ONE MG occasion. It means there will be lot PHY overhead, even the MG occasion to be canceled can be pre-defined.

Semi-static schemes

Pros:

- As mentioned in the cons for dynamic approach, it is less complicated to UE implementation that UE can have a stable long-term measurement pattern.

- Semi-static schemes can support the case that MG occasion skipping without impacting RRM performance. If NW can indicate the positions of MG occasions to be skipped, UE can arrange the measurement pattern to guarantee that there is no impacts on measurement delay.

- A single RRC single has less overhead that DCI which is per-occasion indication.

Cons:

- As mentioned above, it is straightforward that semi-static scheme is that flexible as dynamic indication. However, the motivation of the enhancement is to fix case that XR phase will inevitably collide with MG occasion due to non-integer frame. Thus, a pre-define MG occasion skipping can suit the XR service.

Then, the most controversial part is the procedure-wise as commented by companies in last meeting. Some companies argued that RAN1 only ask about Alt-1 in the LS, thus RAN4 shall not say anything about approach. From our understanding, we are not sure what is the expected RAN1 action is RAN4 only shown the concerns for Alt-1 as mentioned above. Will RAN1 check other alternatives one by one in another LS to RAN4? The reason why RAN1 send the LS to RAN4 is because the discussion about MG skipping and impact on RRM requirements is more within RAN4 expertise, and RAN1 is more responsible for PHY design about the indication, which could be observed from current RAN1 discussion. What’s more, the RRC-based semi-static approach is more relevant to RAN2, and RAN2 is also having similar discussion with following agreements in last meeting. It could be observed that RAN2 is also considering the RRC-based approach and RAN2 is waiting for RAN4 evaluation.

RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

Observation 3: RAN2 is also consider RRC-based approach and RAN2 is waiting for RAN4 evaluation.

Based on the analysis above, it is technical reasonable to suggest RAN2 to also have RRC-based semi-static solution due the technical reason mentioned above.

Proposal 8:

RAN4 suggest RAN2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps, and UE can have a stable measurement pattern/period.

Whether UE supports both approaches can be up to UE capability discussion.

Regarding the time offset value, it is really a UE implementation issue. Before a certain measurement occasion, there could be complicated processing procedures, e.g. RF tuning for measure a particular frequency. Thus, the value can not be too short. One example RAN4 can refer to is the 5ms for pre-MG. The status of this issue is summarized as follows；

## Issue 4-1: Time offset requirement for measurement gap skipping <Way forward>: - Option 1: 7 ms - Option 2: 5 ms - Option 3: DCI decoding time defined in TS 38.214 + [5]ms - Option 4: assume subclause 5.3.1 of TS38.214 as a baseline for X offset. - Option 5: Fraction of a slot - Option 6: besides the DCI decoding delay, no additional processing delay is needed. - Option 7: N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214)

As analyzed in above issues, it depends on the type of measurement to be measured within the MG. If both NR and LTE inter-RAT measurement is performed within the MG, the time offset should be at least 5 ms. If only NR frequency layer is to be measurement within the MG, shorter time offset could be considered. E.g. 4 ms.

Proposal 9: For the time offset for MG skipping, it should be at least 5 ms if both NR frequency layers and LTE inter-RAT is to be measurement within the MG. If only NR frequency layer is to be measurement within the MG, shorter time offset could be considered. E.g. 4 ms.

# Conclusions

Proposal 1: Deprioritize FR2-2 scenario for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

Proposal 2: RAN4 to considering following type of measurement:

  - Intra-frequency

  - Inter-Frequency

  - LTE inter-RAT

  - And the requirements do not apply when UE is required to perform 2G and/or 3G measurement.

Proposal 3: RAN4 to only consider Tx/Rx in occasions of L3 measurement.

Proposal 4: Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction.

Proposal 5: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether to the feature can apply to this MG type.

Observation 1: Even considering the minimum requirements in the spec without reducing the number of samples, there is already unused Gap occasion.

Observation 2: If gNB take these unused gap occasion for XR service, there is no impact on mobility measurement performance, and has minor impacts on current UE implementations

Proposal 6: To enable different UE implementations, RAN4 to introduce UAI with different measurement requirements, and it is up to UE to report one of / or both UAI:

- UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply.

- UAI about the numbers and/or positions of MG can be skipped, when measurement delay will be extended accordingly.

Proposal 7:

To enable different UE implementations and give flexibility from NW perspective to skip MG with different urgency:

For the case where there is no measurement impact, the conditions are as follows:

- UE reports the UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply, and

- The MGs indicated to be canceled by NW is within the MGs occasion indicated via UAI.

For case when the measurement delay is extended, the conditions are as follows:

- UE reports the UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply, and

- The MGs indicated to be canceled by NW is more than/ or different from the MGs occasion indicated via UAI.

Or

- UAI about the numbers and/or positions of MG can be skipped, when measurement delay will be extended accordingly, and

- The MGs indicated to be canceled by NW is within the MGs occasion indicated via UAI.

Observation 3: RAN2 is also consider RRC-based approach and RAN2 is waiting for RAN4 evaluation.

Proposal 8:

RAN4 suggest RAN2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps, and UE can have a stable measurement pattern/period.

Whether UE supports both approaches can be up to UE capability discussion.

Proposal 9: For the time offset for MG skipping, it should be at least 5 ms if both NR frequency layers and LTE inter-RAT is to be measurement within the MG. If only NR frequency layer is to be measurement within the MG, shorter time offset could be considered. E.g. 4 ms.

# References

[1] RP‑234080 New WID: XR (eXtended Reality) for NR Phase 3

[2] RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3

[3] R4-2416864 WF on RRM requirements for XR_Ph3

[4] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[5] R1-2405736 LS on UE assistance information

[6] R1-2407561 LS on gaps/restrictions that are caused by RRM measurements