3GPP TSG-RAN WG4 Meeting # 113	R4-2418661

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on maintenance for L1/L2-based inter-cell mobility

Source: 	Huawei, HiSilicon

Agenda item:	5.23.1

Document for:	Discussion

Introduction

L1/L2 based inter-cell mobility was discussed in last meeting [1]. This contribution provides further analysis on maintenance requirements on L1/L2 based inter-cell mobility from RRM requirement perspective.

Discussion

Early TCI state activation

The issue on early TCI state activation delay is still open [1].

Issue 1-2-1: Whether to consider early TCI state activation for multiple cells at the same time < Agreement>: - Interested companies can have further offline discussion. If no further progress, the following will be agreed in the Nov meeting. - No requirements of early TCI state activation delay are specified for the case that multiple LTM TCI activation commands are received at the same time.

If we consider the scenario that multiple LTM TCI state activation commands are received at the same time, some requirements shall be further updated. Herein some examples are provided,

- In FR1, if the candidate neighbor cells TCI states associated are on different frequency layers and the SSB from these candidate cells are overlapped within MGL, UE is only able to acquire the SSB from one neighbor cell at one MGL duration. In this case, the number of frequency layers would be scaled when defining TCI state activation delay.

- Still in FR1, if RTD between TCI states associated candidate cells is larger than CP, UE with [capability of measurement with RTD>CP] needs to measure SSB on multiple intra-frequency layers in sequence. In this case, the number of intra-frequency layers is to be scaled as well when defining TCI state activation delay.

- In FR2, if the TCI state activation commands from multiple candidate cells are indicated, UE is not able to acquire SSB from different cells at the same time, as the cells are from different directions. In this case, the Rx beam number is supposed to be scaled when defining TCI state activation delay.

In our understanding, the number of LTM candidate cell is limited, it is not a typical configuration that network simultaneously indicates multiple LTM TCI activation MAC CE for multiple frequency layers or multiple cells. Moreover based on the above analysis, such configuration would severely prolong TCI state activation delay. Therefore we suggest not to specify requirements for multiple LTM TCI activation MAC CEs reception scenarios.

Proposal 1: No requirements of early TCI state activation delay are specified for the case that multiple LTM TCI activation commands are received at the same time.

Issue 1-2-2: The definition of Tfirst-SSB in early candidate cell’s TCI state activation delay for inter-frequency with gap for known TCI state case RAN4#111 < Agreement>: - In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG. - FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG. - Same agreement applies to the definition of Tfirst-SSB in PDCCH order RACH delay requirement.. < Way Forward > FFS the following options: - Option 1 (Apple, MTK, Ericsson): - remove “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG” in definition of TSSB in PDCCH ordered RACH and early TCI activation requirements. - Option 2 (ZTE, vivo): - Confirm the FFS.

Tfirst_SSB in TCI state activation for LTM candidate cell is for fine time tracking. The key point of the issue is that the time/frequency sync procedure within gap occasion is related with L1 inter-frequency capability or L3 inter-frequency measurement capability. In common understanding, coarse timing is acquired in L3 measurement procedure. However fine time tracking can be performed along L1 measurement. In this sense, the applicability of Tfirst_SSB within gap depends on L1 measurement capability.

Proposal 2: In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG. This is applicable to UE supporting inter-frequency L1 measurement with MG.

LTM Cell switch delay requirements

TRS as QCL source in cell switch delay

Issue 1-4-1-1: T/F tracking when TRS as QCL source in cell switch delay Ad hoc agreement <Agreement>: - Confirm current cell switch delay requirements are applicable to the case that TRS is configured as a QCL source in the indicated TCI state in cell switch command. - FFS whether Tfirst-ssb in cell switch delay requirements need to be updated, e.g. based on TRS.

The key point of the issue is that if TRS is configured as a resource RS in TCI state which is QCL type-A, whether new requirements are to be introduced. In current LTM cell switch delay requirement, only SSB is considered for T/F fine tracking. If SSB is configured as a source RS in TCI state which is QCL type-C, according to RAN1 (duplicated in below) UE does not expect to be indicated type-A properties, and assumes PDCCH/PDSCH receptions based on SSB is QCL-type A. Therefore the current RAN4 requirements where SSB is used for T/F tracking are well aligned with RAN1.

TS38.213 clause 21

On the other hand, for RACH based LTM cell switch, as during legacy L3 handover delay, SSB is used for T/F tracking, it is sufficient to use SSB to perform T/F tracking for LTM cell switch as well. For RACH-less based LTM cell switch, either SSB or TRS can work to obtain T/F synchronization. As UE is switched to a new cell, the conservative implementation is using low MCS in the initial stage. SSB based fine time sync can guarantee demodulation performance at low MCS.

Proposal 3: SSB based T/F tracking is sufficient for LTM cell switch delay. Suggest not to define requirements of TRS based enhancement.

Issue 1-4-1-2: Conditions of Tfirst-RS =0 in cell switch delay Ad hoc agreement <Agreement>: - For cell switch with L1 measurement: Extend the condition for Tfirst-RS = 0 “the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than [160 ms]” from the agreed 160 ms, to TCI state activation delay stated in section 8.25.3 + 160 ms. - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - the time gap between completion of TCI activation and cell switch command is larger than 480ms - Note: This is not to extent 160ms to 480ms directly. Longer L3 measurement delay may be expected in the 480ms duration. - Discuss the wording in CR. - FFS the following assumption - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - the time gap between early RACH transmission and cell switch command is larger than [160ms] and the L3 measurement interval is larger than 160 ms.

In the case that neighbor cell L1 measurement is not supported/configured, there is no opportunity for UE to acquire SSB on the candidate cell after UE performed early PRACH on that cell. Then it is challenge for UE to keep the T/F sync on the candidate cell if the time duration is larger than 160ms. In other word, one SSB occasion is needed during cell switch if the time gap between early RACH transmission and cell switch command is larger than 160ms.

Proposal 4: In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if the time gap between early RACH transmission and cell switch command is larger than 160ms and the L3 measurement interval is larger than 160 ms.

Early ASN.1 decoding and validity/compliance check

In LTM cell switch delay, TLTM_RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command. RAN4 is discussing the conditions / cases for TLTM_RRC-processing =0.

Issue 1-4-3-1: Which cell(s) TLTM-RRC-processing = 0 apply to when candidate cells configured are more than UE capability? Online agreement <Agreement>: - If the total number of cells to be fast RRC decoded does not exceed UE capability maxNumberStoredConfigCells-r18 and the number of LTMCandidateConfigs fast decoded does not exceed maxNumberConfigs-r18, TLTM_RRC-processing =0 applies to the LTM candidates with early TCI activation or early PDCCH order RACH, - Further discuss: - FFS: Otherwise, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) [and/or] PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18. - FFS: This is at least applicable to the case that NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion. - FFS: Whether to support and further optimize the case that NW triggers TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

In case the candidate cells configured are more than UE capability, UE shall select some candidates which are within UE capability for early ASN.1 decoding and validity check. The most promising way is to select the LTM candidates with the most recently activated TCI states and PDCCH-order PRACH transmission. In previous meeting, companies think the term of “most recent” is not clear enough. As triggering early TCI state activation and early PDCCH order RACH at the same time is not typical, we suggest to exclude the case.

Proposal 5: In case the configured candidate cells are more than UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

- The requirements are applicable to the case NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

Issue 1-4-5-1: Whether are cell switch delay requirements applicable to FR2 without L1 measurement report? < Way Forward > FFS the following options: - Option 1 (OPPO): - Yes when SNR of the TCI state≥ -3dB and TCI state has been activated, it is also required that the RS associated to the target TCI state is available at least every 1280ms after TCI state activation command. - Option 2 (Nokia): Yes with the following conditions: - The target TCI state in the cell switch command is activated not more than 1280 ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or - The target TCI state in the cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every 1280 ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB - Option 3 (Ericsson, QC): Yes with the following conditions: - [- The TCI state is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state is ≥ -3dB; where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same; or] - [- The TCI state is activated before the reception of the cell switch command (where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same) and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or] - Option 4 (MTK): - Not to extend cell switch delay requirements to FR2 without L1 measurement report

One motivation of R18 LTM is to acquire beam information of candidate cells in advance. Herein the beam refers to the network transmit beam, and the best Rx beam UE determined through RX beam sweeping. Upon receiving cell switch command, UE can quickly use the corresponding beam for data reception. To quickly utilize the beam for data reception, the beam shall be fine beam for demodulation. In FR2, the beam acquired by L3 measurement is rough beam and the beam acquired by L1 measurement is fine beam. If only L3 measurement (without L1 measurement report) is performed in FR2, UE shall perform beam fining after cell switch command. The gain of LTM feature in FR2 would almost vanished.

Moreover in RAN4#111 meeting, RAN4 discussed the issue and had agreement that for FR2, RAN4 Rel-18 related LTM requirements are not applicable if L1 measurements for LTM candidate cells are not supported by the UE, or not configured by the network.

Reply to the Question 1: For whether intra-frequency and inter-frequency L1 measurements and reporting are prerequisites to support intra-frequency and inter-frequency LTM, RAN4 analysed RAN4 requirements and achieved following agreements - For FR1, RAN4 Rel-18 related LTM requirements are applicable to the case without L1 measurement and reporting of LTM candidate cells. - For FR2, RAN4 Rel-18 related LTM requirements are not applicable if L1 measurements for LTM candidate cells are not supported by the UE, or not configured by the network.

Proposal 6: Not to extend cell switch delay requirements to FR2 without L1 measurement report.

Conclusions

This contribution provides further analysis on maintenance requirements on L1/L2 based inter-cell mobility. The following proposals are provided:

Proposal 1: No requirements of early TCI state activation delay are specified for the case that multiple LTM TCI activation commands are received at the same time.

Proposal 2: In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG. This is applicable to UE supporting inter-frequency L1 measurement with MG.

Proposal 3: SSB based T/F tracking is sufficient for LTM cell switch delay. Suggest not to define requirements of TRS based enhancement.

Proposal 4: In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if the time gap between early RACH transmission and cell switch command is larger than 160ms and the L3 measurement interval is larger than 160 ms.

Proposal 5: In case the configured candidate cells are more than UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

- The requirements are applicable to the case NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

Proposal 6: Not to extend cell switch delay requirements to FR2 without L1 measurement report.

Reference

- R4-2413960, WF on Further NR mobility enhancements, MediaTek

- R4-2410303, Reply LS on LTM L1 intra and inter-frequency measurements