3GPP TSG-RAN WG4 Meeting # 113							                                     R4-2418665

Orlando, US, 18th – 22nd November, 2024

Title: 					Discussion on incremental BWP switch delay for multi-carrier enhancements

Source: 	Huawei, HiSilicon

Agenda item:	5.18.2

Document for:	Discussion

Introduction

Due to the introduction of new UE capability for switching between non-dormant and dormant BWPs triggered by the new DCI format 0-3 and 1-3, one issue on simultaneous DCI based BWP switch delay on multiple CCs was identified in RAN4#112bis meeting [1]. The contribution provides analysis on the issue.

Issue 1-1: Report value for FG6-3 and FG38-9 <Way forward> - Option 1: The same value D is assumed in the requirements between RAN4 features 6-3 and 38-9. - Option 2: The different value D is assumed in the requirements between RAN4 features 6-3 and 38-9. Issue 2-1: Requirements for DCI based BWP switch delay on multiple CCs <Way forward> - Option 1: Keep current requirements. - Option 2: Update is needed, e.g., different increment value for different UE capabilities is used in the requirements, and will be discussed under Rel-18 maintenance. - Option 3: Further discuss whether update on D is needed for simultaneous switching between non-dormant BWPs and switching between non-dormant and dormant BWPs by using DCI 0-1/1-1/2-6.

Discussion

The new DCI formats 0-3 and 1-3 were introduced for R18 multi-carrier enhancements and a new UE capability 49-9 on SCell dormancy indication within active time in DCI format 0-3/1-3 in NR multi-carrier enhancement were also introduced in RAN1.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-9 | SCell dormancy indication within active time in DCI format 0_3/1_3 | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_3/1_3 | 6-5, at least one of {49-1, 49-1b, 49-2,49-2b} |

In existing BWP switching delay on multiple CCs, the incremental delay for each additional CC depends on bwp-SwitchingMultiDormancyCCs-r16 for switching between non-dormant and dormant BWPs where

bwp-SwitchingMultiDormancyCCs-r16 (that’s FG 6-3 as below) is defined as below, and the prerequisite of FG 6-3 is FG 18-4 or 18-4a, which are for format 0-1/1-1 or format 2-6, respectively.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 6. LTE_NR_DC_CA_enh | 6-3 | Dormant BWP switching on multiple CCs RRM requirements | Incremental delay for BWP switch processing on additional SCells in DCI based simultaneous dormant BWP switching on multiple SCells | RAN1 feature 18-4 or 18-4a |
| 18. MR-DC/CA enhancement | 18-4 | SCell dormancy indication within active time | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_1/1_1 | 6-5 |
| 18. MR-DC/CA enhancement | 18-4a | SCell dormancy indication outside active time | Support for SCell dormancy indication sent outside the active time on PCell with DCI format 2_6 | 19-1 |

RAN4 sent an LS to RAN2 to introduce a new R18 UE capability for dormant BWP switching on multiple CCs RRM requirements with DCI 0-3/1-3 [2].

RAN2 replied the LS and introduced a field bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 corresponding to RAN4 feature 38-9, and a field scellDormancyWithinActiveTime-DCI-0-3-And-1-3-r18 corresponding to its prerequisite feature (RAN1 feature 49-9) [3]. And RAN2 think it is up to RAN4 whether the UE shall report the same value between RAN4 features 6-3 and 38-9.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-9 | Dormant BWP switching on multiple CCs RRM requirements with DCI 0-3/1-3 | Incremental delay for BWP switch processing on additional SCells in DCI based simultaneous dormant BWP switching on multiple SCells | 49-9 |

The existing requirements for simultaneous DCI based BWP switch delay on multiple CCs are specified as below. According to the current requirements, the same incremental D is assumed for three UE capabilities (bwp-SwitchingMultiCCs-r16, SwitchingMultiDormancyCCs-r16 and bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18) when simultaneous BWP switching between non-dormant BWPs and between dormant and non-dormant BWP are triggered.

8.6.2A.1 Simultaneous DCI based BWP switch delay on multiple CCs UE shall finish BWP switch within the time duration TMultipleBWPswitchDelay + Y, which is defined as: TMultipleBWPswitchDelay = TBWPswitchDelay + D*(N-1) Where: - TBWPswitchDelay is the BWP switching delay on single CC defined in Table 8.6.2-1 depending on UE capability bwp-SwitchingDelay [2]. TBWPswitchDelay shall be based on the smallest SCS among SCS of all involved CCs before and after BWP switch. If the BWP switch on multiple CCs results in the change of the SCS on any CC among involved CCs, TBWPswitchDelay shall be based on the smallest SCS among all SCS values of all involved CCs. - D is the incremental delay for each additional CC involved in simultaneous BWP switch and depends on UE capability bwp-SwitchingMultiCCs-r16 [TS 38.306, 14] for switching between non-dormant BWPs, and bwp-SwitchingMultiDormancyCCs-r16 or bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 for switching between non-dormant and dormant BWPs.

Table 1 listed the UE capability, BWP switching type and corresponding triggered DCI type.

| UE capability | Function | DCI triggering |
| --- | --- | --- |
| bwp-SwitchingMultiDormancyCCs-r16 | BWP switching between non-dormant BWPs | DCI format 0-1, 1-1, DCI format 0-3, 1-3 |
| bwp-SwitchingMultiDormancyCCs-r16 | BWP switching between dormant and non-dormant BWP | DCI 0-1, 1-1 and 2-6 |
| bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 | BWP switching between dormant and non-dormant BWP | DCI format 0-3, 1-3 |

It is observed that, simultaneous switching between non-dormant BWPs and between dormant and non-dormant BWP can happen in the following scenarios:

Scenario 1: Single DCI 0-3/1-3 sent on PCell triggers simultaneous switch between non-dormant BWPs on PCell and BWP switching between dormant SCell and non-dormant SCells, or

Scenario 2: One DCI 0-1/1-1 sent on PCell triggers switching between non-dormant BWPs on PCell, and simultaneously in the same slot another DCI 0-1/1-1 sent on PCell triggers BWP switching between dormant SCell and non-dormant SCells.

In our understanding, compared with switching between non-dormant BWPs, UE may need additional time for warm up operation for switching from one dormancy SCell to non-dormancy SCell. Therefore UE may report different values of incremental D for switching between non-dormant BWPs and switching from one dormancy SCell to non-dormancy SCell.

For the UE with bwp-SwitchingMultiDormancyCCs-r16 capability or bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 capablity, the switching delay between non-dormant and dormant BWPs indicated by different DCI may be the same. However to leave more flexibility for UE implementation, we suggest to consider different incremental D for bwp-SwitchingMultiDormancyCCs-r16 or bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 as well.

Proposal 1: The different value D can be assumed for

bwp-SwitchingMultiCCs-r16 for switching between non-dormant BWPs,

bwp-SwitchingMultiDormancyCCs-r16 for switching between non-dormant and dormant BWPs,

bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 for switching between non-dormant and dormant BWPs.

The equation for simultaneous DCI based BWP switch delay on multiple CCs would be updated as:

TMultipleBWPswitchDelay = TBWPswitchDelay + i=1N-1Di

Where Di is the incremental delay for ith additional CC involved in simultaneous BWP switch and depends on UE capability bwp-SwitchingMultiCCs-r16 [TS 38.306, 14] for switching between non-dormant BWPs, and bwp-SwitchingMultiDormancyCCs-r16 or bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 for switching between non-dormant and dormant BWPs.

Proposal 2: Simultaneous DCI based BWP switch delay on multiple CCs would be updated as:

TMultipleBWPswitchDelay = TBWPswitchDelay + i=1N-1Di

Where Di is the incremental delay for ith additional CC involved in simultaneous BWP switch.

The issue is not only caused by R18 new capability bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18, but also exists for legacy Simultaneous DCI based BWP switch delay on multiple CCs. The case corresponds to scenario 2 as illustrated above. The requirements update can be applied from R16.

Proposal 3: Update legacy requirements for simultaneous DCI based BWP switch delay on multiple CCs from R16.

Conclusions

This contribution provides analysis on simultaneous DCI based BWP switch delay on multiple CCs. According to the analysis, we propose the following proposals:

Proposal 1: The different value D can be assumed for

bwp-SwitchingMultiCCs-r16 for switching between non-dormant BWPs,

bwp-SwitchingMultiDormancyCCs-r16 for switching between non-dormant and dormant BWPs,

bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 for switching between non-dormant and dormant BWPs.

Proposal 2: Simultaneous DCI based BWP switch delay on multiple CCs would be updated as:

TMultipleBWPswitchDelay = TBWPswitchDelay + i=1N-1Di

Where Di is the incremental delay for ith additional CC involved in simultaneous BWP switch.

Proposal 3: Update legacy requirements for simultaneous DCI based BWP switch delay on multiple CCs from R16.

References

- R4-2416878, WF on UE report of incremental BWP switch delay for multi-carrier enhancements, DoCoMo

- R4-2410299, LS on UE capability for multi-carrier enhancement, vivo

- R2-2407463, Reply LS on UE capability for multi-carrier enhancements, DoCoMo