3GPP TSG-RAN WG4 Meeting # 113	R4-2418672

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on conditional Intra-CU LTM

Source: 	Huawei, HiSilicon

Agenda item:	7.24.2.3

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

This contribution provides analysis on intra-CU LTM from RAN4 RRM requirement perspective.

Discussion

Regarding the third objective conditional intra-CU LTM, according to the latest revised WID, the conditional LTM and subsequent conditional LTM are to be supported. The analysis on the RRM requirements for conditional LTM and subsequent conditional LTM are provided in the followings.

Conditional LTM

One open issue regarding L3 measurement based conditional LTM was widely discussed during last RAN4 meeting[2].

Issue 1-1-3: Measurement requirements on conditional LTM < Agreement > - On whether L3 measurement based conditional LTM in FR1 can be supported - RAN4 can further discuss this issue in FR1 and make recommendation to RAN2 if RAN4 consensus can be reached. - As generic procedure for the objective led by RAN2, if any conclusion will be made in RAN2, RAN4 will follow the RAN2 conclusion unless any technical issue is identified.

In the following, the analysis is to be provided from execution conditions, early TCI state activation and PDCCH order RACH perspective in C-LTM procedure in FR1.

In RAN2 it is agreed that Event LTM3-like and LTM5-like are used as the conditional LTM execution condition.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

Event LTM3 and LTM5 are defined as L1 LTM events in RAN2.

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

For FR1 UE, as the RX antenna is omni-directional, the antenna reception gain has no difference for L1 and L3 measurements. However as L3 measurement result shall average multiple physical samples, the L3 measurement delay would be larger than that of L1 measurement. Then the event evaluation based on L3 measurement may be slower than L1 measurement. Anyway if companies agree on L3 measurement based conditions for conditional LTM, RAN2 may have some clarification on L3 beam specific LTM events/conditions.

For FR1 UE, to perform early candidate TCI state activation for early DL sync, UE shall report beam information to network. In conditional LTM procedure for FR1, the SSB index or CSI-RS resource index can be reported by L3 measurement report or L1 measurement report. Although the reported beam index which is selected by L3 measurement may be different from the beam selected by L1 measurement, each scheme has its own advantage, the impact on mobility performance is hard to compare. Anyway from configuration and implementation perspective, it is feasible to perform L3 based early TCI state activation in FR1 in C-LTM.

As we know, the L3 based PDCCH order PRACH in FR1 is supported in R18 LTM. Regarding PDCCH order RACH in R19 C-LTM procedure, RAN2 had plenty discussion. the R-18 PDCCH order RACH procedure may not be directly reused due to some reasons (e.g., how to deliver the TA value from network to UE before the triggering C-LTM, how UE can receive PDCCH order in time, etc.). However these identified issues exist for both L1 and L3 based PDCCH RACH, rather than specific issue only for L3 based.

- 6.2.2C PDCCH ordered Random Access for LTM The requirements in this clause apply if PDCCH order-based RACH on candidate cell is triggered based on L1-RSRP report or L3-RSRP beam-level report in FR1, or based on L1-RSRP report in FR2.

Based on the above analysis, it is observed that in FR1 it is feasible to support L3 based C-LTM. The L3 based condition evaluation period may be slower than L1 based and RAN2 may need some clarification on L3 beam specific LTM events/conditions.

Proposal 1: In FR1 it is feasible to support L3 measurement based C-LTM. The possible impacts are,

- the L3 based condition evaluation period may be slower than L1 based

- RAN2 may need some clarification on L3 beam specific LTM events/conditions.

Conditional LTM delay

Legacy conditional handover delay is defined as DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. Taking legacy CHO delay as a reference, in the following, we try to analyse each component for conditional LTM delay.

In legacy CHO delay, the starting point is from UE receives a RRC message implying conditional handover. After UE successfully decodes the RRC signalling, UE waits for the conditional conditions are met and then performs handover to the target cell. The ending point of CHO delay is when UE starts the transmission of the new uplink PRACH channel.

For C-LTM, LTM cell switch related conditions are also signalled by RRC. So it is straight forward that the starting point of conditional LTM is the instance when the UE receives a RRC message implying conditional LTM.

Proposal 2: The starting point of conditional LTM delay is the instance when the UE receives a RRC message implying conditional LTM.

It is agreed in RAN2 that both RACH based and RACH-less conditional LTM are supported. For RACH based conditional LTM, the ending point is UE transmit the first available PRACH. For RACH-less conditional LTM, the UE should send the RRCReconfigurationComplete as the first UL transmission to the target cell/beam to complete the procedure, like the legacy LTM procedure. Therefore for both cases, the ending point of conditional LTM delay is the instance the UE transmits the first UL message on the target cell.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Proposal 3: For RACH based and RACH-less conditional LTM, the ending point of conditional LTM delay is the time instance the UE transmits the first UL message on the target cell.

In legacy CHO delay, the component TEvent_DU and Tmeasure are specified for the time duration when the condition event is really met at UE side.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists at the measurement reference point which will trigger the NES-based conditional handover

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

In last meeting, RAN2 agreed to support early TCI state activation/deactivation and UL sync on candidate cells during conditional LTM procedure. In our understanding, early DL (e.g., TCI state activation) and UL sync (UE based TA, or PDCCH ordered based RACH) procedures on candidate cells shall be performed before UE evaluating the execution conditions.  However the collaboration of these procedures needs carefully consideration.

Taking PDCCH order RACH as an example, in Rel-18 network triggered LTM, it is the source DU that sends the PDCCH order and the LTM cell switch MAC CE, so it is easier to determine when to send the PDCCH order and when to send the LTM cell switch MAC CE. However, in conditional LTM, the source DU does not know when the UE will switch to another cell. If the source DU sends a PDCCH order very early, the TA may be out of date when the UE triggers cell switch later. Conversely, there is also a case that when the conditions are met in UE side, UE starts to switch to the target cell, however at the same time, network is triggering PDCCH ordered RACH, then UE failed to receive the PDCCH order.

In addition, after UE performed PDCCH ordered early TA, as no dedicated cell switch command is indicated to UE during conditional LTM procedure, how to deliver the TA value on candidate cell to UE also needs to be discussed.

We believe all these issues are under discussion in RAN2. RAN4 can wait for more progress. If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evolution, it is no need to introduce the additional time duration for DL sync and UL sync, as TEvent_DU(the waiting time duration for fulfilling condition) has already covered the two components.

Proposal 4: TEvent_DU can wait for more progress in RAN2 as DL sync and UL sync are supported for conditional LTM.

Proposal 5: If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evaluation, the delay for DL sync and UL sync can be contained in TEvent_DU.

In conditional LTM, Tmeas is the measurement time. The L1 measurement period can be applied.

Proposal 6: In C-LTM delay, Tmeas is the L1 measurement period

- If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM.

- If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19.

- FFS on L3 measurement period depending on the conclusion of whether L3 based measurement is considered in R19 C-LTM

In legacy CHO delay, TCHO_execution is the UE execution preparation time for conditional handover. This time duration originally called TRRC_2 for RRC processing time of the target cell. We think TCHO_execution can be reused in conditional LTM

Proposal 7: TCHO_execution can be reused in conditional LTM.

In legacy CHO delay, Tinterrupt comprises Tprocessing + TIU + T∆ + Tmargin. In legacy CHO procedure, interruptions happen from UE starts to execute CHO to target cell. However in C-LTM, interruptions may also happen during early RACH procedure. Therefore from terminology perspective, we suggest the term Tinterrupt  is not used when defining conditional LTM delay.

Proposal 8: The term Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PDCCH RACH procedure.

In C-LTM, upon condition is met, after UE finish associated operations and leave the source cell (TCHO_execution), the steps which are defined in R18 LTM cell switch delay. i.e., TLTM-processing , Tfirst-RS , TRS-proc , TLTM-IU, shall be followed.

In R18 LTM cell switch delay,

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the uncertain time in transmit the first UL transmission.

Proposal 9: After TCHO_execution, the component TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused.

In summary, the C-LTM delay can be depicted as below equation:

DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

Proposal 10: C-LTM delay can be depicted as

DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

Subsequent conditional LTM

The requirements of subsequent CPA and subsequent CPC have been specified in R18. The delay for subsequent CPA/CPC is alike CHO delay in some extent. The difference is the starting point. In subsequent CPA/CPAC the starting point is from the time UE transmits RRC Reconfigurationcomplete message for the previous PSCell addition or change. For subsequent conditional LTM, in our understanding either RACH based or RACH-less cell switch, the starting point of the subsequent conditional cell switch delay can also reuse the point that UE transmits RRC Reconfigurationcomplete message for the previous PCell.

Proposal 11: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell.

Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM.

Proposal 12: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM.

Conclusion

The contribution provides analysis on intra-CU LTM from RAN4 RRM requirement perspective. The following proposals are proposed:

Proposal 1: In FR1 it is feasible to support L3 measurement based C-LTM. The possible impacts are,

- the L3 based condition evaluation period may be slower than L1 based

- RAN2 may need some clarification on L3 beam specific LTM events/conditions.

Proposal 2: The starting point of conditional LTM delay is the instance when the UE receives a RRC message implying conditional LTM.

Proposal 3: For RACH based and RACH-less conditional LTM, the ending point of conditional LTM delay is the time instance the UE transmits the first UL message on the target cell.

Proposal 4: TEvent_DU can wait for more progress in RAN2 as DL sync and UL sync are supported for conditional LTM.

Proposal 5: If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evaluation, the delay for DL sync and UL sync can be contained in TEvent_DU.

Proposal 6: In C-LTM delay, Tmeas is the L1 measurement period

- If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM.

- If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19.

- FFS on L3 measurement period depending on the conclusion of whether L3 based measurement is considered in R19 C-LTM

Proposal 7: TCHO_execution can be reused in conditional LTM.

Proposal 8: The term Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PDCCH RACH procedure.

Proposal 9: After TCHO_execution, the component TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused.

Proposal 10: C-LTM delay can be depicted as

DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

Proposal 11: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell.

Proposal 12: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2416863, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom