3GPP TSG-RAN WG4 Meeting #113                                      R4-2418717

Orlando, US, November 18th – November 22nd, 2024

Source: 	Huawei, HiSilicon

Title: 	On methods for reducing the number of UE Rx chain

Agenda Item:	7.6.2

Document for:	Discussion

Introduction

The SI of NR FR1 DL Fragmented Carriers was approved at RAN#104. During RAN4#112bis, following agreements related were captured in the way forward.

- Sub-topic 1-2: Study scope clarification

Agreement:

    - Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz as illustrated below:

    - Any update of scope needs be decided by RAN

- Merge issue 2-0/2-1 discussion:

Agreement:

The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter

  - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers

  - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823)

In this contribution, we would like to discuss some remaining open issues.

Discussion

Reference RF architecture

In RAN4#112bis meeting, the reference architecture for existing DL NCCA requirements and for R19 fragmented carrier were agreed. While there was another option raised in previous meetings, i.e ‘supporting only 2Rx in the mandatory 4Rx band’ was also raised as a potential option for RF architecture.

  - Option 2: RAN4 also studies the possibility of supporting only 2Rx in the mandatory 4Rx band below 2700MHz

This option assumes that UE is allowed to support only 2Rx per CC in the mandatory 4Rx band when intra-band CA is configured. With fully shared RF architecture, UE could receive both CCs on 2Rx, and the remaining 2Rx could be used in another band. We think the assumption needs first to be confirmed in RAN4. As in TS 38.101-1 section 7.2, the test cases other than single carrier REFSENS needs to be verified with four Rx antenna ports, which include the intra-band CA test cases.

Observation 1: Clarification might be needed on the assumption that UE is allowed to support only 2Rx per CC in the mandatory 4Rx band when intra-band CA is configured.

If the assumption could be confirmed by RAN4, to our understanding the UE still needs to support 4Rx in the mandatory 4Rx band when there is only one carrier configured on this band. The ability of supporting higher order CA configurations is limited by both the number of antennas and the number of PLLs. For example, a UE supporting inter-band CA between band nX (4Rx mandatory) + B (2Rx band) needs to be equipped with at least 6 Rx antennas and 2 PLLs (Figure 3a). To support NCCA on band nX, both PLL would be used with partially shared or separated architecture, therefore the inter-band CA can’t be supported (Figure 3b).

| a) CA_nXA-nYA | b) CA_nX(2A) |
| --- | --- |

Figure 3	CA ability with partially shared or separated architecture

If fully shared architecture is adopted, the UE could support NCCA on band nX with only one PLL, therefore the higher order inter-band CA configuration could also be supported (Figure 4a). While at the same time it’s not necessary to switch to 2Rx on band nX as suggested in the Option 2 (Figure 4b).

| - CA_nX(2A)-nYA | - CA_nX(2A)-nYA |
| --- | --- |

Figure 4	CA ability with fully shared architecture

Observation 2: The option of ‘supporting only 2Rx in the mandatory 4Rx band’ is not necessary and brings no benefit.

Baseband assumption

In RAN4#112bis, the reference architecture was agreed as fully shared RF chain. No agreement was made regarding baseband. In our previous contribution [2] we analysed the impact of baseband assumption and came to the conclusion that the baseband assumption has very little impact on the RF requirements if the digital filter could be designed properly. We would like to propose to focus on the performance of RF chain, and leave the baseband to UE implementation.

Proposal 1: Baseband assumption could be up to UE implementation assuming the digital filter could be designed properly, i.e. single/separate digital filter, single/separate FFT are up to UE implementation.

Fallback solution

In RAN4#112, there were documents discussing the UE behaviour when fully shared architecture is not applicable due to degraded RF requirements, e.g. large in-gap interference, severe self-interference in FDD bands, etc. We want to provide some initial thoughts on this point.

One of the fallback option could be the legacy CA behaviour, i.e. partially shared or separated RF architecture. However, the UE’s complexity would increase to support dynamically switching between fully shared and partially shared RF architecture. UE needs to be equipped with spare mixers, and the routing would be more complicated.

- Intra-band CA with Fully shared architecture

- Intra-band CA with Partially shared architecture

Figure 9	Dynamic switching between Fully shared and Partially shared architecture

Dynamically switching between fully shared and separated RF architecture would be less complicated, but it leads to reduction of DL MIMO layers on each CC.

- Intra-band CA with Fully shared architecture

- Intra-band CA with Separated architecture

Figure 10	Dynamic switching between Fully shared and Separated architecture

Another fall back option could be switching to non-CA on the relevant band. This option requires the lowest UE implementation complexity, and could reuse the existing signalling scheme.

- CA with Fully shared architecture

- Non-CA

Figure 11	Dynamic switching between CA with Fully shared architecture and Non-CA

Below table summaries the three fallback options. We could tell that the Option 1 of switching to Partially shared architecture will lead to high UE complexity and hardware cost. We prefer not to consider this option.

Comparing with Option 2, the option 3 of switching to Non-CA could provide more DL MIMO layers with low complexity regarding both UE implementation and Network Signalling.

|  | UE Complexity | Signalling Complexity | Hardware cost | DL MIMO layers | DL carriers |
| --- | --- | --- | --- | --- | --- |
| Option 1: CA with Partially shared architecture | High | High | High | 10 | 3 |
| Option 2: CA with Separated architecture | Mid | High | Low | 4 | 2 |
| Option 3: Non-CA | Low | Low | Low | 6 | 2 |

Proposal 4: As fallback solution, don’t consider option 1, i.e. dynamic switching between Fully shared architecture and Partially shared architecture.

Proposal 5: As fallback solution, prioritize option 3, i.e. dynamic switching between CA with Fully shared architecture and Non-CA.

Conclusion

In this contribution we discussed the general aspects of the fragmented carriers. According to the analysis, we have the following observations and proposals:

Observation 1: RAN4 to clarify the assumption that UE is allowed to support only 2Rx per CC in the mandatory 4Rx band when intra-band CA is configured.

Observation 2: The option of ‘supporting only 2Rx in the mandatory 4Rx band’ is not necessary and brings no benefit.

Proposal 1: Baseband assumption could be up to UE implementation assuming the digital filter could be designed properly, i.e. single/separate digital filter, single/separate FFT are up to UE implementation.

Proposal 2: As fallback solution, don’t consider option 1, i.e. dynamic switching between Fully shared architecture and Partially shared architecture.

Proposal 3: As fallback solution, prioritize option 3, i.e. dynamic switching between CA with Fully shared architecture and Non-CA.

- References

- RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc, RAN#104 Meeting, June 2024

- R4-2415804, “On methods for reducing the number of UE Rx chain”, Huawei, HiSilicon, RAN4#112bis meeting, August 2024.

- R4-2417201, “WF on NR FR1 DL fragmented carriers study”, MediaTek Inc, RAN#112bis Meeting, October 2024