3GPP TSG-RAN WG4 Meeting #113				                                      R4-2418725

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.21.2.2

Source: 	CMCC

Title: 	Discussion on A-IoT co-existence evaluation

Document for:	Discussion

# 1. Introduction

In RAN plenary #102 meeting, a new SID of A-IoT is approved with following RAN4 lead RF part objectives, and in last meeting, a WF is approved [1]:

- Coexistence study of Ambient IoT and NR/LTE.

- RF requirements study for Ambient IoT:

  - Ambient IoT BS transmission and reception

  - Ambient IoT Device, as per the General Scope, transmission and reception

  - Intermediate node (UE), as per the General Scope, transmission and reception

In this contribution, we provide simulation results and conclusions for co-existence evaluation.

- Evaluation cases and parameters

In this contribution, we simulated the following baseline cases.

| deployment scenario and topology |  | spectrum | Evaluation case No. | (Aggressor -> Victim) | note |
| --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | D2R |  |
|  |  |  | b | NR DL->reader | D2R |  |
|  |  |  | c | Reader->NR DL | R2D |  |
|  |  |  | d | NR DL->device | R2D |  |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | D2R |  |
|  |  |  | b | NR DL->reader | D2R |  |
|  |  |  | c | Reader->NR DL | R2D |  |
|  |  |  | d | NR DL->device | R2D |  |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | D2R |  |
|  |  |  | f | NR UL->reader | D2R |  |
|  |  |  | c | Reader->NR DL | R2D |  |
|  |  |  | d | NR DL->device | R2D |  |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | D2R |  |
|  |  |  | f | NR UL->reader | D2R |  |
|  |  |  | c | Reader->NR DL | R2D |  |
|  |  |  | d | NR DL->device | R2D |  |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | D2R |  |
|  |  |  | f | NR UL->reader | D2R |  |
|  |  |  | g | Reader->NR UL | R2D |  |
|  |  |  | h | NR UL->device | R2D |  |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | D2R |  |
|  |  |  | f | NR UL->reader | D2R |  |
|  |  |  | g | Reader->NR UL | R2D |  |
|  |  |  | h | NR UL->device | R2D |  |

For evaluation parameters, we use the agreed parameters in WF [1]. For the parameters that have multiple options, we use following values for evaluation.

CW cancellation capability

|  | CW cancellation capability |
| --- | --- |
| D1T1-A2 | 140dB |
| D1T1-B | 150dB |
| D2T2-A2 | 130dB |
| D2T2-B | 140dB |

A-IOT reader and NR BS

|  | In-band |  |
| --- | --- | --- |
|  | Tx | Rx |
| NR UE/A-IOT Intermediate UE | Legacy UE IBE (QPSK) | In-band A-IOT signal interference to NR is not considered when calculating SINR. |
| NR BS | Option 1: 30dBc Option 3: 12dBc | In-band A-IOT signal interference to NR is not considered when calculating SINR. |
| A-IOT BS | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dBc，ACLR2:50dBc) | ACS of legacy gNB |

Tx for device 1 and 2a

- 25dBc with 15KHz bandwidth

Rx for device 1 and 2a

- Assume no frequency selectivity

Active rate of reader for D1T1 and D2T2

- Minimum distance between active readers: 60m

- 2 readers are activated in one drop

Indoor UE percentage for scenario option 1-2

- Option 1: 10%

Issue 4-2-6: Penetration loss for O2I

- Use the equation of 7.4-2 in 38.901

- PLin = 0.5 * d2D-in where d2D-in is the distance to nearest factory/office boundary on the line between Tx and Rx point.

- Set maximum value of d2D-in as 25m

- Evaluation results and conclusion

For D2R, when reader is victim (case b and f), link level SINR level with 10% BLER is provided as below:

|  | D1T1 D2R (dB) | D2T2 D2R (dB) |
| --- | --- | --- |
| min | -7.8 | -7.6 |
| max | 0.23 | -5 |
| average | -4.3 | -6 |

For R2D, when device is victim (case d and h), link level SINR level with 10% BLER is provided as below:

|  | R2D SNR for device 1 (dB) | R2D SNR for device 2a (dB) |
| --- | --- | --- |
| min | -14.8 | -14.1 |
| max | 9.3 | 5.1 |
| average | -6.5 | -7.3 |

| deployment scenario and topology |  | spectrum | Evaluation case No. | (Aggressor -> Victim) | Tentative Co-existence conclusion | Proposals |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | b | NR DL->reader | TBA | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | b | NR DL->reader | TBA | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
|  |  |  | c | Reader->NR DL | FFS: NR DL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range FFS on the UE blocking issue. Interference coordination/mitigation solution can be considered. | about 10% throughput degradation for average throughput is observed, about 70% throughput degradation for cell edge throughput is observed |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
|  |  |  | f | NR UL->reader | TBA | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
|  |  |  | f | NR UL->reader | TBA | SINR degradation is 17dB for cell edge and 4dB for cell average. Outage percentage is: - 17% @ 0.23dB SNR, - 13% @-4.3dB - 10% @ -7.8dB SNR |
|  |  |  | c | Reader->NR DL | FFS: NR DL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range level FFS on the UE blocking issue. Interference coordination/mitigation solution can be considered. | about 30% throughput degradation for average throughput is observed, about 70% throughput degradation for cell edge throughput is observed |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | f | NR UL->reader | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
|  |  |  | g | Reader->NR UL | TBA | about 6% throughput degradation is observed for average throughput, about 13% NR BS throughput degradation is observed for cell edge throughput. |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. | SINR degradation is less than 1dB. Inter-system interference is negligible. For device 1, 4% outage percentage @ 9.3dB SNR (max), no outage percentage for average and min SNR. No outage percentage for device 2a |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | f | NR UL->reader | TBA | SINR degradation is 20dB for cell edge and 4.8dB for average Outage percentage is: - 12% @ -5dB SNR, - 11% @-6dB - 10% @ -7.6dB SNR |
|  |  |  | g | Reader->NR UL | FFS: NR UL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range Interference coordination/mitigation solution can be considered. | about 6% throughput degradation is observed for average throughput, about 12% NR BS throughput degradation is observed for cell edge throughput. |
|  |  |  | h | NR UL->device | TBA | SINR degradation is 3dB for cell edge and 5dB for average. For device 1: - 11% @ 9.3dB SNR (max) - No outage percentage for min and average SNR For device 2a: - 5% @ 5.1 dB SNR (max) - No outage percentage for min and average SNR |

For the cases where ambient IOT reader/device is victim, outage percentage is smaller than 5% for some cases. It is recommended that if outage percentage is smaller than [5%], it can be considered no outage or no performance impact.

For case c (Reader->NR DL) in deployment scenario 1-2 and 2-2 (i.e. when legacy NR UE is indoor), throughput degradation is observed for both cell edge and cell average throughput. In order to avoid interference from ambient IOT to NR UE, some existing mechanisms based on gNB implementation can be considered. In existing deployed network, for RRC idle mode, NR UEs camp on cells based on network frequency priority, which can avoid NR UEs camp on the same frequency as ambient IOT. For RRC connected mode, gNB can handover NR UEs to a different cell (e.g. indoor gNB) based on existing measurement report triggering and event.

Proposal 1: If outage percentage is smaller than [5%], consider no outage or no performance impact for ambient IOT reader/device.

Proposal 2: Capture the following contents as the general remarks on coexistence findings.

- For the deployment scenarios with legacy NR UE indoor (scenario 1-2, 2-2, 5-2, 6-2), which NR throughput degradation is observed, some mechanisms based on gNB implementation can be considered. In existing deployed network to avoid/mitigate the interference from ambient IOT to NR UE. For example, for RRC idle mode, NR UEs can camp on a frequency different from ambient IOT system based on frequency priority configured by network; for RRC connected mode, gNB can handover NR UEs to a different cell (e.g. indoor gNB) based on existing measurement report triggering and event.

# 3. Conclusions

In this contribution, co-existence observations and proposals are provided as follows:

Proposal 1: If outage percentage is smaller than [5%], consider no outage or no performance impact for ambient IOT reader/device.

Proposal 2: Capture the following contents as the general remarks on coexistence findings.

- For the deployment scenarios with legacy NR UE indoor (scenario 1-2, 2-2, 5-2, 6-2), which NR throughput degradation is observed, some mechanisms based on gNB implementation can be considered. In existing deployed network to avoid/mitigate the interference from ambient IOT to NR UE. For example, for RRC idle mode, NR UEs can camp on a frequency different from ambient IOT system based on frequency priority configured by network; for RRC connected mode, gNB can handover NR UEs to a different cell (e.g. indoor gNB) based on existing measurement report triggering and event.

| deployment scenario and topology |  | spectrum | Evaluation case No. | (Aggressor -> Victim) | Proposals |
| --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | b | NR DL->reader | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | b | NR DL->reader | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
|  |  |  | c | Reader->NR DL | about 10% throughput degradation for average throughput is observed, about 70% throughput degradation for cell edge throughput is observed |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
|  |  |  | f | NR UL->reader | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
|  |  |  | f | NR UL->reader | SINR degradation is 17dB for cell edge and 4dB for cell average. Outage percentage is: - 17% @ 0.23dB SNR, - 13% @-4.3dB - 10% @ -7.8dB SNR |
|  |  |  | c | Reader->NR DL | about 30% throughput degradation for average throughput is observed, about 70% throughput degradation for cell edge throughput is observed |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | f | NR UL->reader | SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. |
|  |  |  | g | Reader->NR UL | about 6% throughput degradation is observed for average throughput, about 13% NR BS throughput degradation is observed for cell edge throughput. |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. For device 1, 4% outage percentage @ 9.3dB SNR (max), no outage percentage for average and min SNR. No outage percentage for device 2a |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | f | NR UL->reader | SINR degradation is 20dB for cell edge and 4.8dB for average Outage percentage is: - 12% @ -5dB SNR, - 11% @-6dB - 10% @ -7.6dB SNR |
|  |  |  | g | Reader->NR UL | about 6% throughput degradation is observed for average throughput, about 12% NR BS throughput degradation is observed for cell edge throughput. |
|  |  |  | h | NR UL->device | SINR degradation is 3dB for cell edge and 5dB for average. For device 1: - 11% @ 9.3dB SNR (max) - No outage percentage for min and average SNR For device 2a: - 5% @ 5.1 dB SNR (max) - No outage percentage for min and average SNR |

- Text Proposal to TR 38.769

------------Start of the Change-------------

# 6.5.6 Summary of co-existence evaluation

# ============unchanged part agreed in R4-2416356 are omitted=========

For D1T1 deployment scenario 1-1 and 2-1, which is NR macro BS/NR UE outdoor and ambient IOT system indoor, no performance degradation for NR system and [no performance impact for ambient IOT system.]

For D1T1 deployment scenario 1-2 and 2-2, which is NR macro BS outdoor and NR UE/ambient IOT system indoor, NR DL throughput degradation is observed for both cell edge and cell average throughput, outage percentage for ambinet IOT is TBA.

For D1T1 deployment scenario 3 and 4, TBA

For D2T2 deployment scenario 5-1, which is NR macro BS/NR UE outdoor and ambient IOT system indoor, NR DL throughput degradation is TBA.

For D2T2 deployment scenario 5-2, which is  NR macro BS outdoor and NR UE/ambient IOT system indoor, NR UL throughput degradation is observed for both cell edge and cell average throughput.

For D2T2 deployment scenario 6-1 and 6-2, TBA.

For the deployment scenarios with legacy NR UE indoor (scenario 1-2, 2-2, 5-2, 6-2), which NR throughput degradation is observed, some mechanisms based on gNB implementation can be considered. In existing deployed network to avoid/mitigate the interference from ambient IOT to NR UE. For example, for RRC idle mode, NR UEs can camp on a frequency different from ambient IOT system based on frequency priority configured by network; for RRC connected mode, gNB can handover NR UEs to a different cell (e.g. indoor gNB) based on existing measurement report triggering and event.

# 5. Reference

- R4-2417196, WF on co-existence study for A-IOT, CMCC

- R4-2414304, WF on Co-existence study for ambient IoT

- R4-2406714, WF on Ambient IoT in NR, CMCC

- R4-2407008, LS on Ambient-IoT evaluation scenarios and assumptions, CMCC