3GPP TSG-RAN WG4 Meeting # 113                                                              R4-2418731

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.10.2.2

Source: 	CMCC

Title: 	Discussion on UE RF requirements for ATG with inter-band CA

Document for:	Discussion

# 1. Introduction

RAN#105 meeting approved revised WID on Enhancements for Air-to-ground network for NR in Rel-19 [1]. After discussions of several meetings, we have made good progress on most of the ATG inter-band CA requirement, and we will focus on the n39 UL for CA_n3A-n39A.

# 2. Discussion

## 2.1. n39 UL for CA_n3A-n39A

During last meeting, we have reached the agreement as below.

Issue 2-1: whether to allow n39 UL for CA_n3A-n39A Agreement: - The conclusions about allowing n39 UL for CA_n3A-n39A if any should only apply to ATG and will not have any impact on existing NR specification for legacy NR UE. - Further check if the performance of these parameters in the following table can be achieved in ATG UE with 5MHz frequency gap in n39 filter. - Parameters; Values. Antenna Isolation; 20dB. The rejection for band n3 Rx filter at 1885~1915MHz; 50dB. n39 Tx filter rejection at 1805~1880MHz; 50dB. - Other RF architectures or parameters are not precluded.

According to the current specifications, only UL for band n39 has been considered. Companies have verified the cross-band isolation from band n3 UL to band n39 DL to address the back effect within band n39. Taking into account the distinct architectures between handheld UE and ATG CPE, the aforementioned RF architectures and parameters are deemed feasible.

Proposal 1:  The above RF architecture and parameters are feasible.

If the above architectures and parameters are feasible for every company, the MSD test should be discussed. Based on our spectrum deployment, the following test point should also be considered.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | [TBD] | ACLR2 |

Proposal 2:  Another test point could also be considered.

## 2.2. UE antenna type for inter-band CA

For UE antenna type, we have the following agreement during RAN4 112 meeting. It means that the UE antenna type for the band in the CA could be same as antenna type per band.

Issue 3-1-2: whether new capability is needed for ATG CA Agreement: - no need for new capability for UE antenna type for ATG CA - Antenna type per band could be applied to the same band in the band combination.

During the last meeting, RRM part discussed the following issue, and this issue need RF part to clarity whether case 4 is excluded based on the RAN4 agreement.

Issue 1-1-2: UE antenna type for inter-band CA Agreement: - Refine the agreement from last meeting as: - For intra-band contiguous CA, same antenna type should be applied on a band. - For inter-band CA, either different or same antenna type can be applied on different bands. - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC. - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC. - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC. - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.] - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed. Further check which understanding is correct: - Understanding #1: Case 4 is excluded in RF session. - Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.

In our view, the consensus reached during the RF session was to define the type of UE antenna and associated RF requirements. The five cases presented are distinct implementation proposals aimed at achieving the specified UE antenna configurations. Specifically, for case 4, if two bands assume the same antenna type and the component CCs are in close proximity, it is possible to share a single antenna array. Therefore, we believe that the aforementioned cases should be considered, and these implementations are feasible.

Proposal 3: RF session doesn’t exclude the Case 4 based on the reached agreement.

# 3. Conclusions

In this contribution, ATG UE with inter band CA requirements are discussed with the following proposals:

Proposal 1:  The above RF architecture and parameters are feasible.

Proposal 2:  Another test point could also be considered.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | [TBD] | ACLR2 |

Proposal 3: RF session doesn’t exclude the Case 4 based on the reached agreement.

# 4. References

[1] RP-242381, Revised WID on Enhancements for Air-to-ground network for NR