3GPP TSG-RAN WG4 Meeting # 113                                    R4-2418739

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.20.2.3

Source: 	CMCC

Title: 	Discussion on existing Rx requirements for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [1]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 ACS requirements

Following capture the agreements from TR 38.858 for ACS requirements:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

Last meeting, we have the following requirements.

Agreement: - The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement. WF: - FFS on the followings: - If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed. - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

Similar to ACLR requirements, adjacent-channel co-existence simulation only focuses on not less than 10% grid shift values, and 0% grid shift is avoided. Considering it is hard to define the co-location ACS requirement, it is better to prioritize discussing the scenarios under non-colocation case first.

The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

Proposal 1: Prioritize discussing the scenarios under non-colocation case first.

Proposal 2: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

2.2 In-band blocking

Last meeting we have approved the WF below:

WF: - FFS on the followings: - For FR1 WA BS, RAN4 need to discuss whether a unified in-band blocking requirement is defined? - Option 1: a unified blocking requirement to consider both scenarios: coexistence with legacy TDD system in adjacent channel, and coexistence with new SBFD system in adjacent channel. - Option 2: separate requirements are defined, - One set of requirements for coexisting with legacy TDD system in adjacent channel - One set of requirements for coexisting with new SBFD system in adjacent channel - For FR1 WA BS co-existence study, FFS grid-shift values should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - In-band blocking requirement for FR1 MR SBFD BS - In-band blocking requirement for FR2-1 WA and MR SBFD BS - For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements.

For the first issue whether a unified in-band blocking requirement is defined, we think it is hard to define two set of requirements for coexisting with legacy TDD or SBFD system. Since it is also challenging to encapsulate these requirements within the specification.

Proposal 3: Set a unified blocking requirement to consider both scenarios.

Since the study item utilizes a 100% grid-shift as the simulation assumption, to align with the conclusions drawn from the study item, it is important to maintain consistency in simulation assumptions. Therefore, we recommend considering a 100% grid-shift for our simulations.

Proposal 4: 100% grid-shift should be considered.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: Prioritize discussing the scenarios under non-colocation case first.

Proposal 2: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

Proposal 3: Set a unified blocking requirement to consider both scenarios.

Proposal 4: 100% grid-shift should be considered.

# 4. Reference

[1] TR 38.858,

[2] R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF