3GPP TSG-RAN WG4 Meeting # 113                                    R4-2418743

Orlando, US, 18th – 22nd November, 2024

Agenda item:	6.11.2

Source: 	CMCC

Title: 	Discussion on simultaneous Rx/Tx for CA_n40-n41

Document for:	Discussion

# 1. Introduction

In RAN4 #112bis, simultaneous Rx/Tx operation for CA_n40-n41 were discussed and no consensus reached. The contribution provides continue discussion for this band combination.

- Discussion

Last meeting, we have reached the following WF

<Way forward> - Companies are asked to verify whether the same MSD value defined for CA_n40-n41 with 4Rx can be applied to 2Rx RF front end architecture for CA_n40-n41. - Provide further information on the need for new test point for CA_n40-n41, taking into consideration the applicable bandwidth and frequency range of n40 and n41 from operator(s). For example, wider frequency separation between n40 and n41, and wider DL bandwidth. - Companies are asked to propose suitable test point reflecting operator demand in next meeting - 2Rx RF front end architecture can be considered. - For n41, consider if a new signalling is necessary to support maximum of 2-layer MIMO with 2Rx antenna ports, if the UE reports both the capability of 4-layer MIMO for band n41 and simultaneous Rx-Tx for CA_n40A-n41A.

First of all, based on the WF and our spectrum deployment, we provide our suggestion of the new test point for CA_n40A-n41A.

Table 1 Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3/PC2 aggressor NR UL band for CA_n40-n41

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2565 | 100 | [TBD] | ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | [TBD] | ACLR2 |

Proposal 1: Consider use the above table as the new test point of CA_n40A-n41A.

Since bands n40 and n41 are close in frequency, their antennas might share the same physical elements. This architecture reduces deployment complexity and the number of required antennas but also introduces the problem we are discussing. From the network's perspective, there is no clear benefit to relaxing the 4Rx requirement to support simultaneous Rx-Tx, and we also notice the MSD analysis with n41 2Rx from some companies, it shows that the architecture of 2Rx doesn’t provide enough enhancement. So we do not dismiss the possibility that some legacy UEs could implement simultaneous Rx-Tx with n41's 4Rx capability.

Observation 1: During last meeting, the MSD analysis with n41 2Rx didn’t show enough enhancement.

We also believe there is no need to introduce new UE signaling for this matter. Initially, this situation arises only when CA_n40-n41 supports simultaneous Rx-Tx. Moreover, 4Rx is a mandatory requirement for n41, and whether a UE can concurrently support Uplink MIMO and simultaneous Rx/Tx should be based on the antenna deployment.

Based on the above analysis, it is preferable to maintain the requirement for n41 to use 4Rx in simultaneous Rx-Tx for CA_n40-n41.

Proposal 2: Keep the requirements with n41 using 4Rx in simultaneous Rx-Tx for CA_n40-n41.

# 3. Conclusions

In this contribution, question have been discussed with following observations and proposals.

Proposal 1: Consider use Table 1 as the new test point of CA_n40A-n41A.

Table 1 Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3/PC2 aggressor NR UL band for CA_n40-n41

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2565 | 100 | [TBD] | ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | [TBD] | ACLR2 |

Observation 1: During last meeting, the MSD analysis with n41 2Rx didn’t show enough enhancement.

Proposal 2: Keep the requirements with n41 using 4Rx in simultaneous Rx-Tx for CA_n40-n41.

# 4. Reference

R4-2417058, WF on simultaneous Rx-Tx for CA_n40-n41.