3GPP TSG RAN WG4 Meeting#113                                    R4-2418749

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	7.17.2.2

Source:	vivo

Title:	Discussion on AI/ML two-sided model logistical issues for CSI compression

Document for:	Discussion

- Introduction

It was agreed that data sharing (ML models of encoder or decoder or both, and/or datasets for training/testing/inference) will be required [1]. In last meeting, we have the following summary of discussion for AIML two-sided model logistical issues for CSI compression.

Agreement: All agreements for data sharing are only for the CSI compression use case feasibility study. These agreements can be revisited in an eventual WI phase. 2.1 AI/ML model format Agreement for AI/ML formats: Use at least ONNX format for sharing. Companies can also share PyTorch model The models in different formats from one company shall be the same Use version v1.16 or later ONNX version and opset version number to be included in the file 2.2 Dataset format Agreement: Use NumPy for dataset sharing Do not use pickled data (compression mechanism) Use npy – single array in each file 2.3 Training Data file format Agreement: Dataset/model input file format: - N (samples) X 2 (IQ) X nSB (number of subbands) X nPorts (number of CSI-RS ports) X nLayers (number of layers) - In other words, dataset should content following info: - 1st dimension: Number of samples - 2nd dimension: Real and imaginary - 3rd dimension: Number of sub-bands - 4th dimension: Number of antenna ports - 5th dimension: Number of layers – present only if >1 layer - Note: Each element of the dataset will be a float32 real number The dataset includes eigenvectors calculated based on average covariance channel matrix for the given MIMO layer for each sub-band based on pre-processing assumptions in WF R4-2414447 Single file for fixed scalar quantization(only dataset), TBD whether additional file is needed for variable/trainable quantization Dataset files can be split into multiple files to enable easier upload. - Use the 2 digits for split files starting from 00, increment for each additional file Split files and then archive each file 2.4 File naming scheme Agreement: - Folders for AI/ML data sharing and current WI/use case to be created under “RAN4 folder” - Subfolder created for each meeting File naming scheme( ML model file and dataset file) - a unique identifier for the company (4 characters – list to be maintained by RAN4 secretary) - meeting number - differentiate model and dataset by identifier (2 characters ml and ds) - additional identifier with 2 characters (TBD) - ec for encoder - dc for decoder - ei for encoder input - do for decoder output - lt for latent (decoder input or encoder output) - others to be added as needed - dataset can be split in multiple files – 2 digits - files to be compressed to zips and uploaded Example: Folders to be created under https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/ Folder for models: /Data_sharing/ NR_AIML_air/CSI_compression/Models/R4_XXX Folder for datasets: /Data_sharing/ NR_AIML_air/CSI_compression/Datasets/R4_XXX Example for model file: vivoR4_113mlec.onnx Example for dataset file: vivoR4_113dsei00.npy 2.5 Files to be shared Agreement: Files to be shared, to be decided based on option under study - Dataset containing encoder input per subband (as described in Section 5-3) - Encoder and/or decoder model (as described in Section 5-1)

In this contribution, we further provide our views on AI/ML two-sided model logistical issues for CSI compression.

- Discussion on logistical issues for CSI compression

Most of AI/ML two-sided model logistical issues for CSI compression have been addressed in last meeting. Only a few remaining issues are left to be discussed, and this contribution will focus on these remaining issues.

Since most companies commented that testing and inference data should be a subset of the training dataset, one open question is how to obtain the testing data from the dataset. Since the simulation data is generated under the i.i.d. assumption, the method of dividing the dataset would have small impact on the performance so that RAN4 could pick a simple dividing method. To reduce the differences between different companies, it would be helpful to make sure that different companies can obtain the same testing data from one aligned dataset. Then, randomly choosing testing data would be not suitable. One simple but effective method is that the testing data could be directly divided by ordering of samples. For example, the first 10%~12.5% or last 10%~12.5% samples are testing data, and the rest samples of dataset are training data (including validation data).

Proposal 1: For simplicity, it is suggested that the testing data could be directly divided by ordering of samples. For example, the first 10%~12.5% or last 10%~12.5% samples are testing data, and the rest samples of dataset are training data (including validation data).

The “lifetime” of the data will be discussed now. Temporary shared dataset or model from companies, which are outcomes of intermediate steps in the RAN4 study, would need to be stored for one release. So that companies would be enough of time to download and double check the model/dataset of other companies. Also, the storage for one release would not take so many storage resources.

The aligned dataset and model would need to be stored forever, since they may be captured in WF, TR or spec. The size of aligned dataset and model would be not large, so the storage resources would be affordable. If agreed, the reference model/dataset would need to be stored forever, since they will be captured in spec.

Proposal 2: For the lifetime of data:

- Temporary shared dataset or model from companies, which are outcomes of intermediate steps in the RAN4 study, would need to be stored for one release

- The aligned dataset and model would need to be stored forever.

- The reference model/dataset would need to be stored forever.

- Summary

Based on above analysis, following proposals and observations are present.

Proposal 1: For simplicity, it is suggested that the testing data could be directly divided by ordering of samples. For example, the first 10%~12.5% or last 10%~12.5% samples are testing data, and the rest samples of dataset are training data (including validation data).

Proposal 2: For the lifetime of data:

- Temporary shared dataset or model from companies, which are outcomes of intermediate steps in the RAN4 study, would need to be stored for one release

- The aligned dataset and model would need to be stored forever.

- The reference model/dataset would need to be stored forever.

- References

[1] R4-2414323, “WF on AI/ML”, Qualcomm Incorporated, RAN4#112