3GPP TSG RAN WG4 Meeting#113                                    R4-2418750

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	7.17.3.1

Source:	vivo

Title:	Discussion on testability and interoperability issues for CSI prediction

Document for:	Discussion

- Introduction

In R18 SI, we have studied on general aspects, specific issues related to use cases, and interoperability and testability aspects. The agreements have been collected in TR [1]. In RAN#105 meeting, CSI prediction has been agreed for normative work and then the requirement of CSI prediction needs to be defined in this release [2].

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

In previous meetings, we have the following agreements for CSI prediction.

Agreement: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined: - what is the baseline that throughput would be compared to Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

In this contribution, we further provide our views on testability and interoperability aspects.

- Discussion on CSI prediction

In the previous meetings, RAN4 have agreed to use relative throughput as requirement metric for CSI prediction, and companies are invited to bring contribution on baseline throughput.

Before the discussion of AI CSI prediction, we would like to review the RAN4 agreement of non-AI CSI prediction. In RAN4#110, it has been agreed to use random precoding based on Type I Single Panel codebook as the baseline throughput.

Issue 1-1-4: clarify test metric for PMI reporting requirements with ‘typeII-Doppler-r18’ codebook Agreement: - Test metric defined as γ1=ttypeII-dopplertrnd, where ttypeII-doppler is X % of the maximum throughput obtained at SNRtypeII-doppler using the typeII-Doppler-r18 precoder configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-doppler with random precoding based on Type I Single Panel codebook

In RAN4#110bis, the test setup and simulation assumptions of non-AI prediction have been agreed.

2.1 Sub-topic 1-1 Test setup and simulation assumptions for TypeII Doppler Issue 1-1-2: N4 and K configuration Agreement: - N4=1 and K=4 for both FR1 FDD and FR1 TDD. Issue 1-1-3: X% of the maximum throughput in Test metric Agreement: - 90% of the maximum throughput in Test metric Issue 1-1-4: Test metric of TypeII-Doppler-r18 codebook Decide the gamma value based on the updated simulation results on the May meeting Issue 1-1-5: Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook Agreement: - For FR1 FDD 15kHz SCS case, test setup is as below descriptions: - Non-CSI slots: scheduled in mod(i, 10) = {1,3,5,7,8,9}, i={0,1,2,…,19} - CSI-RS is scheduled in mod(i, 10) = {0,2,4,6}, i={0,1,2,…,19} - SSB slots: i=0 - TRS slots: i=10 and 11. - CSI report is transmitted in slot#10n+10 to estimate precoder in slot#10n+11 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 17, 18, 19, 21, 23} Issue 1-1-6: Test setup for FR1 TDD case of TypeII-Doppler-r18 codebook Agreement: - For FR1 TDD 30kHz SCS case, test setup is as below descriptions for the simulation of following Rel18 Doppler codebook vs random Type I single panel codebook: - Scheduling pattern for TDD SCS=30kHz with the periodicity of 20ms (40 slots): - Non-CSI slots: scheduled in mod(i, 10) = {5,6}, i={0,1,2,…,39} - CSI slots: scheduled in mod(i, 10) = {0,1,2,3}, i={0,1,2,…,39} - SSB slots: i=0 - TRS slots: i=20 and 21. - CSI report is transmitted in slot#10n+9 to estimate precoder in slot#10n+10 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 16}

From the view of test design, AI prediction and non-AI prediction could follow the similar test procedure, to simply the RAN4 test. Then, the agreement of non-AI prediction could be used as reference and the baseline of AI prediction could be the throughput from random precoding.

In the last meeting, we have the following agreement that AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based). Since the requirement of non-AI prediction has been agreed, AI-prediction would have similar or higher gamma value compared to non-AI prediction.

Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

Observation 1: In non-AI CSI prediction, random precoding based on Type I Single Panel codebook has been agreed as the baseline throughput

Proposal 1: The baseline of AI CSI prediction could be the throughput from random precoding, which means that AI prediction and non-AI prediction could follow the similar test procedure. AI-prediction would have similar or higher gamma value compared to non-AI prediction.

In the study of R18 and R19, AI CSI prediction is supposed to achieve some performance gain compared to non-AI CSI prediction, and then it may be not suitable for AI CSI prediction to directly reuse the the test setup and simulation assumptions of non-AI CSI prediction. RAN4 could work on the test setup and simulation assumptions of AI CSI prediction, by the future evaluations. For this one-sided use case, reference model should be defined to align results from companies, and then the requirements could be derived based on the aligned results. Then in the next meeting, RAN4 could discuss the simulation assumption and reference model, where RAN1 agreements may be used as starting point.

Proposal 2: RAN4 to discuss simulation assumptions and reference model for AI CSI prediction.

- Summary

Based on above analysis, following proposals and observations are present.

Observation 1: In non-AI CSI prediction, random precoding based on Type I Single Panel codebook has been agreed as the baseline throughput

Proposal 1: The baseline of AI CSI prediction could be the throughput from random precoding, which means that AI prediction and non-AI prediction could follow the similar test procedure. AI-prediction would have similar or higher gamma value compared to non-AI prediction.

Proposal 2: RAN4 to discuss simulation assumptions and reference model for AI CSI prediction.

- References

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep 9-12, 2024.