3GPP TSG RAN WG4 Meeting #113	R4-2418751

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.17.4.1

Source:	vivo

Title:	Discussion on RRM core requirements for beam management

Document for:	Discussion

- Introduction

In RAN4#112-bis meeting, there were some further discussions on key issues including KPIs related, the simulation assumption of measurement error, and the potential RRM requirements for beam prediction. The latest outcome was recorded in the latest WF [1].

In this contribution, we will analyze the impacts on requirements for beam prediction and further provide some simulation results on candidate KPIs/ Test Metrics with consideration of the measurement error modeling derived from the agreed simulation assumption [2].

- Requirements for beam predictions

In the last meeting, the discussion on the RRM impacts on beam management was triggered. The package of requirements for beam predictions was initially identified as follows. Next, we will provide our analysis for each requirement.

Issue 2-5: Requirements for beam predictions Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed

2.1	Measurement period for prediction

For the requirements on measurement period for prediction, it aims to specify the latency of data collection for prediction before inference. To be specific, the process of data collection for model inference data refers to UE performing the measurements for set B and obtaining the corresponding results as model inputs. For AI/ML, in addition to data collection for inference, the prediction also needs the involvement of other relevant procedures, e.g., model inference, and the reporting of prediction results. On a related note, there was a relevant agreement on inference latency handling, which was made in the discussion on the general aspect in RAN4#111. The details are shown below:

Issue 1-4: Inference latency handling Agreement: - Use the following as the starting point - Inference latency can be implicitly captured in the delay of corresponding procedures (e.g. reporting delay) and/or performance requirements. - Inference latency is part of delay of the corresponding procedure and no separate requirement will be specified for inference latency. - Check the above bullets based on per use case, and if other WGs define the inference latency, RAN4 can revisit the above bullets.

It can be observed from the above agreement that the inference latency is regarded as a part of the delay of the corresponding procedure and it would be implicitly captured in the delay of procedure. In our understanding, the procedures herein refer to the entire prediction process starting from the reception of radio signal by UE and ending at the reporting of prediction results. Specifically, the measurements of set B are also included. From this point of view, it is better to define the latency requirements for the entire prediction process of BM, including the measurements for prediction, inference process, and prediction reporting, rather than treating each delay as a separate requirement.

Based on the above considerations, we have the following proposal:

Proposal 1: RAN4 to define the latency requirements on entire prediction process for BM, the following can be the baseline:

- The latency starts from the reception of radio signals by UE and ends at the reporting of prediction results.

- The delay components at least include:

  - Measurement delay for prediction: the time required to collect input data (measurements of set B) for inference

  - Inference delay: the time for using a trained AI/ML model to produce BM prediction results

  - Delay uncertainty for reporting: the delay uncertainty in acquiring the first available reporting occasion

2.2	TCI state known/unknown

Issue 2-1: UE Rx beam knowledge Proposals - Option 1: UE Rx beam is known - the UE Rx beam gain has to be taken into account when making the prediction - Option 2: UE Rx beam is unknown - Option 3: depends on some conditions - which conditions? - Option 4: others

RAN4 further discussed the issue of UE Rx beam knowledge shown above. To be specific, the main divergence lies in whether the Rx beam is implicitly considered as input information for DL Tx beam prediction. In our understanding, it is up to UE on how to implement determining the corresponding Rx beam based on the predicted Tx beam. In some implementations, UE first predicts the Tx beam and further determines the best Rx beam paired with the Tx beam by beam sweeping. For some UEs with specific model implementation, the information of Rx beam may be implicitly indicated or included as the model input, and then UE can directly know Rx beam corresponding to the best TX beam. From UE implementation, it is not preferred to make it mandatory to assume UE always knows the Rx beam paired with the predicted Tx beam.

For the definition of requirements on known TCI state, one approach is to differentiate different UE implementations by introducing UE capability. This capability refers to UE being capable of supporting DL Tx beam prediction together with the determination of paired Rx beam. For the UE that supports this capability, the subsequent procedure by using beam sweeping on each Rx beam to determine the best Rx beam can be skipped.

Proposal 2: For whether Rx beam is assumed as known in case of DL Tx beam prediction, RAN4 to introduce UE capability of supporting DL Tx beam prediction with knowledge of paired Rx beam

2.3	Performance monitoring

In addition to the aspects listed in issue 1-4, the framework on performance monitoring is beginning to take shape after RAN1/2 discussion. For information, we copy the latest RAN1 conclusions on performance monitoring for BM as follows.

Agreements in RAN1#118bis - For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives - At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results.

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, the method of UE-assisted performance monitoring is supported, which is achieved by comparing the prediction results with the actual measurements results, calculating the final performance metric(s) and then reporting it to NW for assisting further decision-making on model management (e.g., model activation/deactivation, switch, fall-back).

From RAN4 perspective, the requirements on performance monitoring shall be considered. However, before discussing the definition of requirements in detail, we believe that it needs to first figure out the testing goal for performance monitoring.

As we mentioned above, for UE-assisted performance monitoring, the set of monitoring actions, including the comparison between prediction results and actual measurement results and the evaluation by calculating performance metrics, are performed on UE side. From UE perspective, it is expected to correctly perform the whole monitoring process. As alternatives, the corresponding UE behavior can be verified from different perspectives as follows:

-	Option 1: The testing goal is to verify whether the performance metrics can be correctly calculated by UE

-	Option 2: The testing goal is to verify whether UE can report the performance metrics within a certain latency upon performance monitoring is required.

-	Option 3: both of the testing goals in Option 1 and 2 need to be verified

For Option 1, from NW perspective, the decision-making for subsequent actions (e.g., model deactivation, fallback) highly depends on the monitoring results of prediction performance. Therefore, the correctness of the performance metric reported by UE is a key point for verifying.

In terms of Option 2, it focuses on the performance metric that can be reported to the network within the specific latency and further ensures NW can make corresponding model management according to the results in time. Further details on reporting, e.g., when and how to report are still under discussion in RAN1.

For Option 3, both Option 1 and 2 are required.

Based on the above analysis, we propose to first figure out what is to be verified for performance monitoring and further work on the requirements design based on the agreed testing goal.

Proposal 3: For performance monitoring for BM, RAN4 to first determine the testing goal. The following alternatives can be considered:

- Option 1: The testing goal is to verify whether the performance metrics can be correctly calculated by UE;

- Option 2: The testing goal is to verify whether UE can perform monitoring and report the performance metrics within a certain latency upon performance monitoring is required;

- Option 3: The combination of Option 1 and Option 2.

2.4	KPIs related

<RSRP accuracy>

For the discussion on KPIs for beam management, there was an initial alignment on the definition of RSRP Absolute accuracy. Specifically, the comparison of RSRP difference is between predicted L1-RSRP and ground-truth of L1-RSRP with the same beam index. The agreement is shown as follows.

Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on ground-truth L1-RSRP].

For the FFS part in bracket [in top-K beams based on ground-truth L1-RSRP], we would like to put several possibilities on the table for further analysis.

- Definition 1: The index i may be any beam index

- Definition 2: The index i may be any beam index in top-K beams based on ground-truth L1-RSRP

- Definition 3: The index i may be any beam index in top-K beams based on predicted L1-RSRP

For definition 1, if we assume that the value of index i can range from 1 to N, where N is the total number of beams in set A, it means that the predicted RSRPs corresponding to all Beams of the predicted set A need to meet RSRP accuracy requirements. Given that only Top-K predicted beams and corresponding predicted RSRP (if needed) are required to report, for the remaining beams, whether the corresponding RSRP is predicted accurately may not be so critical. From this point of view, the definition 1 is not preferred.

For definition 2, if the predicted beams selected to meet accuracy requirement are based on ground-truth L1-RSRP, there may be a case to be noticeable as follows.

Case: The Top-K beam based on predicted L1-RSRP (e.g., beam X) is not any of the Top-K beams based on ground-truth L1-RSRP

To explore the probability of this case occurring in actual predictions, we have some simulations and the results provided below. Table 1 shows the observation that there is a certain possibility that the Top-K beam based on predicted L1-RSRP is not any of the Top-K beams based on ground-truth L1-RSRP. For example, for the Top-index(index=2) beam based on predicted L1-RSRP, the probability that this beam is one of the Top-2 beams based on ground-truth L1-RSRP is 72.15%. Further, if the measurement error is considered in model input, the probability of this happening will be lower.

Table 1 the probability that Top-index predicted beam is one of the Top-K(K=2/5) genie-aided beams

| Simulation results |  | Probability (%) |  |  |
| --- | --- | --- | --- | --- |
| Inference SNR |  | -3dB |  |  |
| Predicted Top-index Beam | the Top-index predicted beam is one of the Top-2 genie-aided beams | No error | Case 1NOTE | Case 2 NOTE |
| Index=1 |  | 92.18 | 86.02 | 91.57 |
| Index=2 |  | 72.15 | 53.41 | 69.64 |
| Predicted Top-K Beam | the Top-index predicted beam is one of the Top-5 genie-aided beams | No error | Case 1 | Case 2 |
| Index=1 |  | 99.60 | 99.12 | 99.52 |
| Index=2 |  | 99.36 | 98.07 | 99.28 |
| Index=3 |  | 97.21 | 93.33 | 96.79 |
| Index=4 |  | 88.91 | 83.29 | 88.59 |
| Index=5 |  | 71.43 | 63.29 | 70.12 |
| NOTE: The case 1 and case 2 refer to the measurement error modelling described in Clause 3 |  |  |  |  |

Observation 1: There is a certain probability that the Top-K predicted beam is not any of the Top-K beams based on ground-truth RSRP.

In this case, according to definition 2, the predicted L1-RSRP of beam X is not required to satisfy accuracy requirement as this beam is not any of the Top-K beams based on ground-truth L1-RSRP. However, the truth is that the corresponding predicted L1-RSRP of beam X still needs to be reported (if needed). For DL Tx beam prediction, UE is responsible for predicting and reporting the strongest beams and corresponding predicted RSRP since any reported beam may be selected by the network as the Tx beam to be used, so the corresponding RSRP for each reported beam needs to have a certain accuracy. Therefore, if we go with definition 2, in the case of the Top-K beam based on predicted L1-RSRP is not any of the Top-K beams based on ground-truth L1-RSRP, the corresponding predicted RSRP cannot be guaranteed.

Observation 2: For RSRP Absolute accuracy, if the index i can be any beam index in top-K beams based on ground-truth L1-RSRP, the RSRP accuracy for those beams which belong to Top-K predicted beam but not any of the Top-K beams based on ground-truth RSRP cannot be guaranteed.

Based on the above analysis, we propose definition 3 that the index i refers to any beam index in top-K beams based on predicted L1-RSRP. For this definition, it aims to ensure the accuracy of all predicted RSRPs of reported beams.

Proposal 4: The absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i can be any beam index in top-K beams based on predicted L1-RSRP.

<Beam prediction accuracy>

On the reporting content for BM, it was agreed in RAN1 that at least the beam index is mandatory to be reported. From KPIs perspective, the accuracy of beam index prediction shall be considered. On the other hand, as we mentioned in clause 2.3, RAN1 is working on the details for performance monitoring, and the metric was determined as beam prediction accuracy. Based on these two points, it can be confirmed that beam prediction accuracy is considered as one of metrics/KPIs for defining requirements on BM.

Proposal 5: RAN4 to confirm to use beam prediction accuracy as one of metrics/KPIs for defining requirements on BM.

In this section, we share our initial thinking on metrics/KPIs for BM. Next, we provide further analysis on RSRP accuracy and beam prediction accuracy combined with the impact on the measurement error in clause 3.

- The impact of measurement error for inference performance

Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point.

In the last meeting, it was agreed that RAN4 would start a specific study to assess the impact of measurement errors on prediction accuracy. For reference, there were some initial simulation results provided in our paper in RAN4#112 and RAN4#112-bis meeting. In this section, we make some updates on the results based on the agreed simulation assumption [2] and further provide some observations and proposals from the results. The detailed simulation assumptions and AI model description are shown in the Appendix.

3.1 The baseband error impact on prediction accuracy for different metrics for beam management

Here we provide several cases to show performance on beam prediction accuracy under LLS-based measurement error modeling and truncated Gaussian distribution.

- Case 1: SNR is assigned as -3dB for LLS-based modeling

- Case 2: Truncated Gaussian distribution is modeled between -2.5dB to 2.5dB.

- Case 3: SNR is assigned as -6dB for LLS-based modeling

- Case 4: SNR is assigned as 0dB for LLS-based modeling

In Table 2, the metrics including Top-1 (%), Top-K/1 (%) and Top-1/K (%) are considered.

Table 2 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction by considering measurement error

| Simulation results |  | Prediction accuracy (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inference SNR |  | -3dB |  |  | -6dB |  | 0dB |  |
| Case |  | No error | Case 1 | Case 2 | No error | Case 3 | No error | Case 4 |
| Top 1 | +0dB | 68.72 | 49.00 | 66.35 | 69.10 | 66.20 | 72.19 | 71.17 |
|  | +1dB | 90.98 | 72.85 | 87.79 | 89.40 | 85.17 | 92.22 | 91.12 |
| Top 2/1 | +0dB | 87.95 | 77.35 | 86.59 | 84.09 | 82.93 | 88.67 | 87.91 |
|  | +1dB | 92.74 | 85.54 | 91.65 | 89.48 | 89.64 | 92.65 | 92.39 |
| Top 3/1 | +0dB | 91.38 | 85.54 | 90.36 | 87.99 | 87.08 | 92.31 | 91.21 |
|  | +1dB | 92.74 | 88.27 | 92.53 | 89.40 | 89.31 | 93.41 | 92.98 |
| Top 4/1 | +0dB | 93.38 | 89.00 | 92.77 | 90.22 | 89.98 | 94.00 | 93.66 |
|  | +1dB | 93.62 | 89.48 | 93.01 | 90.56 | 90.56 | 94.25 | 93.91 |
| Top 5/1 | +0dB | 96.41 | 92.29 | 95.50 | 93.45 | 93.37 | 96.70 | 96.70 |
|  | +1dB | 96.41 | 92.45 | 95.58 | 93.62 | 93.45 | 96.70 | 96.70 |
| Top 1/2 | +0dB | 87.47 | 74.78 | 86.35 | 85.34 | 83.51 | 88.59 | 87.40 |
|  | +1dB | 95.45 | 88.03 | 94.78 | 94.45 | 93.21 | 96.37 | 95.86 |
| Top 1/3 | +0dB | 94.01 | 85.22 | 92.93 | 91.22 | 89.48 | 93.66 | 92.73 |
|  | +1dB | 98.08 | 94.70 | 97.67 | 97.10 | 96.52 | 98.06 | 97.72 |
| Top 1/4 | +0dB | 96.97 | 90.12 | 95.74 | 94.95 | 93.79 | 96.96 | 95.69 |
|  | +1dB | 99.20 | 96.71 | 98.80 | 98.59 | 98.43 | 99.41 | 99.07 |
| Top 1/5 | +0dB | 98.00 | 92.85 | 97.35 | 97.02 | 96.44 | 98.22 | 97.46 |
|  | +1dB | 99.52 | 97.83 | 99.20 | 98.92 | 98.92 | 99.66 | 99.75 |

On the performance impact of measurement error, based on the simulation results, we have several observations:

- No matter for which metrics, measurement errors will degrade the beam prediction performance with AI/ML. Especially for the metric Top-K/1 and Top-1/K when K≤1,2, the degradation is non-negligible.

- With the decrease of SNR, the beam prediction accuracy will also decrease. However, the difference in prediction accuracy between Case 1 (SNR= -3dB) and Case 4 (SNR= 0dB) is not obvious.

- Compared with Truncated Gaussian distribution, the error derived from LLS-based modeling has a greater impact on the decline of beam prediction accuracy performance.

Observation 3: There is a non-negligible performance degradation in beam prediction accuracy when considering measurement error modeling for inference.

Observation 4: With the decrease of SNR, the beam prediction accuracy will also decrease. However, when the SNR reaches a certain level (e.g., 0dB), this performance decline will stabilize.

Observation 5: Compared with Truncated Gaussian distribution, the error derived from LLS-based modeling has a greater impact on the decline of beam prediction accuracy performance under similar SNR levels.

Based on the above observations, we believe the measurement error and corresponding methods of error modeling are the important factors affecting the prediction performance. Thus, it needs to be considered when RAN4 define the requirements for beam prediction accuracy

Proposal 6: RAN4 to define beam prediction accuracy requirements with consideration of the impact on measurement error.

Additionally, we also study the performance when [X]dB margin is considered in the beam prediction accuracy. Specifically, in our simulations, the Top-1 with X=1dB margin means that the percentage of the Top-1 predicted beam "whose ideal L1-RSRP is within X=1dB of the ideal L1-RSRP of the Top-1 genie-aided beam".

And for Top-K/1 with X=1dB margin, the criteria for judgment are:

- If the Top-1 genie-aided beam is one of the Top-K predicted beams, recorded as correct prediction

- Else if all the differences between the ideal RSRP of ideal Top-1 beam and the predicted RSRP of predicted Top-K beam are within the X=1dB range, recorded as correct prediction, recorded as correct prediction

- Otherwise, recorded as wrong prediction

Besides, for Top-1/K with X=1dB margin, the criteria for judgment are

- If the Top-1 predicted beam is one of the Top-K genie-aided beams, recorded as correct prediction

- Else if the predicted RSRP of predicted Top-1 beam is within the range of {[ideal RSRP of ideal Top-K beam – 1dB], [ideal RSRP of ideal Top-1 beam + 1dB]}, recorded as correct prediction

- Otherwise, recorded as wrong prediction

From the results, we can observe that if 1dB margin is considered in statistics, it will bring a noticeable improvement in the rate of prediction accuracy, especially for the Top 1 case.

Observation 6: In addition to the rate of correct beam prediction, when additional 1dB RSRP margin is considered (e.g., For the success rate of the difference between the ideal RSRP of ideal Top-1 beam and the predicted RSRP of predicted Top-K beam are within the X=1dB range), it will bring a noticeable improvement in the rate of prediction accuracy.

Furthermore, to figure out the impact of difference values of X on prediction accuracy and to facilitate RAN4 discussion on whether and how to determine the appropriate value of X margin, we provide some simulations on the probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the X= [1, 2.5, 5] dB range. Specifically, for the selection of the benchmark (i.e., the RSRP of Top-1 beam), we consider including

1)	Predicted RSRP of predicted Top-1 beam;

2)	Ideal RSRP of ideal Top-1 beam;

3)	Predicted RSRP of predicted Top-1 beam with consideration of 4dB measurement error;

4)	Ideal RSRP of ideal Top-1 beam with consideration of 4dB measurement error

Table 3 The probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the X=1dB range

| Simulation results |  | Probability (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Predicted top 1 |  |  | Ideal top 1 |  |  |
|  |  | No error | Case 1 | Case 2 | No error | Case 1 | Case 2 |
| Top 1 | X=1dB | 100.00 | 100.00 | 100.00 | 88.91 | 52.85 | 78.88 |
| Top 2/1 |  | 98.40 | 95.90 | 98.47 | 82.68 | 43.45 | 67.47 |
| Top 3/1 |  | 59.46 | 52.21 | 60.32 | 50.52 | 25.38 | 41.04 |
| Top 4/1 |  | 4.71 | 5.70 | 4.66 | 5.27 | 4.90 | 7.31 |
| Top 5/1 |  | 1.76 | 2.65 | 1.85 | 1.92 | 1.12 | 1.29 |

Table 4 The probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the X=2.5dB range

| Simulation results |  | Probability (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Predicted top 1 |  |  | Ideal top 1 |  |  |
|  |  | No error | Case 1 | Case 2 | No error | Case 1 | Case 2 |
| Top 1 | X=2.5dB | 100.00 | 100.00 | 100.00 | 97.45 | 91.57 | 96.95 |
| Top 2/1 |  | 99.68 | 99.60 | 99.68 | 96.09 | 88.76 | 95.34 |
| Top 3/1 |  | 97.37 | 94.30 | 97.19 | 91.86 | 79.12 | 90.04 |
| Top 4/1 |  | 60.18 | 57.35 | 58.80 | 53.87 | 48.35 | 53.49 |
| Top 5/1 |  | 13.73 | 13.09 | 13.25 | 11.41 | 11.16 | 11.33 |

Table 5 The probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the X=5dB range

| Simulation results |  | Probability (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Predicted top 1 |  |  | Ideal top 1 |  |  |
|  |  | No error | Case 1 | Case 2 | No error | Case 1 | Case 2 |
| Top 1 | X=5dB | 100.00 | 100.00 | 100.00 | 99.92 | 99.68 | 99.92 |
| Top 2/1 |  | 100.00 | 100.00 | 100.00 | 99.76 | 99.44 | 99.84 |
| Top 3/1 |  | 99.36 | 99.20 | 99.36 | 98.64 | 98.39 | 98.63 |
| Top 4/1 |  | 96.65 | 93.25 | 96.31 | 94.81 | 90.44 | 93.90 |
| Top 5/1 |  | 34.24 | 33.41 | 33.01 | 31.92 | 31.81 | 31.33 |

From the Table 3-5, we can observe that:

- As the X value increases, the probability of Top-K falling into this range increases. When X is 1, the probability of Top-2 falling into this range is only 70%-80%

- When [Predicted RSRP of predicted Top-1 beam] and [Ideal RSRP of ideal Top-1 beam] are used as benchmarks, the probability of falling into the X range is similar

- The measurement error has some impact on the probability when the value of X is small

Observation 7: The probability of predicted RSRP of predicted Top-K beam falling into the range of [the RSRP of Top-1 beam± XdB] has significant improvement with the increase of X value.

Observation 8: When using predicted RSRP of predicted Top-1 beam and Ideal RSRP of ideal Top-1 beam as the benchmark, the probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the [X]dB range are similar.

Besides, based on the simulation results of following metrics, i.e., Top-1, Top-K/1 and Top-1/K, we have the following observations:

- With the increasing number of K (i.e., Top K/1 and Top 1/K), the prediction accuracy increases accordingly

- When measurement error is considered, there is significant performance degradation on Top-1 beam prediction (even lower than 50% accuracy in some cases).

- The performance on prediction accuracy defined as Top-K/1and Top-1/K are closed.

From RRM requirement perspective, RAN4 needs to determine one metric on beam prediction accuracy among these candidate metrics. Firstly, as per the observations above, the metric of Top-1 lacks certain robustness. And for the metrics of Top-K/1 and Top-1/K, the performance is approximate so that RAN4 can only choose one of them as the KPI. In our thinking, considering there is a case in UE that just reports the beam ID of Top-K without any ordering information. The Top-1 predicted beam may not be known, Top-K/1 is more appropriate as the KPI to figure out even without any rankings.

Proposal 7: RAN4 to use Top-K/1 as the metric of beam prediction accuracy for further performance evaluation.

For RSRP accuracy, according to our analysis in clause 2.4, here we will assume the following definition as RSRP accuracy and the following simulations for RSRP difference using it as KPI.

The absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i can be any beam index in top-K beams based on predicted L1-RSRP

Table 6-8 show the simulation results of the RSRP accuracy under different SNR levels, including SNR=-6, -3, 0, respectively. Note that the simulation is under the case that the ground truth without measurement error is regarded as the benchmark. The measurement errors are only added in the model input.

Table 6 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beams and the ideal L1-RSRP of the same beams (SNR=-3dB for inference)

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 1.46 |  |  |  |  |
| Top 5/1 | 1.46 | 1.71 | 1.91 | 2.36 | 2.70 |
| RSRP difference (dB) considering errors case 1 (only add in model input) |  |  |  |  |  |
| Top 1 | 3.86 |  |  |  |  |
| Top 5/1 | 3.86 | 3.58 | 3.76 | 3.99 | 4.86 |
| RSRP difference (dB) considering errors case 2 (only add in model input) |  |  |  |  |  |
| Top 1 | 2.08 |  |  |  |  |
| Top 5/1 | 2.08 | 2.03 | 2.47 | 2.60 | 3.22 |

Table 7 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beams and the ideal L1-RSRP of the same beams (SNR=-6dB for inference)

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 2.79 |  |  |  |  |
| Top 5/1 | 2.79 | 2.58 | 2.80 | 3.26 | 3.40 |
| RSRP difference (dB) considering errors case 3 (only add in model input) |  |  |  |  |  |
| Top 1 | 3.14 |  |  |  |  |
| Top 5/1 | 3.14 | 3.09 | 3.03 | 3.10 | 4.18 |

Table 8 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beams and the ideal L1-RSRP of the same beams (SNR=0dB for inference)

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 1.40 |  |  |  |  |
| Top 5/1 | 1.40 | 1.56 | 1.71 | 2.20 | 2.47 |
| RSRP difference (dB) considering errors case 4 (only add in model input) |  |  |  |  |  |
| Top 1 | 1.60 |  |  |  |  |
| Top 5/1 | 1.60 | 1.82 | 2.06 | 2.42 | 2.78 |

Furthermore, based on the objectives in latest WID [3], RAN4 aims to specify necessary core requirements for beam management. For core part, it includes the definition of delay requirements for prediction. Basically, the current measurement period requirement for L1-RSRP can be as the reference and the number of samples sample (i.e., sample =1) can be reused. Specifically, for BM case-2, given that it is the prediction in time dimension, the definition of delay requirements is also relevant to prediction methodology (e.g., Case A and Case B specified in RAN1) and related parameters for measurement and prediction (e.g., number of time instances for measurement/prediction, the time duration for beam prediction). For this case, more discussion is needed on how to define corresponding requirements with consideration of characteristics of time dimension prediction.

Proposal 8: For core requirements for beam management, RAN4 to define 1-sample based delay requirements for beam prediction of Spatial-domain in BM case-1. For Temporal beam prediction in BM case-2, the definition of delay requirements may take into account specific methods (e.g., caseA and caseB) and characteristics of time-domain.

3.2 RF error modeling

RF error is another important factor for measurement error modeling. Here we provide the evaluation results of prediction accuracy and RSRP difference when different RF errors modeling are considered, for which RF errors are modeled as 4dB offset, ±4dB uniform distribution, ±4dB Gaussian distribution, respectively.

Table 9 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction by considering RF errors as 4dB offset, ±4dB uniform distribution, ±4dB Gaussian distribution (SNR=-3dB for inference)

| Simulation results | Prediction accuracy (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | No error | 4dB offset | ±4dB uniform distribution | ±4dB Gaussian distribution |
| Top 1 | 68.72 | 68.08 | 30.41 | 36.63 |
| Top 2/1 | 87.95 | 87.71 | 61.21 | 66.64 |
| Top 3/1 | 91.38 | 91.06 | 74.30 | 80.53 |
| Top 4/1 | 93.38 | 93.46 | 80.29 | 83.96 |
| Top 5/1 | 96.41 | 96.25 | 85.40 | 88.91 |
| Top 1/2 | 87.47 | 87.55 | 54.27 | 61.77 |
| Top 1/3 | 94.01 | 93.93 | 68.56 | 76.14 |
| Top 1/4 | 96.97 | 96.97 | 76.22 | 81.80 |
| Top 1/5 | 98.00 | 98.00 | 81.17 | 86.43 |

Table 10 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1 and Top-5 predicted beams and the ideal L1-RSRP of the same beam (SNR=-3dB for inference)

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 1.46 |  |  |  |  |
| Top 5/1 | 1.46 | 1.71 | 1.91 | 2.36 | 2.70 |
| RSRP difference (dB) considering 4dB offset errors (only add in model input) |  |  |  |  |  |
| Top 1 | 4.51 |  |  |  |  |
| Top 5/1 | 4.51 | 4.68 | 4.85 | 5.00 | 5.05 |
| RSRP difference (dB) considering ±4dB uniform distribution errors (only add in model input) |  |  |  |  |  |
| Top 1 | 7.89 |  |  |  |  |
| Top 5/1 | 7.89 | 7.47 | 7.12 | 6.55 | 6.35 |
| RSRP difference (dB) considering ±4dB gaussian distribution errors (only add in model input) |  |  |  |  |  |
| Top 1 | 5.88 |  |  |  |  |
| Top 5/1 | 5.88 | 5.77 | 4.96 | 5.35 | 5.72 |

From Table 9, it can be observed that the prediction accuracy is not impacted by the 4dB RF offset. The possible reason is that the trend of all the datasets remains unchanged when a 4dB offset is added to all the RSRP elements. Furthermore, based on the simulation result based on the error modeled by uniform distribution and Gaussian distribution, the uniform distribution error brings more degradation to prediction accuracy compared with the 4dB offset value as the errors.

From Table 10, when different RF error modeling is considered, there are some noticeable performance differences in the RSRP difference. Among them, the error modeled by uniform distribution has the greatest impact on the accuracy of RSRP prediction.

Based on the initial evaluation we provided above, there is some impact on beam prediction and predicted RSRP value due to RF error. From this perspective, we believe the modeling method for RF error also needs to be involved in the study.

Observation 9: There is some impact on beam prediction and predicted RSRP value due to RF error.

Proposal 9: For the study on the impact of measurement error, the method of RF error modeling needs to be involved in RAN4 discussion, e.g.,

- An offset value in the original dataset;

- Truncated Uniform distribution;

- Truncated Gaussian distribution.

- Summary

In this contribution, we further provided our views related issues on beam management. Based on the above analysis, the following proposals are presented.

Proposal 1: RAN4 to define the latency requirements on entire prediction process for BM, the following can be the baseline:

- The latency starts from the reception of radio signals by UE and ends at the reporting of prediction results.

- The delay components at least include:

  - Measurement delay for prediction: the time required to collect input data (measurements of set B) for inference

  - Inference delay: the time for using a trained AI/ML model to produce BM prediction results

  - Delay uncertainty for reporting: the delay uncertainty in acquiring the first available reporting occasion

Proposal 2: For whether Rx beam is assumed as known in case of DL Tx beam prediction, RAN4 to introduce UE capability of supporting DL Tx beam prediction with knowledge of paired Rx beam

Proposal 3: For performance monitoring for BM, RAN4 to first determine the testing goal. The following alternatives can be considered:

- Option 1: The testing goal is to verify whether the performance metrics can be correctly calculated by UE;

- Option 2: The testing goal is to verify whether UE can perform monitoring and report the performance metrics within a certain latency upon performance monitoring is required;

- Option 3: The combination of Option 1 and Option 2.

Observation 1: There is a certain probability that the Top-K predicted beam is not any of the Top-K beams based on ground-truth RSRP.

Observation 2: For RSRP Absolute accuracy, if the index i can be any beam index in top-K beams based on ground-truth L1-RSRP, the RSRP accuracy for those beams which belong to Top-K predicted beam but not any of the Top-K beams based on ground-truth RSRP cannot be guaranteed.

Proposal 4: The absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i can be any beam index in top-K beams based on predicted L1-RSRP.

Proposal 5: RAN4 to confirm to use beam prediction accuracy as one of metrics/KPIs for defining requirements on BM.

Observation 3: There is a non-negligible performance degradation in beam prediction accuracy when considering measurement error modeling for inference.

Observation 4: With the decrease of SNR, the beam prediction accuracy will also decrease. However, when the SNR reaches a certain level (e.g., 0dB), this performance decline will stabilize.

Observation 5: Compared with Truncated Gaussian distribution, the error derived from LLS-based modeling has a greater impact on the decline of beam prediction accuracy performance under similar SNR levels.

Proposal 6: RAN4 to define beam prediction accuracy requirements with consideration of the impact on measurement error.

Observation 6: In addition to the rate of correct beam prediction, when additional 1dB RSRP margin is considered (e.g., For the success rate of the difference between the ideal RSRP of ideal Top-1 beam and the predicted RSRP of predicted Top-K beam are within the X=1dB range), it will bring a noticeable improvement in the rate of prediction accuracy.

Observation 7: The probability of predicted RSRP of predicted Top-K beam falling into the range of [the RSRP of Top-1 beam± XdB] has significant improvement with the increase of X value.

Observation 8: When using predicted RSRP of predicted Top-1 beam and Ideal RSRP of ideal Top-1 beam as the benchmark, the probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the [X]dB range are similar.

Proposal 7: RAN4 to use Top-K/1 as the metric of beam prediction accuracy for further performance evaluation.

Proposal 8: For core requirements for beam management, RAN4 to define 1-sample based delay requirements for beam prediction of Spatial-domain in BM case-1. For Temporal beam prediction in BM case-2, the definition of delay requirements may take into account specific methods (e.g., caseA and caseB) and characteristics of time-domain.

Observation 9: There is some impact on beam prediction and predicted RSRP value due to RF error.

Proposal 9: For the study on the impact of measurement error, the method of RF error modeling needs to be involved in RAN4 discussion, e.g.,

- An offset value in the original dataset;

- Truncated Uniform distribution;

- Truncated Gaussian distribution.

- References

- R4-2414323, WF on AI/ML, Qualcomm

- R4-2417211, Simulation assumptions for beam prediction, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- Appendix

6.1 Simulation assumptions

Table 11 LLS simulations assumption for the evaluation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz. |
| Subcarrier spacing | 120kHz |
| Channel model | NLOS channel: CDL-C DS = 100ns |
| BS antenna configurations | One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline. Number of Tx beams is 32 |
| BS antenna element radiation pattern | Table 2 in [2] |
| BS antenna height and antenna array down-tilt angle, if needed (Option 1 above) | 25m, 110° |
| Beamforming characteristic of the BS pattern | Azimuth Angle in degree for Tx beams: [-78.75 -56.25 -33.75 -11.25 11.25 33.75 56.25 78.75] Zenith Angle in degree for Tx beams: [84 90 94 102] The power pattern of 32 Tx beams in azimuth cut and elevation cut are shown in Fig. 1 and Fig. 2. Note: Elevation angle in degree = Zenith angle in degree-90° |
| UE antenna configurations | Panel structure: (M, N, P) = (1, 4, 2), - 2 panels (left, right) with (Mg, Ng) = (1, 2) Azimuth Angle in degree for Rx beams for single panel: [-67.5 -22.5 22.5 67.5] Zenith Angle in degree for Rx beams for single panel: 45 |
| UE antenna element radiation pattern | Table 3 in [2] |
| UE moving speed | 3km/h |
| Reference signal | CSI-RS |
| DRX | No |
| Number of samples in L1 averaging | 1 |

Fig. 1 Azimuth cut of BS 32 Tx beams power pattern

Fig. 2 Elevation cut of BS 32 Tx beams power pattern

6.2 AI/ML model

FC model used in simulation is shown in Fig. 3:

Fig. 3 AI fully connected model description