3GPP TSG RAN WG4 Meeting #113	R4-2418757

Orlando, Florida, USA, 18th – 22nd November, 2024

Agenda item:			7.15.2.1

Source:	vivo

Title:	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor.

- Discussion

- 2.1 L3 measurement delay reduction by optimizing Rx BSF for UE supporting multiple-Rx simultaneous reception

In the last meeting, following issues are captured in the WF

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF:

Agreement in online session: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

In the previous meetings, some companies propose to consider criteria/condition(s) of the UE locations and/or UE mobility to activate and de-activate the L3 FBS measurement. We agree the approaches and RAN4 could use the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD as a baseline. For example, the relaxed measurement criterion could be used for high/low mobility to (de-) activate FBS. For UE locations, RAN4 could use the RSRP value or some positioning method to judge whether it is in cell-center or in cell-edge.

Relaxed measurement criterion for good serving cell quality could also be considered for L3 FBS, i.e. the threshold Qin+XdB. Qin is specified in TS 38.133 and X is an offset configured by RRC signalling defined in TS 38.331. For L3 FBS, RAN4 could consider Qin+an X offset configured by RRC signalling as the activated criterion/threshold, where X can be old or new values.

Proposal 1: For scenario 1/2/3/4, RAN4 to consider the Qin+XdB as the criterion, where X is configured by RRC signalling, to evaluate the serving cell quality for activating/de-activating the L3 BSF.

FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier

For this issue, we support opt 1 and since 2 searchers are occupied by single carrier, it may have impacts to other CCs. For option 2 that two searchers are occupied by single carrier, extend to CA scenario would be difficult.

Proposal 2: RAN4 only to consider the condition that 1 searcher is assumed for single carrier.

FFS: SSB processing delay/time for processing multiple beams received in a SMTC

SSB processing delay/time for processing multiple beams received in a SMTC. Since the interval between two SMTC is long enough for buffering and processing, there is no need to consider such issue for L3 measurement delay reduction in every measurement. However, an additional processing delay may be added in the last measurement sample for defining reporting delay requirement.

Proposal 3: No need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay in the last sample for defining reporting delay requirement.

Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

In the RAN4#112 meeting, following scenarios were at least agreed to use the L3 measurement delay reduction by optimizing Rx BSF.

- SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Handover

- RRC Re-establishment/RRC Connection Release with Redirection

Take SSB based Intra-frequency measurement without MG as an example, in the current spec, beam sweeping factors related value M are corresponding to different power class in measurement delay requirement.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps = 36. Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36. MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72 samples. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48 samples. For a UE supporting FR2 power class 3, MSSB_index_intra = 48 samples.

It can be observed that the M value for FR2-1 in PC 3 is 24. In fact, the M value is a compromise value, which includes the BSF value and the measurement samples (e.g., M = BSF*sample). We can not derive the real value of BSF from the M since the sample value is unknow. If sample=4, the BSF value =6 and if sample=3, BSF value=8. Some company mentioned that Rx BSF can be reduced to 2, 4 or 6 or use Rel-18 BSF value as baseline

| 30. NR_FR2_multiRX_DL | [30-2] | Fast beam sweeping for layer-1 measurement | Supports beam sweeping factor reduction for SSB-based layer-1 measurement. | Per band | FR2-1 only | Candidate values for Component 2: {2,4,6} for FR2-1 Note: It is only supported for power class 3. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- |

Reuse {2,4,6} is a feasible way but may not applicable to L3 measurement. Based on the above analysis, we have the following approaches to consider this issue.

- Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc.

- Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc.

- Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4.

Proposal 5: RAN4 to consider the following approaches as solutions:

- Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc.

- Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc.

- Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4.

- Summary

In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. Based on above analysis, following proposals are present.

Proposal 1: For scenario 1/2/3/4, RAN4 to consider the Qin+XdB as the criterion, where X is configure by RRC signalling, to evaluate the serving cell quality for activating/de-activating the L3 BSF.

Proposal 2: RAN4 only to consider the condition that two searchers are occupied by single carrier.

Proposal 3: No need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay in the last sample for defining reporting delay requirement.

Proposal 4: RAN4 to consider additional conditions for triggering FBS for L3 measurement for power consumption issue, including active condition and de-active conditions.

Proposal 5: RAN4 to consider the following approaches as solutions:

- Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc.

- Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc.

- Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4.

- References

- R4-242380 Revised WID: NR Radio Resource Management (RRM) Phase 5

- R4-2416853 WF on RRM requirements for NR_RRM_Ph5_Part1