3GPP TSG RAN WG4 Meeting #113	R4-2418759

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.25.2

Source:	vivo

Title:	Further Discussion on RRM impacts for XR and reply LSs

Document for:	Discussion

# Introduction

In the RAN4#112bis meeting, there were further discussions on RRM requirements impact for XR. Issues related Following agreements were made and captured in the WF [1].

## Issue 1-1: Deployment scenarios < Agreement>: - Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation <Way forward>: - Further discussion needed for - FR2-2 - NR-DC, EN-DC, NE-DC

RAN1 had discussed UE assistance information related to measurement occasions for several meeting. In the RAN1#117 meeting, conclusions were made and an LS [2] was sent to RAN4 to trigger further discussions in RAN4. In addition, RAN1 concluded solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions as working assumption and an LS [3] was sent to RAN4 for confirmation.

In [2], we provided our views on the RRM requirements impact and issues raised in the RAN1 LSs. In this contribution, we further provide our views on RRM requirements impact for XR in Rel-19 and on questions raised in the two RAN1 LSs.

# RRM requirements impact

## 2.1 gaps/restrictions

RRM measurements can be categorized into L3 measurements and L1 measurements. L3 measurements can be further grouped as measurements within gaps and outside gaps. The scheduling restrictions are different for the three types of measurements.

L3 measurements within gaps

The L3 measurements within gaps are performed when measurement gaps are configured. During the measurement gaps, the UE is not required to conduct reception/transmission from/to the corresponding serving cells. The entire measurement gap is used for measurement only by receiving reference signals such as SSB, PRS.

L3 measurements outside gaps

The L3 measurements outside gaps are performed on SMTC occasions outside of measurement gaps if they are not fully overlapped. The scheduling restrictions are summarized as below.

- For UE which do not support simultaneousRxDataSSB-DiffNumerology for SSB with different subcarrier spacing than PDSCH/PDCCH on FR1 - The UE is not expected for transmission/reception on SSB symbols configured to be measured, and on 1 data symbol before and after each consecutive SSB symbols configured to be measured - intra-band carrier aggregation: apply to all other serving cells in the same band - intra-band non-contiguous carrier aggregation configured for a UE indicating intraBandNR-CA-non-collocated-r18: scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band if nonCollocatedTypeNR-CA-r18 is provided. - TDD bands on FR1 - The UE is not expected to transmit on SSB symbols configured to be measured, and on 1 data symbol before and after each consecutive SSB symbols configured to be measured - When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells - When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18, apply to all other serving cells in the same band if nonCollocatedTypeNR-CA-r18 is provided - When TDD inter-band carrier aggregation is performed, apply to another serving cell in a different band if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair. - FR2 - The UE is not expected for transmission/reception on restricted symbols, on SSB symbols configured to be measured, and on 1 data symbol before and after each consecutive SSB symbols configured to be measured - intra-band carrier aggregation: apply to all other serving cells in the same band - inter-band carrier aggregation: for UE supporting IBM, similar restrictions as TDD bands on FR1

L1 measurements

For L1 measurements, it has similar restrictions as L3 measurement outside gaps but the restrictions are only on reference signal symbols for L1 measurements. It is summarized as below.

- FR1 - For UE which do not support simultaneousRxDataSSB-DiffNumerology SSB with different subcarrier spacing than PDSCH/PDCCH on FR1, - The UE is not expected for transmission/reception on symbols corresponding to the SSB indexes configured for L1-RSRP measurement - intra-band carrier aggregation: apply to all other serving cells in the same band - intra-band non-contiguous carrier aggregation configured for a UE indicating intraBandNR-CA-non-collocated-r18: scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band if nonCollocatedTypeNR-CA-r18 is provided. - FR2 - The UE is not expected for transmission/reception symbols corresponding to the SSB indexes and/or periodic/semi-persistent/aperiodic CSI-RS resources which is NOT QCLed with active TCI state for PDCCH/PDSCH configured for L1 measurement - intra-band carrier aggregation: apply to all serving cells in the band on the symbols that fully or partially overlap with restricted symbols.

Moreover, L1 measurements including RLM/BFD/CDB/L1-RSRP/L1-SINR measurements. The reference signals for different L1 measurements may or may not be the same, or may or may not be overlapped.

It is observed that the UE cannot be scheduled during the measurement gaps. For L3 measurements outside gaps when certain conditions are met, the UE cannot be scheduled on some or all of the symbols within SMTC. For L1 measurements when certain conditions are met, the UE cannot be scheduled only on the reference signal symbols.

Observation 1: Scheduling restrictions are different in terms of conditions and restricted symbols for L3 measurements within gaps, L3 measurements outside gaps and L1 measurements.

## 2.2 Scope

There were initial discussions on deployment scenarios and type of measurements to be considered in the WI for gap skipping.

## Issue 1-1: Deployment scenarios < Agreement>: - Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation <Way forward>: - Further discussion needed for - FR2-2 - NR-DC, EN-DC, NE-DC ## Issue 1-2: Types of measurements to be considered <Way forward>: - Option 1: Agree at least on the following and further discuss other scenario - Intra-frequency - Inter-Frequency - LTE inter-RAT - Option 2: Agree at least on the following and further discuss other scenario - Intra-frequency - Inter-Frequency - LTE inter-RAT - XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC - Option 3: include all related scenarios

Regarding the DC scenarios, i.e., EN-DC, NR-DC, NE-DC, it needs further considerations whether it should be supported. For example, in EN-DC, the per-UE gap or FR1 per-FR gap is configured by MN. The measurement configuration information is exchanged between the MN and the SN.

In EN-DC and NGEN-DC, the measurement gap configuration from the MN to the UE indicates if the configuration from the MN is a per-UE gap or an FR1 gap configuration. The MN also indicates the configured per-UE or FR1 measurement gap pattern and the gap purpose (per-UE or per-FR1) to the SN. Measurement gap configuration assistance information can be exchanged between the MN and the SN.

However, if a gap/restriction occasion is cancelled by MN via DCI, it may not be able to inform SN timely. At least, There are no RAN2 discussions on this so far. It means the SN cannot schedule the UE during the gap occasion being cancelled. Thus, benefit to support the feature is compromised in DC scenarios.

From RAN4 perspective, we may not define requirements for DC scenarios, or we send an LS to RAN2/RAN1 to confirm whether it is supported or not.

Proposal 1: Send an LS to RAN2 and RAN1 to ask if DC scenarios are supported for gap cancellation from their perspective.

Regarding the type of measurements to be considered, the only supported 3G inter-RAT measurement is UTRAN FDD for SRVCC when there UE is not in EN-DC scenarios. We don’t see any reason to preclude the measurements when gap skipping is enabled. In EN-DC, measurements on 2G/3G can be configured by E-UTRA PCell. If there is no strong motivation to preclude 2G/3G measurements, all measurements can be considered. Nevertheless, whether gap skipping is supported for MR-DC is separate discussion.

Proposal 2: All intra-frequency, inter-Frequency, LTE inter-RAT and UTRAN FDD measurements are supported for gap skipping in NR SA and carrier aggregation.

L3 measurements within gaps

L3 measurement within gaps will be supported in this WI. However, there are many different measurement gaps being introduced in multiple releases, e.g., The per-UE gap and per-FR gap in Rel-15, pre-configured measurement gap, concurrent measurement gaps, NCSG and MUSIM gaps in Rel-17. Combination of concurrent gap and pre-MG or NCSG was further enhanced in Rel-18. RAN4 needs to decide what type of measurement gaps are considered in this WI. It would have a lot of requirements to be defined if all the gap types are considered.

## Issue 2-1: General approach for defining requirements for measurements with gaps <Way forward>: - Option 1: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type. - Option 2: As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS. ## Issue 2-2: Types of measurement gaps <Way forward>: - In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG. - Companies may bring proposals regarding the priorities of each gap type for RAN4 to work based on the following table Cap types; First prio; Second prio; Don’t include. Type 1. Type 2. NCSG. MUSIM. Positioning. Rel 18 concurrent gap. PreConfigured.

In the previous meetings, there were discussions on the type of measurement gaps to be considered, including Type 1 MG, Type 2 MG, NCSG, MUSIM gaps, positioning gaps, concurrent gaps and pre-configured gaps. Views from companies are captured in the WF [2].

It is noted that Type 1 MG and Type 2 MG are defined as below.

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17

Type 1 MG is the baseline from Rel-15 and it should be supported. It includes both per-UE gap and per-FR gap configured via GapConfig.

The Type-2 MG, however, is not so clear. Whether it is a single gap configured via GapConfig-r17 or it can be multiple gaps. If it is the latter one, then what would be the difference between type-2 MGs and concurrent gaps. If it is the former one, then it should be supported as there is no difference compared to Type-1 MG in terms of gap skipping.

Concurrent gaps are quite complicated. Moreover, whether concurrent gaps with NCSG/pre-configured gaps are included or not should be clarified. It may not be necessary that NW configures concurrent gaps when XR service is enabled. It is preferred not to consider concurrent gaps in Rel-19.

For NCSG, in general there would be no difference compare to Type 1 MG or Type 2 MG from scheme of triggering gap skipping perspective. However, the interruption ratio due to VIL is relatively small, which is 2ms in FR1 and 1.5ms in FR2 within one visible interruption repetition period. In typical measurement gap configuration, 6ms MGL with 40/80ms MGRP would be configured for the UE. The interruption rate due to measurement gap is up to 15%. If 20ms MGRP is considered in FR2, the interruption rate is up to 30%. However, for NSCG the interruption rate can only be up to 7.5% for 20ms MGRP in FR2. Considering the possibility of collision between XR traffic and NCSG, the impact to XR service is very small. Thus, there is no need to consider NCSG when gaps are to be skipped.

Pre-configured measurement gaps are similar to Type-2 MG when it is activated which can be triggered by UE autonomous control or network configuration. When it is not activated, it is the same as no measurement gaps are configured. Thus, it can be considered.

Positioning measurement gaps are not clear by itself. In Rel-16, legacy per-UE gap is used for PRS measurements. When RRM measurements also need gaps, then gap sharing between RRM and PRS measurements is used. In Rel-17, per-FR gap can also be used for PRS measurements. Additionally, pre-configured MG by PosGapConfig-r17 is also supported for PRS measurement which can be activated via DL MAC CE or UL MAC CE depends on UE capability. If positioning measurements are based on legacy measurement gap, then it would be naturally be considered. For Rel-17 pre-configured MG for positioning, it is proposed not to consider it.

MUSIM gaps is for operations in a different network which can minimize impact under current network operation. If network believes XR service is more import it can reject to allocate MUSIM gaps when it is requested by UE. Similarly, if a device believes XR service is more import, it may not request MUSIM gaps at all. There is no need to specify complicated solutions to skip MUSIM gaps.

Observation 2: Type 1 MG and Type 2 MG, including per-UE gap and per-FR gap, and pre-configured MG can be considered in XR WI in Rel-19, where Type 2 MG needs further clarification.

From procedure wise, it is more straightforward and feasible to specify requirements for Type 1 MG as baseline. Other type of MGs can be FFS if there is time available in later phase.

Proposal 3: As baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

L3 measurements outside gaps

There were extensive discussions on whether L3 measurements outside gaps should be supported for gap skipping.

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions <Way forward>: - Option 1: Support skipping of measurements without gaps - Option 2: Measurement skipping does not apply for measurements without gaps

L3 measurements outside gaps could be intra-frequency measurements, inter-frequency measurements and inter-RAT measurements. As analyzed in section 3.2, there could be some SMTC occasions for a MO within a time period are not used for measurements depending on measurement configuration. These SMTC occasions can be used for data transmission/reception. Similar to gaps, the SMTC occasions not used for measurements in the first place may not be enough for XR traffic. Additional SMTC occasions may be indicated by NW for data transmission/reception.

On the other hand, in FR1 the scheduling restrictions for L3 measurements outside gaps is only for UE which do not support simultaneousRxDataSSB-DiffNumerology. In general, mixed numerology of data and SSB is not a typical deployment. Moreover, it should be no extra complexity for UE to support this UE capability. Thus, it is better choice for XR device to support the UE capability simultaneousRxDataSSB-DiffNumerology so that it can be scheduled without restriction under mixed numerology deployment. In FR2 the scheduling restrictions for L3 measurements outside gaps is due to Rx beam sweeping. It cannot be avoided during measurements. Intra-frequency measurement is very important for UE mobility and long measurement delay in FR2 has been a concern that there is dedicated WI to reduce beam sweeping factor. If the SMTC occasions are skipped then longer measurement delay is expected. Especially when any SMTC occasions overlapped with XR data is skipped then mobility performance could be highly degraded.

Proposal 4: Measurement skipping does not apply for L3 measurements without gaps.

L1 measurements

For L1 measurements, it also needs to be discussed whether this should be supported.

## Issue 1-4: L1 measurements with scheduling restrictions and procedures <Way forward>: - Option 1: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 2: Measurement skipping does not apply for L1 measurements and procedures

L1 measurements include RLM, BFD, CBD or L1-RSRP/L1-SINR for beam reporting. L1 measurements are performed outside gaps. Similar to L3 measurements outside gaps, in FR1, the scheduling restriction is for UE which do not support simultaneousRxDataSSB-DiffNumerology. In FR2, it is due to Rx beam sweeping for SSB based measurements. However, for CSI-RS based L1 measurements there is no scheduling restriction when certain conditions are met.

The reference signals for L1 measurements includes SSB and/or CSI-RS, which may or may not be fully overlapped with SMTC occasions outside measurement gaps. If it is not fully overlapped with SMTC occasions, the measurement is performed on the reference signal symbols not overlapped with SMTC occasions. Otherwise, in FR2 it needs to share SMTC occasions for L3 measurements outside gaps. Since scheduling restriction for L1 measurements are only on L1-RS symbols which would not be many, the benefit of enabling data transmission/reception on the symbols is very small. Furthermore, it could have big impact on L1 measurement performance, especially for RLM/BFD measurements which is essential for link quality monitoring.

Therefore, it is not preferred to enable L1-RS symbols for data transmission/reception when there are scheduling restrictions.

Proposal 5: RAN4 does not consider enabling data transmission/reception on L1-RS symbols that need to be used for L1 measurements if there is scheduling restriction.

There were also proposals for L1 measurements for LTM.

## Issue 1-5: L1/L2 triggered mobility measurements and procedures <Way forward>: - Option 1: support skipping of L1 measurements and procedures for LTM within and outside gap - Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM

L1 measurement for LTM within gap is sharing the gap for L3 measurements. When a gap occasion is skipped, the L1 measurements for LTM may also be impacted. Moreover, only Type 1 measurement gap is support for LTM. Thus, it would be no problem to support L1 measurements for LTM within gaps.

As for L1 measurements for LTM outside gaps, it does not need to be considered similar to L3 measurements outside gaps and L1 measurements for serving cells.

Proposal 6: Only L1 measurements for LTM within gaps are supported for gap skipping.

## 2.3 Impact to requirements

There were extensive discussions on the impact to requirements due to gap skipping and how the requirements are to be specified. The following issues were captured.

## Issue 3-1: How to capture impact of measurement skipping <Way forward>: - Option 1: - As minimum requirements, cell detection, index reading, and measurement period is extended. - FFS how to extend - Reuse existing measurement accuracy requirements. - Option 2: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - Reuse existing measurement accuracy requirements. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information.

The most important part is whether both the cases are supported, i.e., measurement impact is expected and measurement impact is not expected due to gap skipping.

L3 measurements within gaps

For the example of measurement configuration depicted in Fig 1, measurement gap is configured together with one inter-frequency layer. For this configuration, CCSFwithin_gap,i is 1 for the frequency layer. The UE needs all of the measurement gaps to meet the requirements.

Fig 1. Configuration of measurements on one inter-frequency layer with gaps

In this case, when some of the gaps are used for data scheduling the UE cannot meeting existing measurement requirements. UE needs additional gap occasions to get enough samples to meet the accuracy requirements. Since this is a very simple case, measurement delay can just be extended by number of skipped gap occasions.

For the example of measurement configuration depicted in Fig 2, measurement gap is configured together with three inter-frequency layers where different SMTC periodicity is used. For this configuration, CCSFwithin_gap,i is 3 for all the frequency layers.

Fig 2. Configuration of measurements on multiple inter-frequency layers with gaps

In this case, some of the gaps may not always be used to meet minimum measurement period requirements. The requirements are assumed that there are always 3 candidates of MOs to be measured within a gap. For MO#1 and MO#2, it is not the case. So, some of the gaps could be left unused as long as measurement period requirements are satisfied.

Suppose 2 samples are needed for one measurement period. one of the possible measurement implementations is depicted in Fig 3. Another possible measurement implementation is depicted in Fig 4.

Fig 3. Measurement implementation example 1

Fig 4. Measurement implementation example 2

Based on above analysis, it is observed that some of the gaps may not be used for measurements and it is up to UE implementation which gaps are not used for measurements.

Observation 3: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied.

Observation 4: It is up to UE implementation which gaps are not used for measurements under current measurement configuration.

In the case as depicted in Fig 2, Fig 3 and Fig 4, the unused gaps are about 37.5% (9/24). It is significant gain if these gaps can be used for data transmission/reception. Since it is up to UE implementation which gaps are not used, the information can only be provided by UE. Therefore, there is obvious benefit if UE can provide assistance information about which gaps are not used for measurements.

Observation 5: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance.

Moreover, for some advanced UE, it may not use all the gaps during the measurement period specified by minimum requirements. For example, measurement period is based on 8 samples for FR1 inter-frequency measurements. The advance UE may just use 5 of them. So, the UE either not to use some of the gaps or use all the gaps with shorter measurement period. If the former case, then the gaps not used for measurement can be reported to network to schedule the UE during the gaps. Even for the example in Fig 1, there could be unused gaps due to advanced UE implementation.

If there are enough unused gaps and it is used for data scheduling there is no impact to measurements. On the other hand, depending on measurement configuration, there may be not enough unused gaps in which case there is impact to measurements. Therefore, both of the two cases should be supported.

Proposal 7: Both of the two cases, i.e., there is impact to measurements and no impact to measurements due to gap skipping, are supported.

For the case where there is no impact to measurements, UE assistance information is needed. Since it is up to UE implementation which gaps are not used, the information can only be provided by UE. The patterns of gaps/SMTC occasions not used for measurement within a time period should be provided.

Proposal 8: The patterns of gaps occasions not used for measurement within a time period should be provided as UE assistance information.

Then it comes the question that the reported gaps not used for measurement may not be enough for data transmission/reception considering QoS requirements. In some cases, there may be no gaps not used for measurement depending on measurement configuration. However, when enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, it needs to keep impact to mobility performance limited. In order to achieve this goal, it needs UE to provide information of gaps not used for measurement. If it is totally up to NW indication, the mobility performance could highly be degraded as gaps may have different priority for measurements, e.g., the most loaded gap has highest priority and it may be used to measure sparse reference signals and should not be used for data transmission/reception, or if gap sharing scheme is not equal sharing, some gaps may have high priority to meet requirements.

One possible solution is that NW indicates the percentage of gaps not used for measurement during a period of time. It is up to UE to decide which gaps are not used for measurement during the period of time. This could be the best balance between QoS requirements of data transmission/reception and UE mobility performance.

Proposal 9: NW may indicate the percentage of measurement gaps not used for measurement during a time period for UE to provide information about gaps not used for measurement.

If both of the cases are supported, network needs to know whether measurements are impacted or not. This can also be indicated by UE because only UE knows whether it can meet existing requirements based on measurement configurations and UE implementation.

Proposal 10: UE to indicate whether it can meet L3 measurements requirements when unused gaps reported by UE is used for data scheduling.

When DRX is configured, the UE may measure once within gaps in one DRX cycle. Only the gaps occasions overlapped with DRX on duration can be used data transmission/reception. There could be different reporting scheme for UE assistance information, e.g., only reporting gaps overlapped with DRX on duration within a time period.

Proposal 11: RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured.

Regardless of the gaps not used for measurements is due to measurement configuration or advanced UE implementation, it is more like UE capability rather than UE assistance information. It is better that RAN4 to discuss which scheme should be used. The difference is that NW will follow UE reporting if it is UE capability, but the NW may not follow UE reporting if it is UE assistance information depending on current specification. At least for the case where measurements is not impacted, it is better to report as UE capability similar to “NeedForGaps”.

Proposal 12: RAN4 to discuss the information about gaps not used for measurement are reported as UE capability or UE assistance information.

## Issue 3-2: Principles for measurement delay calculation <Way forward>: - Option 1: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay. - Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period. Other options are not precluded

For the case where there is no impact to measurements, the UE should meet existing measurement requirements.

For the case where there is impact to measurements, the UE cannot meet existing requirements. Based on RAN1 working assumption, dynamic indication by DCI to enable Tx/Rx in particular gap/restriction is supported. If there is no restriction on the dynamic indication, then it would be difficult, if possible, to define new requirements because it is unknown what percentage of gaps and/or which gaps are skipped. The RRM measurement requirements are based on gap periodicity and gap sharing scheme. In multi-frequency layers scenario, frequency layers would have different ‘priority’. For example, for a frequency layer configured with 160ms SMTC periodicity may have higher priority than other frequency layers with shorter SMTC periodicity when competing a measurement gap. If those gaps for the frequency layer are all indicated by network for data transmission/reception, there is no chance for the UE to measure the frequency layer. Thus, UE mobility performance will highly be degraded.

Thus, it is expected that some restrictions should be put on the dynamic indication. For example, the gaps to be skipped during a time period is indicated by RRC in advance. The DCI indicated gaps are only selected from those configured by RRC. This will largely simplify UE measurement scheduler which only needs to be conducted after RRC reconfiguration and ensure mobility performance is not degraded too much.

Other potential restriction(s) that can help to achieve good trade-off between mobility performance and data delivery delay can also be considered. From requirements perspective, it is reasonable to use NR-U like approach to extend the delay by the number of measurement gaps skipped during the measurement period. By limiting the max number of gaps skipped during the measurement period, the mobility performance can be ensured to some extent.

Proposal 13: When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements.

Proposal 14: When there is impact to measurements due to gap skipping, RRM measurement requirements are defined by using NR-U like approach with carefully deciding the max number of gaps skipped during the measurement period.

Proposal 15: RAN4 to study possible restrictions on dynamic indication by DCI, if it is confirmed, to achieve good trade-off between data deliver delay and mobility performance.

L3 measurements outside gaps

L3 measurements outside gaps could also be intra-frequency measurements, inter-frequency measurements and inter-RAT measurements. Of course, this part depends on the agreements whether skipping SMTC occasions for L3 measurements without gaps is supported.

If it is supported, similar conclusion as for L3 measurements within gaps can be reused.

# UE assistance information

RAN1 had discussed UE assistance information related to measurement occasions for several meeting. In the RAN1#117 meeting, conclusions were made and an LS [3] were sent to RAN4 to trigger further discussions in RAN4.

Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.

In section 2.3, views on necessity of UE assistance information are provided. It is focused on the L3 measurements within gaps. For L3 measurement outside gaps, similar conclusion can be made and it is also depending on the discussions whether it is supported or not. Detailed analysis was provided in [4].

There serval aspects about UE assistance information had been discussed in RAN1.

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable;

As discussed in section 2.3, the patterns of gaps/SMTC occasions not used for measurement within a time period should be provided to achieve best balance between QoS requirements and mobility performance.

Other options will have unknown and unexpected impact on UE mobility performance and it provides no information on which gaps have higher priority, e.g., most loaded gaps or gaps for sparse reference signals.

- FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig);

s-MeasureConfig is the threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. When serving cell RSRP is above the threshold, UE may not perform measurements on neighbour cells. In this case, measurement gaps, if configured, may not be used for measurement at all.

However, UE still needs to perform measurements on SMTC occasions for serving cell, which are also the same for all neighbour cells on the serving cell frequency, if the measurements are performed outside gap. The channel condition of serving cell RSRP does not help UE to provide information about which SMTC occasions within a time period are not used for measurements.

Moreover, when serving cell RSRP is below the threshold, UE needs to measure neighbour cells where measurement gaps could be used. The information about gaps not used for measurement should still be provided.

Observation 6: Channel condition, e.g., RSRP, does not provide information about which SMTC occasions are not used for measurements. Moreover, additional information about gaps/SMTCs not used for measurement is still needed.

- FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance);

The UE assistance information related to traffic is not RAN4 expertise. It would be better to be discussed in RAN2.

- FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not

There is no L3 measurement requirements related to UE mobility. In connected mode, requirements for UE fulfilling relaxed measurement criteria are specified for RLM and BFD measurements. The relaxed measurement criteria include good serving cell quality and low mobility. There is no L3 measurement requirements related to UE mobility. Thus, UE behaviour on L3 measurement is not changed based on different UE mobility status. If UE mobility is to be used as additional UE assistance information, relaxed L3 measurement requirements should be specified accordingly.

Observation 7: UE mobility information does not help how gaps/SMTC occasions are used for measurement unless relaxed L3 measurement requirements are specified.

# Solutions for enabling Tx/Rx in gaps/restrictions

In RAN1, the discussions had been continued for several meetings on solutions to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. In the RAN1#118 meeting, conclusions were made and an LS [4] were sent to RAN4 to confirm the working assumptions.

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

In the RAN1#118bis meeting, further agreements were made on indication by DCI to skip a particular gap/restriction.

Agreement At least for self-scheduling case, for explicit indication by DCI to skip a particular gap/restriction, one bit indication is included as part of DCI formats 0_1/1_1 and 0_2/1_2. - FFS: Cross carrier scheduling. - FFS: DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. Proposal 2.1.2-v3 For explicit indication by DCI to skip a particular gap/restriction: - Bit equal to 1 indicates the corresponding gap/restriction occasion is to be “skipped”. - Option 1: Once a gap/restriction is indicated to be “skipped” by a DCI, UE does not expect the gap/restriction to be indicated as ‘not skipped’ by later DCI. - Bit equal to 0 indicates the corresponding gap/restriction occasion is to be “not skipped”. Once a gap/restriction is indicated to be “not skipped” by a DCI, the gap/restriction can be indicated to be “skipped” by later DCI. Agreement Clarify the note from previous agreement as follows: - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. Agreement In case of cross carrier scheduling, for explicit indication by DCI to skip a particular gap/restriction, one bit indication included as a part of DCI formats 0_1/1_1 and 0_2/1_2 corresponds to a scheduled cell.

## 4.1 Indication solution

Based on the agreements so far on triggering gap skipping by network signaling, there are two aspects that need RAN4 consideration.

One aspect is that RAN1 designed a uniform triggering mechanism for both gap skipping and restriction skipping. However, from RAN4 perspective, it is under discussion whether skipping restrictions for L3 measurement outside gap and/or for L1 measurement is supported or not. It needs to inform RAN1 of the agreements to be made so that RAN1 may made changes on the design accordingly.

Proposal 16: RAN4 is to inform RAN1 on the conclusion of whether L3 measurements outside gap and L1 measurements for restriction skipping are supported or not.

Furthermore, it unified mechanism is used in the end there are some details to be further considered. For example, there could be partial overlapping of SMTC occasion for L3 measurements outside gap and reference signals for L1 measurements. If there is overlap, it is not clear the indication of skipping the restriction is refer to L3 measurements or L1 measurements. The current design is to use a single bit in DCI to indicate the first gap/restriction occasion.

- The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Thus, when there is overlapping between restrictions for L3 measurements and L1 measurements, it is not clear which restrictions should be skipped.

Observation 8: It is not clear which restrictions should be skipped if there is overlapping of reference signals for L1 measurements and SMTC occasions for L3 measurements outside gap.

Another aspect is that the skipped gap is corresponds to a cell.

- In case of cross carrier scheduling, for explicit indication by DCI to skip a particular gap/restriction, one bit indication included as a part of DCI formats 0_1/1_1 and 0_2/1_2 corresponds to a scheduled cell.

The measurement gap is either per UE or per-FR. All the serving cells can be scheduled if a per UE gap is skipped. All the serving cells in the frequency range same as the per-FR gap can be scheduled if the per-FR gap is skipped. There is no need to link the gap skipping to a certain serving cell. It seems that in each DCI to schedule different serving cells the one bit indication should be included. It will at least complicate UE implementation and is not efficient.

Moreover, consecutive indications may be used based on following proposal, which will further degrade the efficiency of gap skipping indication.

- Bit equal to 1 indicates the corresponding gap/restriction occasion is to be “skipped”. - Option 1: Once a gap/restriction is indicated to be “skipped” by a DCI, UE does not expect the gap/restriction to be indicated as ‘not skipped’ by later DCI. - Bit equal to 0 indicates the corresponding gap/restriction occasion is to be “not skipped”. Once a gap/restriction is indicated to be “not skipped” by a DCI, the gap/restriction can be indicated to be “skipped” by later DCI.

It is more efficient to use a MAC-CE command or RRC signaling to indicate once that a following gap occasion is skipped. The time offset could be longer as it takes more time for UE to decoding MAC-CE command and RRC signaling. However, it should be no problem because for DCI based indication the time offset would be at least 5ms. For gap and restrictions due to L3 measurements outside gap (if supported), MAC-CE and RRC is a better choice. L1 measurements, on the other hand, are serving cell specific. So, the indication via DCI can be used.

In the last RAN4 meeting, there were proposals to introduce semi-static indication schemes.

## Issue 6-1: Semi-static schemes <Way forward>: - Option 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1. - Option 2: RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.3) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period. - Option 3: To guarantee mobility performance, one potential improvement on dynamic indication by DCI is configuring gaps to be skipped during a time period by RRC in advance and gaps indicated by DCI should be subset of gaps configured by RRC - Option 4: Follow RAN1 working assumption and consider DCI only

Based on above analysis, following proposals are present.

Proposal 17: RAN4 agrees to introduce indication via MAC-CE command and/or RRC signaling for gap skipping and restriction skipping due to L3 measurements outside gap (if supported).

Proposal 18: RAN4 agrees to use indication via DCI for restriction skipping due to L1 measurements (if supported).

It needs further consideration for per-FR gap indication. For Type-1 MG and Type-2 MG, per-FR gaps may be considered if a UE supports independent measurement gap configurations for FR1 and FR2. In this case, it is not clear which gap(s) are indicated by DCI, especially when the two gaps are overlapped in some occasions as depicted in Fig 6.

Fig 5. Per-FR Configuration of measurement gaps

If the indication is based on MAC-CE and RRC, it also needs to indicate which per-FR gap(s) are to be skipped.

Proposal 19: It needs to be clarified that when per-FR gaps are configured, the indication of the particular gap to be skipped by DCI or MAC-CE/RRC is for which gap pattern(s).

## 4.2 Time offset

According to RAN1 LS [4], minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

In the last RAN4 meeting, following proposals were provided.

## Issue 4-1: Time offset requirement for measurement gap skipping <Way forward>: - Option 1: 7 ms - Option 2: 5 ms - Option 3: DCI decoding time defined in TS 38.214 + [5]ms - Option 4: assume subclause 5.3.1 of TS38.214 as a baseline for X offset. - Option 5: Fraction of a slot - Option 6: besides the DCI decoding delay, no additional processing delay is needed. - Option 7: N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214)

In Rel-17, RAN4 specified requirements for pre-configured measurement gap where additional margin (time offset) is needed for gap activation/deactivation. If the gap activation is upon DCI-based BWP switch, the requirements are as follows.

8.19.2 Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch 8.19.2.1 Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC The requirements in this clause only apply to the case that the DCI/timer-based BWP switch is performed on a single CC with more than one BWP configurations configured on the CC. When BWP switch occurs, which results in status change of pre-configured measurement gap according to clause 9.1.7, UE shall be able to finish pre-configured activation or deactivation within 5 ms after the completion of the active BWP switch. The active BWP switch delay for single CC is defined in clause 8.6.2. Activation/deactivation of Pre-MG takes effect from the first complete MG occasion after the activation and deactivation delay. If the end of activation/deactivation of Pre-MG is within a gap occasion, the Pre-MG status shall not be changed immediately. Instead, the Pre-MG status shall be changed prior to the next gap occasion.

The overall activation delay is depicted in Fig 7. According to the requirements, only the gaps after BWP switch delay + 5ms from starting of slot n will take effect. For example, the gap occasion filled with pattern cannot be used for measurement since the UE processing has not been finished. The 5ms is for UE processing. It is noted the time duration for DCI and DCI decoding is included in BWP switch delay.

Fig 7. Per-FR Configuration of measurement gaps

If network indicates a gap to be skipped by DCI, the overall timeline is depicted in Fig 8. If UE processing time is assumed the same for gap activation/deactivation of pre-configured measurement gap, i.e., 5ms, the overall time offset should include time for DCI and DCI decoding additionally. Thus, the gap filled with pattern cannot be skipped. Only the gap filled by blue can be skipped.

Fig 8. Per-FR Configuration of measurement gaps

The time duration for DCI and DCI decoding would be small. In our view, 5ms UE processing time would be enough to include this time duration. It is proposed the overall time offset between indication and gap occasion to be skipped is 5ms. It means if a DCI indicates skipping a gap in slot n, only the gap after 5ms from starting of slot n can be actually skipped.

Proposal 20: Minimum time offset(s) X between indication to skip and skipped measurement occasion is 5ms.

The updated minimum time offset for DCI based triggering mechanism is as follows.

- Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced.

For MAC-CE based and/or RRC based semi-static indication, minimum time offsets can be defined as follows.

- Minimum time offset(s) between the end of received MAC-CE command or RRC signaling and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced.

For gap skipping triggered by MAC-CE command, the minimum timing offset is proposed as 3Nslotsubframe,μ + 5ms/NR slot length.

For gap skipping triggered by RRC signaling, the minimum timing offset is the same as RRC processing delay defined in Clause 12 of TS 38.331.

Proposal 21: For MAC-CE based and/or RRC based semi-static indication, minimum time offsets is defined as the minimum time offset(s) between the end of received MAC-CE command or RRC signaling and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped.

Proposal 22: Minimum time offset(s) X between indication by MAC-CE command to skip and skipped measurement occasion is 3Nslotsubframe,μ+5ms/NR slot length.

Proposal 23: Minimum time offset(s) X between indication by RRC signaling to skip and skipped measurement occasion is RRC processing delay defined in Clause 12 of TS 38.331.

## 4.3 Working assumption

RAN1 made working assumption of dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. RAN4 was asked whether there is issue with it. RAN4 had preliminary discussion in the last meeting and agreed to further analyse the dynamic indication scheme.

## Issue 7-1: Response on RAN1 working assumption and time offset <Way forward>: - Companies are encouraged to bring analysis on the feasibility of the working assumption from the RAN1 LS in R4-2414910.

Based on discussions in section 4.1, there would be UE complexity issue and low efficiency issue if dynamic indication is used for triggering gap skipping and restriction skipping due to L3 measurements outside gaps. Thus, MAC-CE command and/or RRC signaling indication should be used instead.

Observation 9: There is UE complexity issue and low efficiency issue for dynamic indication via DCI is used for gap skipping and restriction skipping due to L3 measurements outside gaps.

# Summary

In this contribution, we provided views on RRM requirements impact for XR in Rel-19 and on questions raised in the two RAN1 LSs. Following proposals and observations are present.

Proposal 1: Send an LS to RAN2 and RAN1 to ask if DC scenarios are supported for gap cancellation from their perspective.

Proposal 2: All intra-frequency, inter-Frequency, LTE inter-RAT and UTRAN FDD measurements are supported for gap skipping in NR SA and carrier aggregation.

Proposal 3: As baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

Proposal 4: Measurement skipping does not apply for L3 measurements without gaps.

Proposal 5: RAN4 does not consider enabling data transmission/reception on L1-RS symbols that need to be used for L1 measurements if there is scheduling restriction.

Proposal 6: Only L1 measurements for LTM within gaps are supported for gap skipping.

Proposal 7: Both of the two cases, i.e., there is impact to measurements and no impact to measurements due to gap skipping, are supported.

Proposal 8: The patterns of gaps occasions not used for measurement within a time period should be provided as UE assistance information.

Proposal 9: NW may indicate the percentage of measurement gaps not used for measurement during a time period for UE to provide information about gaps not used for measurement.

Proposal 10: UE to indicate whether it can meet L3 measurements requirements when unused gaps reported by UE is used for data scheduling.

Proposal 11: RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured.

Proposal 12: RAN4 to discuss the information about gaps not used for measurement are reported as UE capability or UE assistance information.

Proposal 13: When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements.

Proposal 14: When there is impact to measurements due to gap skipping, RRM measurement requirements are defined by using NR-U like approach with carefully deciding the max number of gaps skipped during the measurement period.

Proposal 15: RAN4 to study possible restrictions on dynamic indication by DCI, if it is confirmed, to achieve good trade-off between data deliver delay and mobility performance.

Proposal 16: RAN4 is to inform RAN1 on the conclusion of whether L3 measurements outside gap and L1 measurements for restriction skipping are supported or not.

Proposal 17: RAN4 agrees to introduce indication via MAC-CE command and/or RRC signaling for gap skipping and restriction skipping due to L3 measurements outside gap (if supported).

Proposal 18: RAN4 agrees to use indication via DCI for restriction skipping due to L1 measurements (if supported).

Proposal 19: It needs to be clarified that when per-FR gaps are configured, the indication of the particular gap to be skipped by DCI or MAC-CE/RRC is for which gap pattern(s).

Proposal 20: Minimum time offset(s) X between indication to skip and skipped measurement occasion is 5ms.

Proposal 21: For MAC-CE based and/or RRC based semi-static indication, minimum time offsets is defined as the minimum time offset(s) between the end of received MAC-CE command or RRC signaling and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped.

Proposal 22: Minimum time offset(s) X between indication by MAC-CE command to skip and skipped measurement occasion is 3Nslotsubframe,μ+5ms/NR slot length.

Proposal 23: Minimum time offset(s) X between indication by RRC signaling to skip and skipped measurement occasion is RRC processing delay defined in Clause 12 of TS 38.331.

Observation 1: Scheduling restrictions are different in terms of conditions and restricted symbols for L3 measurements within gaps, L3 measurements outside gaps and L1 measurements.

Observation 2: Type 1 MG and Type 2 MG, including per-UE gap and per-FR gap, and pre-configured MG can be considered in XR WI in Rel-19, where Type 2 MG needs further clarification.

Observation 3: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied.

Observation 4: It is up to UE implementation which gaps are not used for measurements under current measurement configuration.

Observation 5: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance.

Observation 6: Channel condition, e.g., RSRP, does not provide information about which SMTC occasions are not used for measurements. Moreover, additional information about gaps/SMTCs not used for measurement is still needed.

Observation 7: UE mobility information does not help how gaps/SMTC occasions are used for measurement unless relaxed L3 measurement requirements are specified.

Observation 8: It is not clear which restrictions should be skipped if there is overlapping of reference signals for L1 measurements and SMTC occasions for L3 measurements outside gap.

Observation 9: There is UE complexity issue and low efficiency issue for dynamic indication via DCI is used for gap skipping and restriction skipping due to L3 measurements outside gaps.

- References

- R4-2416864	WF on RRM requirements for NR_XR_Ph3, Nokia

- R1-2405736	LS on UE assistance information, RAN1

- R1-2407561	LS on gaps/restrictions that are caused by RRM measurements, RAN1

- R4-2415198	Further discussion on RRM impacts for XR and reply LS, vivo

- Appendix: Draft reply LS on UE assistance information

3GPP TSG RAN WG4 Meeting #113	R4-24x

Orlando, US, 18th – 22nd November, 2024

Title:	Reply LS on UE assistance information

Response to:	R1-2405736 (R4-2411007)

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name:	Qian Yang

E-mail Address: qian9.yang@vivo.com

1. Overall Description:

RAN4 thanks RAN1 for the LS on UE assistance information.

RAN4 discussed the UE assistance information for reporting not used measurement gas/SMTC occasions to network. Following conclusions are made.

From UE measurements and mobility performance perspective, RAN4 agreed that it is beneficial if UE reports [UE assistance information / UE capability] about not used measurement gaps/SMTC occasions to network and network indicates skipping gaps/SMTC occasions based on the information.

In order for NW and UE to have same understanding on which gaps should be not used for measurement, the patterns of gaps/SMTC occasions not used for measurement within a time period should be provided as [UE assistance information / UE capability].

RAN4 will further study the details and additional conditions of [UE assistance information / UE capability]. RAN1 will be informed once there are new conclusions.

2. Actions:

To: RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to take the above RAN4 agreements into account in the future work.

3. Reference:

- R1-2405736	LS on UE assistance information, RAN1

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #114	 	   February 17 – February 21, 2025			Athens, GR

- Appendix: Draft reply LS on gaps/restrictions

3GPP TSG RAN WG4 Meeting #113	R4-24x

Orlando, US, 18th – 22nd November, 2024

Title:	Reply LS on gaps/restrictions that are caused by RRM measurements

Response to:	R1-2407561

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name:	Qian Yang

E-mail Address: qian9.yang@vivo.com

1. Overall Description:

RAN4 thanks RAN1 for the LS on gaps/restrictions that are caused by RRM measurements.

RAN4 discussed the dynamic indication solution to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. Following conclusions are made.

There is UE complexity issue and low efficiency issue for dynamic indication via DCI is used for gap skipping and restriction skipping due to L3 measurements outside gaps. RAN4 agreed to introduce to introduce indication via [MAC-CE command and/or RRC signalling] for gap skipping and restriction skipping due to L3 measurements outside gap (if supported). RAN4 also agreed to use indication via DCI for restriction skipping due to L1 measurements (if supported).

RAN4 agreed to define requirements for gap skipping. It is under discussion whether or not the restriction skipping due to L3 measurements outside gaps and due to L1 measurements is supported to define requirements.

Additionally, when UE is configured with two per-FR gaps, it is not clear whether all the gaps or which one are to be skipped if the two gaps are colliding on one occasion.

RAN4 agrees that the minimum time offset(s) X between indication to skip and skipped measurement occasion is 5ms.

2. Actions:

To: RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to take the above RAN4 agreements into account in the future work.

3. Reference:

- R1-2407561	LS on gaps/restrictions that are caused by RRM measurements, RAN1

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #114	 	   February 17 – February 21, 2025			Athens, GR

- Appendix: Draft LS on scenarios for gap/restriction skipping

3GPP TSG RAN WG4 Meeting #113	R4-24x

Orlando, US, 18th – 22nd November, 2024

Title:	LS on scenarios for gap/restriction skipping

Response to:

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN2, RAN1

Cc:

Contact Person:

Name:	Qian Yang

E-mail Address: qian9.yang@vivo.com

1. Overall Description:

RAN4 discussed deployment scenarios for gap/restriction skipping. RAN4 made agreements that NR SA and NR carrier aggregation is supported from RAN4 requirements perspective. It is FFS for EN-DC, NR-DC and NE-DC.

If a gap occasion is skipped by MN via DCI indication, there may not be information exchange between MN and SN. It means the SN cannot schedule the UE during the gap occasion cancelled. Thus, benefit to support the feature in DC scenarios is compromised.

RAN4 would like to ask RAN2 and RAN1 if MR-DC scenario is well supported for gap/restriction skipping.

2. Actions:

To: RAN WG2, WG1

ACTION: 	RAN4 kindly asks RAN2 and RAN1 to provide feedback on supporting of MR-DC scenarios for gap/restriction skipping.

3. Reference:

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #114	 	   February 17 – February 21, 2025			Athens, GR