# 3GPP TSG-RAN WG4 Meeting #113	R4-2418770

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.15.3

Source:	OPPO

Title:	On fast SCell activation with EMR

Document for:	Discussion

- Introduction

In last meeting, a WF[1] was approved on fast SCell activation.

In this contribution, we provide our views on fast SCell activation for UE supporting Rel-18 EMR.

- Discussion

#### Issue 1-1-1: Applicability of fast SCell activation delay requirements - FFS Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured Proposals: - Option 1: (vivo, CATT, CMCC, LGE, Apple, ZTE, MTK) - RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured. - Option 2: (OPPO, Nokia, Samsung, Ericsson) - RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR. - Option 2a: (OPPO) - Consider the following two approaches to support the case: - 1) a new indication of valid measurement results of idle/inactive mode can be introduced, or - 2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration. - Option 2b: (Samsung) - RAN4 not to include the case when measIdleValidityDuration-r18 is not configured - RAN4 to include the case when measReselectionValidityDuration-r18 is not configured

If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. It is a valid case.

For R19 UE and network, UE may support R18 eEMR but network doesn’t configure the R18 signaling e.g., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured. The case should not be precluded when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and UE has reported valid results on the SCell to be activated before SCell activation command. The issue here to be solved is how to cover this case with the smaller impact on current eEMR structure.

In our view, for UE supporting the capability of R19 fast Scell activation with EMR, a new indication of valid measurement results of idle/inactive mode can be introduced, or reusing the R18 validity check rule with fulfilling a default value of ValidityDuration. The second option could be more preferred by most companies if RAN4 decided to support. The default value can be set as 5s, like current known condition does. And the spec impact is controllable which can be completed in drafting CR stage.

Proposal 1: RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR.

Proposal 2: To support the case in proposal 1, we suggest two options and prefer alt-2：alt-1)a new indication of valid measurement results of idle/inactive mode can be introduced, or alt-2)reusing the R18 validity check rule with fulfilling a default value of ValidityDuration.

#### Issue 1-1-2: Scope of fast SCell activation for UE supporting Rel-18 eEMR Agreement: - On top of Single Direct SCell Activation at SCell addition, MAC CE based Single SCell Activation is also supported. - Note: MAC CE based PUCCH SCell activation is included.

Agreement: - RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception. - For FR1, - [Y] equals to [5s] for direct SCell activation - FFS: Y for other SCell activation case. - For FR2, - Reuse the existing value of Y in direct SCell activation - FFS: Y for other SCell activation case. Agreement: - the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting). - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

In principle, we think the current valid measurement report in the known/unknown condition can cover the EMR report besides the legacy L3 report, based on the definition of EMR and validation in R18. We confirm the known condition that UE has sent a R18 EMR report within [Y] seconds before SCell activation command reception, and prefer to define the unified values for both RRC based and MAC CE based SCell activation. Y=5s for FR1, and reuse the existing values of Y = 3 or 4s for FR2 with different UE power classes.

Proposal 3: For updated known condition, prefer to define the unified Y values of valid EMR report for both RRC based and MAC CE based SCell activation. Y = 5s for FR1, and reuse the existing values of Y = 3 or 4s for FR2 with different UE power classes.

For continued discussion in next meeting: - For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured: - For FR1: - use existing SSB based known SCell activation delay requirement with [2] SSB samples. - For FR2: - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X] - Option 1 delay components: 1 SSB sample as legacy known case (Apple with short window, OPPO with X=5, MTK, HW) - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion. - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1 - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample (QC, Apple, OPPO, ZTE, Samsung, VIVO, MTK when the validityduration is configured) - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample (QC, ZTE, Nokia, HW, CATT) - K = 0 or 1 if when the “validityduration” is smaller than X - K = 1 or 2 otherwise - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung) - K = 0 or 1 if when the “validityduration” is applied with smaller than X - K = 1 or 2 otherwise - FFS how to test

For SCell activation delay requirements, we prefer to consider “fast” case. Since the target of this WI is to speed up SCell activation delay with EMR, the feasible cases can be identified with the less samples, at least including the case when the “validityduration” is smaller than X. We support option 1 with X=5, because 5s is widely used for the validity check of measurement report for legacy UE. Even for FR2, X could be smaller. In this case, AGC and/or cell search can be skipped.

For other cases if “validityduration” is configured larger than X, the to be activated SCell can be taken as somehow unknown, and SSB samples for AGC and timing acquisition need to be considered.

Therefore, we can support option 4 to include the two cases.

Proposal 4: Support Option 4 for SCell activation delay components:[ K] SSB samples * beam sweeping +1 SSB sample.

•	K = 0 or 1 if when the “validityduration” is smaller than X

•	K = 1 or 2 otherwise

- Conclusion

In this contribution, we discuss the RRM requirement for fast SCell activation with EMR.

Proposal 1: RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR.

Proposal 2: To support the case in proposal 1, we suggest two options and prefer alt-2：alt-1)a new indication of valid measurement results of idle/inactive mode can be introduced, or alt-2)reusing the R18 validity check rule with fulfilling a default value of ValidityDuration.

Proposal 3: For updated known condition, prefer to define the unified Y values of valid EMR report for both RRC based and MAC CE based SCell activation. Y = 5s for FR1, and reuse the existing values of Y = 3 or 4s for FR2 with different UE power classes.

Proposal 4: Support Option 4 for SCell activation delay components: [ K] SSB samples * beam sweeping +1 SSB sample.

•	K = 0 or 1 if when the “validityduration” is smaller than X

•	K = 1 or 2 otherwise

References

[1] R4-2416854, WF on NR_RRM_Ph5_Part2 - fast SCell activation, CATT