3GPP TSG-RAN WG4 Meeting #113	R4-2418774

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.23.4.2

Source:	OPPO

Title:	Discussion on core requirements for LP-WUS/WUR

Document for:	Approval

- Introduction

Previous meeting discussed and reached some agreements regarding LP-WUS/WUR in [1] [2]. This contribution will provide our further considerations on the open issues captured in [3].

- Discussion

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Background: Case #1 and #3 have been agreed RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. RAN4 to further discuss case #2: RRM measurement case index; MR serving cell measurement; MR neighbouring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. Case #4 is covered by issue for higher priority frequency layer: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

During previous meeting, 4 different RRM offloading / relaxation cases were proposed while only case #1 and case #3 were agreed. The remaining issue is whether to additional consider case #2 and case #4. Comparing the case #2 and case #3, we think case #3 is more reasonable. Firstly, if the UE already turns on the MR for serving cell measurement, it could also perform neighboring cell measurement with MR without much extra power consumption. Secondly, working in relaxed cases rather than fully offloading case #1 means that the serving cell quality is not good enough and is likely to further deteriorate, e.g. does not meet the criterion S any longer. To make sure the UE could reselect to a suitable neighboring cell in time, even with equal or lower priority, neighboring cell measurement is necessary. Therefore, we propose to deprioritize case #2. The main purpose of case #4 is to cover higher priority frequency layers, which should be always measured even if the current serving cell has good channel quality. If so, UE could re-select to a new cell with higher priority which may not support LP-WUS/WUR feature. Then UE has to fall back to the normal RRM measurement and PO monitoring operation, and the power saving gain may disappear. To avoid this, case #4 should be deprioritized unless clear benefits are identified.

Proposal 1: Deprioritized case #2 and case #4.

Issue 1-1-2: Measurement requirements to be specified for LP-WUR - Proposals - P1: RAN4 suspends study on measurement requirements for OFDM based LP-WUR based on LP-SS signal at idle/inactive state until more RAN1 conclusions; RAN4 shall focus on following cases: (xiaomi vivo Apple oppo) - Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state - P2: RAN4 needs more progress from RAN1 on whether to have any work on “OFDM-based LP-WUR serving cell measurement requirements based on LP-SS on overlaid OFDM sequences(s) ” scenario. (vivo Apple) - P3: RAN4 to define requirements for OFDM-based LR serving cell measurement based on LP-SS. (Huawei) - P4：RAN4 shall define the serving cell measurement requirements based on LP-SS. However, we deem that the time window like SMTC may not be introduced since LP-SS will not perform neighbor cell measurement. (ZTE) - P5: RAN4 to discuss LP-WUR measurement requirements starting from SSS based measurement and wait for more progress from RAN1 for LP-SS based measurement. For SSS based LP-WUR measurement, the existing intra-frequency cell reselection requirements can be used as baseline with the side condition to be further discussed. (CATT)

Agreement in RAN1 #118bis meeting Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

Different measurement types could be supported by LP-WUR and whether to define the corresponding RRM requirements for each type needs to be discussed. Based on the proposals for issue 1-1-2 above, the main controversial part is whether to consider requirements for OFDM based LP-WUR measurement based on the newly introduced LP-SS. The majority view is to suspend this discussion until more conclusions regarding the overlaid sequence for LP-SS are reached in RAN1. During last RAN1 meeting, it was agreed to support overlaid OFDM sequence(s) for LP-SS. But the design on overlaid OFDM sequence(s) doesn’t target for RRM measurement and no new RAN4 requirements specific to the overlaid sequence(s) are expected from RAN1 perspective. RAN4 should follow RAN1’s guidance and focus on the uncontroversial cases, i.e. OOK based LP-WUR serving cell measurement based on LP-SS and OFDM based LP-WUR serving cell measurement based on existing SSS.

Proposal 2: RAN4 shall NOT specify measurement for OFDM based LP-WUR based on LP-SS, and focus on following two cases:

- Measurement requirements for OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for OFDM based LP-WUR serving cell measurement based on existing SSS at Idle/Inactive state

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation Agreement RAN4 specifies relaxed UE MR measurement requirements for both serving cell and neighbour cell measurements at idle/inactive mode for UE supporting LP-WUR Aspect 1: Agree on relaxation/scaling factor(s) for MR serving/neighbour cell relaxation as the starting point - Support: vivo, ZTE, MTK, Samsung, Apple, HW, China Telecom, QC - Discuss the number later: Nokia, E///, OPPO, Xiaomi, LGE, CATT Aspect 2: The same or different relaxation/scaling factors for MR serving and neighbour cell relaxation - Same: MTK, vivo, QC, China Telecom, HW, OPPO, Xiaomi, CMCC, ZTE, Samsung, Apple - Different: CATT, Nokia - FFS Smaller factor for serving cell or neighboring cell Exact relaxation factor as the starting point - Option 1: Only consider the scaling factor(s) larger than that of legacy Rel-16 UE power saving WI (i.e., 3) (QC, CMCC, LGE, ZTE, HW, China Telecom, Apple, vivo, MTK, Xiaomi, Samsung) - Option 2: 3 is included as one of the factors (E///, Nokia) - For the scaling factor(s) larger than 3, which number can be used as starting point. - 8: CMCC, China Telecom, vivo, Samsung, Ericsson, MTK, QC, Nokia - >=16: vivo, QC, HW, Apple, Samsung, MTK

Last meeting agreed to specify the relaxed MR requirements for both serving cell and neighbour cells. Then the next issue is how to define the relaxed requirements as listed above. For aspect 1, we think the exact number of scaling factor should be determined after sufficient evaluations are performed. As the starting point, we agree with the majority view that the scaling factor should be larger than that of legacy Rel-16 UE power saving WI, i.e. 3. Otherwise, the power saving performance for LP-WUR is worse than Rel-16 UE power saving since UE is additional required to turn on LP-WUR for serving measurement in Rel-18 case #3.

Proposal 3: As the starting point, the scaling factor should be larger than 3.

For aspect 2, we prefer to use the same scaling factor for serving cell and neighbor cells. As mentioned above, our view is to consider case #1 and case #3 only. Then measurement behaviors for serving cell and neighbor cells (e.g. non-relaxed measurement, relaxed measurement, stop measurement) are simultaneous and naturally their scaling factor should be the same. One argument to consider different scaling factors is that the network may expect more frequent measurement or more accurate performance for serving cell. However, we think this could be met even with the same scaling factor since generally one searcher may be shared by multiple frequency layers for neighbor cells measurement, while a dedicated searcher will be used for serving cell measurement.

Proposal 4: Consider the same scaling factor for both serving cell and neighbor cell MR relaxed measurements.

Issue 1-2-3: On interruption related requirements for LP-WUR - Proposals - P1: Interruption requirements could be defined for the duration from LP-WUS received paging indication to MR ready for paging monitoring (CMCC vivo Samsung Docomo CT) - P2: Interruption requirements may need to be specified for the following:(CMCC) - Between LP-WUR measurement result fulfills exit criteria and MR to start measurement - Between MR measurement result fulfills entry criteria and LP-WUR to start measurement - P3: RAN4 only needs to define the wake-up delay requirement, if necessary, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring (Apple) - P4: Wait for more RAN1 progress on PO monitoring before discussing the possible interruption requirements related to LR (Huawei) - P5: RRM requirements for interruption should be based on the RAN1 agreements for wake-up delay (NTT Docomo) - P6: RAN4 shall clarify the timeline from decoding LP-WUS to MR wake-up in order to monitor legacy PO. (ZTE)

Agreement in RAN1 #118 meeting For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

Agreement in RAN1 #118bis meeting Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state. - FFS the values - Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective

Issue 1-2-3 proposed to define the possible interruption between LP-WUS with paging indication and MR ready for paging monitoring. It is reasonable to define such the requirements. In our understanding, this interruption time is the wake-up delay discussed in RAN1, which is also applicable for RRC idle/inactive modes. As shown above, X=3 candidate values will be defined depending on UE capability reporting. And RAN4 should take these values as the starting point to define the interruption time.

Proposal 5: The interruption time between LP-WUR reception and MR ready for paging monitoring could be defined based on the wake-up delay determined in RAN1.

Issue 1-1-8: LP-WUR operating carrier frequency Agreement: - RAN4 will further discuss whether any technical issues regarding the LS from RAN1. - From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2. [Issue for further discussion based on contribution driven: - For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

LS from RAN1, R1-2407559 In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

Whether to support LR and MR operating on different carriers/bands is discussed in RAN1. Although the common understanding in RAN1 is that UE may not support LP-WUS reception on all the bands, further check in RAN2 and RAN4 is needed. During the last RAN4 meeting, we initially agreed to consider the scenario when LR and MR measurements are on the same carrier frequency as baseline to define Rel-19 RRM requirements. And companies are encouraged to provide technical analysis on the RAN1 LS. In our view, the following issues are observed in the scenario of different carrier frequencies. Firstly, the synchronization between different frequencies cannot be guaranteed. Then UE may be required to perform additional time tracking when switching between LR and MR and the power saving gain will be impacted. Besides, the channel conditions between LR and MR carriers may be quite different, for example due to the interference level. Then the following cases may happen:

- Case A: The channel condition for LR carrier is worse than that for MR carrier. Then, the measurement results by LR may be very lower and trigger the UE to fall back to MR measurement. However, the measurement results by MR may be good enough and UE will entry LR measurement again. Without a carefully designed criteria or threshold margin, UE may switch between LR and MR repeatedly and result in ping-pong effect.

- Case B: The channel condition for LR carrier is better than that for MR carrier. When the serving cell quality in MR carrier get worse, UE in fully offloaded state (measuring serving cell by LR only) cannot identify this issue timely and will stay in LR. Once it is waken-up by LP-WUS indication and try to switch to MR, the UE cannot receiving the PO directly and may need to perform synchronization, measurement or even cell re-selection, which will increase the wake-up delay and UE power consumption.

Observation 1: When LR and MR on the different frequencies, the following issues are identified:

- Synchronization issues between different carriers

- Different channel conditions between different carriers, which may results in ping-pong effect between LR and MR switch or longer wake-up delay

- Complicated UE implementation

- More UE power consumption

For LP-WUS monitoring, we believe the similar issues for RRM measurement exist and it is better to take the same carrier frequency as the baseline as well.

Proposal 6: Consider LR-WUS reception and MR on the same carrier frequency as the baseline.

- Conclusion

This contribution gave the following proposals on the core requirements for LP-WUR/WUS.

Proposal 1: Deprioritized case #2 and case #4.

Proposal 2: RAN4 shall NOT specify measurement for OFDM based LP-WUR based on LP-SS, and focus on following two cases:

- Measurement requirements for OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for OFDM based LP-WUR serving cell measurement based on existing SSS at Idle/Inactive state

Proposal 3: As the starting point, the scaling factor should be larger than 3.

Proposal 4: Consider the same scaling factor for both serving cell and neighbor cell MR relaxed measurements.

Proposal 5: The interruption time between LP-WUR reception and MR ready for paging monitoring could be defined based on the wake-up delay determined in RAN1.

Observation 1: When LR and MR on the different frequencies, the following issues are identified:

- Synchronization issues between different carriers

- Different channel conditions between different carriers, which may results in ping-pong effect between LR and MR switch or longer wake-up delay

- Complicated UE implementation

- More UE power consumption

Proposal 6: Consider LR-WUS reception and MR on the same carrier frequency as the baseline.

References

[1] R4-2416883, Coffee break discussion minutes for NR_LPWUS, vivo, RAN4 112bis

[2] R4-2416861, WF on RRM requirements for NR_LPWUS, vivo, RAN4 112bis

[3] R4-2415664, Topic summary for [112bis][217] NR_LPWUS, vivo, RAN4 112bis