3GPP TSG-RAN WG4 Meeting #113	R4-2418790

Orlando, US, November 18th – 22nd 2024

Agenda Item:	7.20.1

Source:	Ericsson

Title:	SBFD general aspects

Document for:	Approval

# 1	Introduction

In this contribution, we present our views and proposals on general aspects for SBFD capable BS regarding band support for SBFD capable BS, SBFD sub-band configuration, and specification structure. Comprehensive analysis of each SBFD BS RF TX/RX requirement and potential new requirements for SBFD capable BS are detailed in separate submissions [1-3].

# 2	Discussion

2.1	Band support

##### Issue 1-1-1: Minimum channel bandwidth for SBFD (X MHz) Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = 40 as the default value - For band n39, X = 20MHz is used instead. ##### Issue 1-1-3: Specification implementation for band/channel bandwidth support for SBFD Agreement: - To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD. - Detailed band list is provided based on the agreement for minimum channel bandwidth for SBFD - FFS the band for NR-U shall be supported

After the intensive discussion in RAN4#112bis, the minimum channel bandwidth for SBFD was agreed as captured in the WF [6]. Regarding how to implement band/channel bandwidth support for SBFD in specification, it was agreed to include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD, where detailed band list needs to be provided based on the agreement for minimum channel bandwidth for SBFD.

In addition, whether the bands for NR-U should support SBFD requires further study. In bands n46 and n96, there are specific regulations and technical considerations that can impact uplink and downlink usage, e.g., both bands are subject to regulations like dynamic frequency selection (DFS) and listen-before-talk (LBT) to ensure fair usage and minimize interference with other technologies operating in the same spectrum. Therefore, in our opinion, bands n46 and n96 should be excluded from the scope.

To include the following text in the specification (either new specification or TS 38.104, which is still under discussion): subband full duplex can be applied to the following TDD bands given in Table 5.2-1 in TS 38.104: n38, n39, n40, n41, n48, n50, n77, n78, n79, n90, n102, n104 for FR1.

As for sub carrier spacing (SCS), for FR1 with 60 kHz SCS, there are no specific Demod and RRM requirements. For FR2 with 60 kHz SCS, there are some Demod and RRM requirements. Despite this, no vendors implement it nowadays. Therefore, we propose to exclude 60 kHz SCS for SBFD symbols.

For FR1 and FR2, do not use 60 kHz SCS for SBFD symbols.

2.2	SBFD subband configuration

2.2.1	Clarification on declaration and flexibility

There are three options identified in the WF [4] from RAN4#112, which were distinguished by their level of flexibility. From a standardization perspective, it is important to exercise caution when allowing flexibility in technical standards/specifications. In our view, SBFD subband configurations need to be captured in RAN4 specification. Manufacturer can declare which configurations defined in specification are supported, and all supported SBFD configurations shall be tested, due to the fact that for SBFD capable BS, achieving different configurations may require adapting hardware components, which may involve variations in analog and/or digital filter characteristics. Most importantly, the BS product shall remain fixed in its configuration during real operation, i.e., it shall not be reconfigurable to a different configuration/DL subband/UL subband/guardband setting other than those that have been declared and tested.

The SBFD capable BS product shall remain fixed in its configuration during real operation, i.e., it shall not be reconfigurable to a different configuration/DL/UL/guardband setting other than those that have been declared and tested.

Based on the AdHoc discussion in RAN4#112, the three (four) options can be down selected to the following two options.

- Option 1: fixed configurations/UL subbands/DL subbands/guardbands.

- Option 2b: fixed UL subbands and vendor declared DL subbands and guardbands.

The pros and cons of the two options are analyzed in table 1, and a critical issue on interoperability is identified: Different vendors' BSs using varying sizes of DL subbands/UL subbands/guardbands may cause interference. Consequently, an operator must either deploy the entire network with a single vendor or ensure same SBFD subband configuration through the flexible configurations between different vendors.

Table 1: Pros and Cons of SBFD subband configuration options

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | - An operator can deploy the network with different vendors. - From a standardization perspective, test models can be clearly defined. - From a regulator perspective, no flexibility and manufacturer declaration is allowed. - Reduce SBFD aware UEs’ complexity. - A limited number of configurations simplify the standardization work, and make sure test models are under control. | - Cannot cover all possibilities, but desired configurations by operator can be added in SBFD enhancement WI. - Difficult to define guardbands for subbands. |
| Option 2b | - Vendor can select guardband freely, and guardband is up to implementation. | - Interoperability issue: one operator has to deploy the whole SBFD network with the same vendor, or needs to ensure a common configuration is available by having the vendors collaborate. - Importance of defining RF requirements in RAN4 specifications are weakened. - Regulator will not accept manufacturer declaration. |

To have a clear test scope and test parameters (test models etc. need to be designed for each configuration), we should have a limited set of SBFD configurations. Although RAN1 allows more flexibility in selecting bandwidths and subcarrier spacings (SCSs), in RAN4, we have restricted these parameters to limited sets. Likewise, we propose limiting SBFD configurations to a predefined set.

Another aspect which needs to be considered is whether SBFD aware UEs need to support all configurations. If UEs need to support every possibility, it is UEs’ complexity as well. To ease the burden of conformance testing and reduce the complexity for SBFD aware UEs, we explore the following ways to streamline SBFD subband configurations.

SBFD configurations shall be captured in Rel-19 RAN4 specification and restricted to a limited set.

Based on our observation, it is reasonable to fix the size of UL subband as described in Option 2b to be able to reuse the FRCs defined in TS 38.104 [5]. Option 2b has two level of flexibilities: one is the size of the UL subband of a supported channel bandwidth; another is the location of the UL subband. In our view, UL should be around 20% of supported channel bandwidth, since feasibility and co-existence only studied 20%, i.e., for one supported channel bandwidth, only one fixed UL subband size is allowed. On one hand, there is no need to support narrower sizes of UL subband, as it doesn’t gain much to deploy SBFD in this case. On the other hand, if the size of UL subband is too wide, the DL subbands will be impacted. Moreover, the UL subband should be placed right in the middle of the channel bandwidth, as we do not see any benefit from flexible placement of the UL subband.

When the two level of flexibilities are out of scope, the only remaining differences between Option 1 and Option 2b are the selection of the guardbands and the sizes of DL subbands. As mentioned in table 1, a critical issue is interoperability between different vendors. Therefore, we propose to proceed with Option 1 and in the following text we will provide our recommendations on the selection of the guardbands and the sizes of DL subbands.

For one supported channel bandwidth, only one fixed UL subband size is specified, which is around 20% of the supported channel bandwidth.

Only symmetric DUD configurations are specified, as there is no foreseeable benefit from flexible placement of the UL subband.

Select Option 1 to specify DL/UL subband configuration in BS RF requirements, due to interoperability issues between different vendors.

2.2.2	Proposal of SBFD subband configuration

It has been agreed that only DUD and DU configurations are specified for the SBFD subband configuration.

In the SI phase, the feasibility and co-existence study was based on the assumption that UL subband occupies 20% BS channel bandwidth. Hence, we propose to continue with the assumption that UL subband occupies 20% BS channel bandwidth for FR1 in the WI phase. As justified by companies in RAN4#112, it is more important to fix the UL subband size than DL subband size: Thus, to be able to reuse the existing channel bandwidths for UL subbands, the percentage is relaxed to 20% - 25% to design suitable SBFD subband configurations for all channel bandwidths between 50 MHz and 100 MHz.

Reuse the existing channel bandwidths for SBFD UL subbands.

The most important part is the selection of guardband size between DL and UL subbands. Different BS classes may require different techniques to suppress interference during SBFD operation, for example, WA BSs would require analogue filtering, digital filtering, and high linearity of the receiver, etc., while for LA BSs there is no need to build analogue filter and highly linear receiver. The guardband size can thus be different for different BS classes. For SBFD-capable WA BSs, wider guardbands than the guards used for BS in normal TDD operation are needed.

For WA BSs, wider guardbands than existing ones are needed for SBFD.

To set the guardbands for WA BS, a general calculation rule is proposed, i.e., the guardband size should be larger than the sum of the minimum guardbands (defined in TS 38.104 [5]) of the adjacent DL subband and the adjacent UL subband.

The calculation approach for SBFD transmission bandwidth configuration for FR1 is demonstrated as below.

- Step 1 – Define NRB,SBFD,UL. Reuse the existing number defined in Table 5.3.2-1 in TS 38.104 [5] corresponding to 20% - 25% of the considered channel bandwidth.

- Step 2 – Define NRB,SBFD,GB. Sum up the minimum guardbands (defined in Table 5.3.3-1 in TS 38.104 [5]) of the adjacent DL subband and the adjacent UL subband. Convert the number in kHz to RB and round up the decimal. The result obtained is the needed NRB,SBFD,GB.

NOTE: When the adjacent DL subband is not standard channel bandwidth, use the larger minimum guardband of the two closest channel bandwidths. E.g., for 70 MHz DUD (27.5 MHz - 15 MHz - 27.5 MHz), 30 kHz SCS, the minimum guardband for DL subband should use 945 kHz (corresponding to 30 MHz) instead of 785 kHz (corresponding to 25 MHz).

- Step 3 – Define NRB,SBFD,DL.

  - For DUD, NRB,SBFD,DL = (NRB – NRB,SBFD,UL – 2 * NRB,SBFD,GB)/2.

  - For DU/UD, NRB,SBFD,DL = NRB – NRB,SBFD,UL – NRB,SBFD,GB.

NOTE: When NRB is an odd number and NRB,SBFD,UL is an even number, e.g., calculation for 50 MHz channel bandwidth, the obtained NRB,SBFD,DL is a decimal of 0.5. In this case, NRB,SBFD,DL for two DL subbands should be 49 and 50.

It is also worth noting that, the NRB,SBFD,DL may not be the existing number defined in TS 38.104 [5], and the corresponding test models need FFS.

To have a straightforward view, the proposed supported BS channel bandwidth and possible subband configurations for WA BS are listed in Table 2 for FR1.

RAN4 define SBFD subband configurations for WA BS as shown in Table 2 for FR1.

Table 2: Transmission bandwidth configuration NRB for SBFD for FR1

|  |  | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | D | 20 | 22.5 | 27.5 | 30 | 35 | 40 |
|  | U | 10 | 15 | 15 | 20 | 20 | 20 |
|  | D | 20 | 22.5 | 27.5 | 30 | 35 | 40 |
| DUD (SCS 30 kHz) | NRB,SBFD,DL | 49/50 | 5857 | 70/71 | 78 | 92 | 106 |
|  | NRB,SBFD,UL | 24 | 38 | 38 | 51 | 51 | 51 |
|  | NRB,SBFD,GB | 5 | 45 | 5 | 5 | 5 | 5 |
|  | D | 40 | 45 | 55 | 60 | 70 | 80 |
|  | U | 10 | 15 | 15 | 20 | 20 | 20 |
| DU/UD (SCS 30 kHz) | NRB,SBFD,DL | 104 | 119 | 146 | 161 | 189 | 217 |
|  | NRB,SBFD,UL | 24 | 38 | 38 | 51 | 51 | 51 |
|  | NRB,SBFD,GB | 5 | 5 | 5 | 5 | 5 | 5 |

NOTE: the subband bandwidths of DUD and DU/UD in Table 2 with strikethrough are in MHz and just for understanding, which will be removed when implemented in spec.

In addition, Table 2 shows that the numbers of NRB,SBFD,GB for different configurations are all 5, except for the case of DUD 60 MHz, which is 4. To maintain uniformity across the table, we recommend using 5 RBs as the guardbands for DUD 60 MHz, and the corresponding NRB,SBFD,DL is reduced to 57, instead of 58.

2.2.3 Discussion on option 2b

##### Issue 1-1-5: DL/UL subband configurations support for SBFD Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - For DUD case, the UL subband should be placed in the middle of channel bandwidth - FFS the number of DL subband sizes to be declared and tested - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed. - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Restriction on UL subband size based on the specific channel bandwidth - FFS to specify a range of ratio for UL subband size over channel bandwidth. - FFS the restriction is different for DU and DUD - FFS the restriction is applied for FR2 case.

From a conformance testing perspective, as agreed in RAN4#112bis, SBFD feature could be supported by a broad range of bandwidths from 40 MHz to 100 MHz for FR1, with SCS of 15 kHz and 30 kHz. Multiple declarations for each combination of CBW and SCS will cause a significant increase in permutations. If more than one guardband is allowed to be declared for each permutation, it will further increase the number of permutations, which indicates that conformance testing of SBFD capable BS would explode.

Increasing the ratio of the uplink subband results in fewer PRBs for the downlink. Allocating the total available power to fewer PRBs will affect all emission-related requirements. From a regulator perspective, conformance should be ensured for all declared values, as identifying the worst case may not be straightforward. The worst case could be either with the smallest guardband, or with the largest guardband, as it creates the highest PSD on the DL, or others. Therefore, it's necessary to demonstrate compliance with general emissions requirements for each DL size, as different parameters can impact the results.

Additionally, changing the ratio can degrade downlink resources, impacting capacity and performance due to reduced DL availability.

Therefore, our view on option 2b is as follows with the condition that all new necessary in-channel adjacent subband leakage ratio and blocking/selectivity requirements are set properly.

- For DUD case, the UL subband should be placed in the middle of channel bandwidth.

- Regarding the number of UL subband sizes to be declared and tested:

  - Only one UL subband size is allowed per channel bandwidth, and it should be 20-25% of channel bandwidth for both DU and DUD for both FR1 and FR2.

- Regarding the number of DL subband sizes to be declared and tested:

  - For each specific channel bandwidth/UL subband size, one DL subband size, one guardband between subbands is declared and tested, operation with other guardband sizes and DL subband sizes than the tested ones are not allowed.

2.2.4	Figures for BS transmission bandwidth configuration for SBFD

##### Issue 1-1-4: Transmission bandwidth configuration for SBFD Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

In the WF [6] approved in RAN4#112bis, under Issue 1-1-4: Transmission bandwidth configuration for SBFD, illustrative figures from Ericsson R4-2416134 will be used as the baseline for further study. Based on offline comments, the figure for DUD pattern is modified as shown below together with DU/UD patterns. Therefore, we propose to introduce Figure 1-3 as shown below, where the agreed notations for transmission bandwidth configuration for SBFD UL/DL subband: NRB,SBFD,DL and NRB,SBFD,UL, as well as guardband between SBFD UL and DL subband: NRB,SBFD,GB are included, to either TS 38.104 or a SBFD specific spec, which is another topic still under discussion.

Introduce Transmission bandwidth configuration for SBFD DUD, DU and UD patterns, as shown in Figure 1-3 below.

Figure 1: Transmission bandwidth configuration for SBFD DUD pattern for one NR channel

Figure 2: Transmission bandwidth configuration for SBFD DU pattern for one NR channel

Figure 3: Transmission bandwidth configuration for SBFD UD pattern for one NR channel

2.3	Specification structure

##### Issue 1-2-1: BS RF Specification structure for SBFD requirements Way Forward: - FFS options of how to introduce BS RF new requirements for SBFD-capable BS: - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option-3: A new specification for SBFD BS RF requirements

In the WF [6] approved during RAN4#112bis, there are two options captured on how to introduce BS RF new requirements for SBFD-capable BS. We prefer to create new specifications for SBFD-capable BS, rather than merging it into TS 38.104. The reasons are as follows.

On one hand, a dedicated SBFD spec ensures clarity. It includes only relevant information related to bands, bandwidth, and SBFD configurations, etc., and avoids unnecessary complexity and provides a concise, focused reference for SBFD-related details, which makes it easier to understand and maintain. Creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 for SBFD-specific existing or new gNB RF requirements could reduce the number of 3GPP specifications, but increase the complexity and reduce the readability of TS 38.104, as what we have on UE side. On the other hand, SBFD-capable BS may require new hardware components, which makes it different from legacy base stations.

Moreover, new requirements, e.g., in-channel adjacent subband leakage ratio and blocking/selectivity, are needed for SBFD, and it is more proper to capture them in a new specification.

RAN4 create new specifications for SBFD-capable BS.

# Conclusion

In this contribution, views, and proposals on general aspects for SBFD WI are presented.

Observation 1	SBFD configurations shall be captured in Rel-19 RAN4 specification and restricted to a limited set.

Proposal 1	To include the following text in the specification (either new specification or TS 38.104, which is still under discussion): subband full duplex can be applied to the following TDD bands given in Table 5.2-1 in TS 38.104: n38, n39, n40, n41, n48, n50, n77, n78, n79, n90, n102, n104 for FR1.

Proposal 2	For FR1 and FR2, do not use 60 kHz SCS for SBFD symbols.

Proposal 3	The SBFD capable BS product shall remain fixed in its configuration during real operation, i.e., it shall not be reconfigurable to a different configuration/DL/UL/guardband setting other than those that have been declared and tested.

Proposal 4	For one supported channel bandwidth, only one fixed UL subband size is specified, which is around 20% of the supported channel bandwidth.

Proposal 5	Only symmetric DUD configurations are specified, as there is no foreseeable benefit from flexible placement of the UL subband.

Proposal 6	Select Option 1 to specify DL/UL subband configuration in BS RF requirements, due to interoperability issues between different vendors.

Proposal 7	Reuse the existing channel bandwidths for SBFD UL subbands.

Proposal 8	For WA BSs, wider guardbands than existing ones are needed for SBFD.

Proposal 9	RAN4 define SBFD subband configurations for WA BS as shown in Table 2 for FR1.

Proposal 10	Introduce Transmission bandwidth configuration for SBFD DUD, DU and UD patterns, as shown in Figure 1-3 below.

Figure 1: Transmission bandwidth configuration for SBFD DUD pattern for one NR channel

Figure 2: Transmission bandwidth configuration for SBFD DU pattern for one NR channel

Figure 3: Transmission bandwidth configuration for SBFD UD pattern for one NR channel

Proposal 11	RAN4 create new specifications for SBFD-capable BS.

# References

R4-2418792, Impact on SBFD BS RF TX requirements, Ericsson

R4-2418793, Impact on SBFD BS RF RX requirements, Ericsson

R4-2418791, Potentially new SBFD BS RF requirements, Ericsson

R4-2413514, Way Forward for [112][307] NR_duplex_evo_General

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

R4-2416584, Way forward for [112bis][306] NR_duplex_evo_General