3GPP TSG-RAN WG4 Meeting #113	R4-2418802

# Orlando, United States,18 – 22 Nov., 2024

Source: 	Huawei, HiSilicon

Title:	Ambient IoT co-existence evaluation cases

Agenda Item:	7.21.2.1

Document for:	Approval

1	Introduction

The SID on solutions for Ambient IoT was revised in RAN#103 [1]. In the latest two meetings, co-existence evaluation cases and results have been provided. In the previous meeting, we agreed on three options for the NR BS inband ACLR value with 1 blank RB: 30dBc, 17dBc, and 12dBc [2]. This contribution shares our opinion on this issue.

2	Discussion

To find out the NR BS in band ACLR value, we measured the inband ACLR of  an 10M NR carrier with blank RBs. As the guard RB between unscheduled RBs and transmitting RB increase, the in-band ACLR also increases. The NR BS in-band ACLR ranges from 19 to 33dB with 0 to 2 guard RBs.

Table 1 measurement results of NR BS in band ACLR

| number of guard RB | NR BS in band ACLR （dB） |
| --- | --- |
| 0 | 19 |
| 1 | 28 |
| 2 | 33 |

Observation 1: Based on measurement, the NR BS in-band ACLR range from 19 to 33dB with 0 to 2 guard RBs.

In the evaluation of A-IoT coexistence, we assume that there are no guard RBs. However, in actual deployment, it will be beneficial to allow some guard RBs to improve coexistence performance.

Proposal 1: Different NR BS in band ACLR values can be used for evaluate the number of guard RBs required to ensure coexistence performance.

Proposal 2: Guard RBs can be used to improve coexistence performance if necessary.

The different value of NR BS in band ACLR will only impact on case 1-1b and case 1-2b. We simulated all three options and the results are as follows.

For case 1-1b, the simulation results are as follows:

Figure 1-1 co-existence simulation results of case 1-1b with 30dB NR BS in band ACLR

Figure 1-2 co-existence simulation results of case 1-1b with 17dB NR BS in band ACLR

Figure 1-3 co-existence simulation results of case 1-1b with 12dB NR BS in band ACLR

For case 1-1b, the simulation results are summarized in the table as follows:

Table 3 Summary of co-existence simulation results for case 1-1b

| NR BS in-band ACLR | Case 1-1b Co-existence simulation result for SNR degradation （dB） |  | Co-existence simulation conclusion |
| --- | --- | --- | --- |
|  | 5% CDF | 50% CDF |  |
| 30dB | 0 | 0 | SINR degradation is less than 1dB. Inter-system interference is negligible. |
| 17dB | 0.8 | 1 | SINR degradation is less than 1dB. Inter-system interference is negligible. |
| 12dB | 5 | 2 | SINR degradation is larger than 1dB For the min D2R SNR -7.8dB，outage percentage is1% For the average D2R SNR -4.3dB，outage percentage is 2% For the max D2R SNR 0.23dB，outage percentage is 4% |

For case 1-2b, the simulation results are as follows:

Figure 2-1 co-existence simulation results of case 1-2b with 30dB NR BS in band ACLR

Figure 2-2 co-existence simulation results of case 1-2b with 17dB NR BS in band ACLR

Figure 2-3 co-existence simulation results of case 1-2b with 12dB NR BS in band ACLR

For case 1-2b, the simulation results are summarised in the table as follows:

Table 3 Summary of co-existence simulation results for case 1-2b

| NR BS in-band ACLR | Case 1-2b co-existence simulation result for SNR degradation （dB） |  | Co-existence simulation conclusion |
| --- | --- | --- | --- |
|  | 5% CDF | 50% CDF |  |
| 30dB | 0 | 0 | SINR degradation is less than 1dB. Inter-system interference is negligible. |
| 17dB | 4 | 4 | SINR degradation is larger than 1dB For the min D2R SNR -7.8dB，outage percentage is 0.1% For the average D2R SNR -4.3dB，outage percentage is 0.5% For the max D2R SNR 0.23dB，outage percentage is 1% |
| 12dB | 8 | 6 | SINR degradation is larger than 1dB For the min D2R SNR -7.8dB，outage percentage is 0.1% For the average D2R SNR -4.3dB，outage percentage is 1% For the max D2R SNR 0.23dB，outage percentage is 3% |

Based on the simulation results, when the NR BS in band ACLR is 30 dBc, for case 1-1b and case 1-2b,  the inter-system interference is negligible.

When the NR BS in band ACLR is 17 dB, the inter-system interference is also negligible for case 1-1b. In case 1-2b, the SINR degradation at 5% and 50% CDF is 4dB, and the outage percentage is less than 1%.

When the NR BS in band ACLR is 12 dB, the SINR degradation for case 1-1b and case -2b are within 5dB and 8 dB respectively, and the outage percentage is within 1% and 3% respectively.

Observation 2:

When the NR BS in band ACLR is 30 dB, for case 1-1b and case 1-2b,  the inter-system interference is negligible.

When the NR BS in band ACLR is 17 dB, the inter-system interference is also negligible for case 1-1b. In case 1-2b, the SINR degradation at 5% and 50% CDF is 4dB, and the outage percentage is less than 1%.

When the NR BS in band ACLR is 12 dB, the SINR degradation for case 1-1b and case -2b are within 5dB and 8 dB respectively, and the outage percentage is within 1% and 3% respectively.

3	Conclusion

This contribution we measured the NR BS in-band ACLR and simulated SNR degradation with different ACLR. We then analysed the effect of these values on the coexistence performance. We have the following observations and proposals:

Observation 1: Based on measurement, the NR BS in-band ACLR range from 19 to 33dB with 0 to 2 guard RBs.

In the evaluation of A-IoT coexistence, we assume that there are no guard RBs. However, in actual deployment, it will be beneficial to allow some guard RBs to improve coexistence performance.

Observation 2:

When the NR BS in band ACLR is 30 dB, for case 1-1b and case 1-2b, the inter-system interference is negligible.

When the NR BS in band ACLR is 17 dB, the inter-system interference is also negligible for case 1-1b. In case 1-2b, the SINR degradation at 5% and 50% CDF is 4dB, and the outage percentage is less than 1%.

When the NR BS in band ACLR is 12 dB, the SINR degradation for case 1-1b and case -2b are within 5dB and 8 dB respectively, and the outage percentage is within 1% and 3% respectively.

Proposal 1: Different NR BS in band ACLR values can be used for evaluate the number of guard RBs required to ensure coexistence performance.

Proposal 2: Guard RBs can be used to improve coexistence performance if necessary.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA, RAN #103, Mar. ,2024

[2]R4-2417196, WF on co-existence study for A-IOT, Oct., 2024, RAN4#112bis