3GPP TSG-RAN WG4 Meeting #113	R4-2418812

Orlando, USA, 18 – 22 November 2024

Source:	Ericsson

Title:	TP for TR 38.718-11-11: DC_68A_n38A

Agenda item:			6.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.719-11-11 to include DC_68A_n38A as defined in WID [1].

# ---Start of changes---

## 6.1	Inter-band DC within FR1

### 6.1.X	DC_68_n38

#### 6.1.X.1	Configuration for DC

Table 6.1.X.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n38A | DC_68A_n38A | No |

#### 6.1.X.2	Maximum output power for DC

Table 6.1.X.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n38A | 23 | +2/-3 |

#### 6.1.X.3	Spurious emission band UE co-existence for DC

Table 6.1.X.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_68_n38 | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

#### 6.1.X.4	Co-existence analysis for DC

Table 6.1.X.4-1 Band 68 and Band n38 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2 792 | 3 490 |  |
| n38 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3 606 |  |
| DL1 | 2570 | 2620 | N/A | - | - | - | - | UL harmonic |
| DL23 | 5140 | 5240 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 7710 | 7860 | - | - | N/A | - | N/A |  |
| DL4 | 10280 | 10480 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 12850 | 13100 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic impacts |  |  |  |  |  |
| UL/DL harmonics |  | n38 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 2570 | 5140 | 7710 | 10280 | 12850 |  |
| 68 | flow | fhigh | 2620 | 5240 | 7860 | 10480 | 13100 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic mixing impact |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.X.4-3: Band 68 and Band n38 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1842 - 1922 |  | 3268 - 3348 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1114 - 1224 |  | 4412 - 4542 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3966 -4076 |  | 5838 - 5968 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 386 - 526 |  | 6982 - 7162 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3684 - 3844 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4664 - 4804 |  | 8408 - 8588 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6536 - 6696 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9782 - 9552 |  | 172 - 342 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10978 – 11208 |  | 5362 - 5532 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6254 - 6464 |  | 2956 - 3146 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9106 - 9316 |  | 7234 - 7424 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.X.4-3, there is no IMD issue for this combination.

#### 6.1.X.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n38 as specified in TS 38.101-3.

Table 6.1.X.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n38 | 0.3 | 0.3 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.X.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n38 | - | - |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

#### 6.1.X.6	Analysis of MSD requirements

Based on the co-existence studies for DC_68_n38, no MSD requirements are needed for this combination.

---End of changes---

# Reference

[1] 	 RP-241786, Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands, Nokia