# 3GPP TSG-RAN WG4 Meeting #113                               R4-2418829

# Orlando, USA, 18th-22th Nov, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on receiver characteristics for LP-WUR

# Agenda Item:   7.23.2.3

# Document for:  Approval

- Introduction

In this contribution, we provided our views on receiver characteristics for LP-WUR.

- Discussion

## 2.1 Maximum input level

In last meeting, RAN4 discussed maximum input level for LP-WUR and most companies think that legacy -25dBm is okay. A noticeable point a whether this value should be scaled-down on the LP-WUR RB.

Issue 3-2-2: Maximum input level requirements Agreements: - The same maximum input power of [-25 dBm] could be starting point for LP-WUR. - FFS whether this value should be scaled-down based on the number of LP-WUS RBs.

In legacy spec, -25dBm is defined as the maximum mean power received at UE antenna port, which is applied on the whole allocated RBs, but LP-WUS only occupies 11 RBs within all the allocated RBs. Even if only one test case with 5MHz channel bandwidth is defined, -25dBm can only apply for 30kHz SCS in which case LP-WUS occupies all the allocated 11RBs. For RAN1’s latest conclusion that LP-WUS also occupies 11 RBs for15kHz SCS, scaled-down to LP-WUS RBs is still needed. In conclusion, -25dBm maximum input level should be scaled-down based on the number of LP-WUS RBs.

Proposal 1: -25dBm maximum input level should be scaled-down based on the number of LP-WUS RBs.

## 2.2 IBB and OBB

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies specified for IBB and OBB. LP-WUR and MR may share the same antenna and baseband processor, which means that LP-WUR and MR will suffer the same level of interference. To make sure that MR can be waken up timely as long as MR can work, same interference level of IBB and OBB as MR should be assumed for LP-WUR.

Proposal 2: Same interference level of IBB and OBB as MR should be assumed for LP-WUR.

Similar to ACS test, bandwidth of the interferer should be 5MHz and WUS should be located at edge of the NR carrier. As for the guard RBs, we can further discuss it after we have preliminary agreements on the test conditions for ACS.

Proposal 3: For IBB and OBB test, bandwidth of the interferer should be 5MHz and LP-WUS should be located at edge of the NR carrier. FFS guard RBs that are used for IBB and OBB.

Proposal 4: Consider the test parameters for IBB of LP-WUR in Table 1, Table 2 and the test parameters for OBB of LP-WUR in Table 3, Table 4. For these test parameters, the missed detection rate shall be ≤1%.

Table 1  In-band blocking parameters for LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LR REFSENS + 6 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 2  In-band blocking for LP-WUR

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | [-56] | [-44] |
| Finterferer (offset to LP-WUS)1 | MHz | -minimum guard band2-[guard RB]-FIoffset, case 1 and minimum guard band2+[guard RB]+FIoffset, case 1 | ≤ -minimum guard band2-[guard RB]-FIoffset, case 2 and ≥ minimum guard band2+[guard RB]+FIoffset, case 2 |
| NOTE 1: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). NOTE 2: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |  |

Table 3  Out-of-band blocking parameters for LP-WUR

| WUS transmission bandwidth configuration (RB) | Power in LP-WUS transmission bandwidth configuration [dBm] |
| --- | --- |
| 11RB | LR REFSENS + 6.0 dB |

Table 4  Out of-band blocking for LP-WUR

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pinterferer | dBm | [-44] | [-30] | [-15] |
| Finterferer (CW) | MHz | -60 < f - FDL_low < -15 or 15 < f - FDL_high < 60 | -85 < f - FDL_low ≤ -60 or 60 ≤ f -FDL_high < 85 | 1 ≤ f ≤ FDL_low -85 or FDL_high + 85 ≤ f ≤ 12750 |

## 2.3 Intermodulation Requirements

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively in TS38.101-1[2]. The missed detection rate shall be ≤1% under these assumptions. Similar to the discussion in Section 2.1, bandwidth of the interferer should be 5MHz and WUS should be located at edge of the NR carrier. As for whether guard RBs are needed, we can further discuss it after we have preliminary agreements on the test conditions for ACS.

Proposal 5: For intermodulation test, bandwidth of the interferer should be 5MHz and LP-WUS should be located at edge of the NR carrier. FFS guard RBs that are used for intermodulation requirements.

Proposal 6: Consider the test parameters for intermodulation of LP-WUR in Table 5. For these test parameters, the missed detection rate shall be ≤1%.

Table 5  Wide band intermodulation parameters for LP-WUR

| Rx parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| Pw in LP-WUS Transmission Bandwidth Configuration, per CC | dBm | LR REFSENS + 6 dB |
| PInterferer 1 (CW) | dBm | [-46] |
| PInterferer 2 (Modulated) | dBm | [-46] |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset to LP-WUS)2 | MHz | -minimum guard band3-[guard RB]-7.5 / minimum guard band3+[guard RB]+7.5 |
| FInterferer 2 (Offset to LP-WUS)2 | MHz | -minimum guard band3-[guard RB]-15 / minimum guard band3+[guard RB]+15 |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

## 2.4 Spurious Response

Spurious response is also used to evaluate the receiver capability when there is presence of an unwanted CW interfering signal on the frequency where out-of-band blocking limit is not met. Spurious response requirement is needed for LP-WUR.

Proposal 7: Consider the test parameters for spurious response of LP-WUR in Table 6 and Table 7. For these test parameters, the missed detection rate shall be ≤1%.

Table 6  Spurious response parameters for LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- | --- |
|  |  | 11RB |
| Power in LP-WUS transmission bandwidth configuration | dBm | LR REFSENS + 6 dB |

Table 7  Spurious response for LP-WUR

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | [-44] |
| FInterferer | MHz | Spurious response frequencies |

- Conclusion

Proposal 1: -25dBm maximum input level should be scaled-down based on the number of LP-WUS RBs.

Proposal 2: Same interference level of IBB and OBB as MR can be assumed for LP-WUR.

Proposal 3: For IBB and OBB test, bandwidth of the interferer should be 5MHz and LP-WUS should be located at edge of the NR carrier. FFS guard RBs that are used for IBB and OBB.

Proposal 4: Consider the test parameters for IBB of LP-WUR in Table 1, Table 2 and the test parameters for OBB of LP-WUR in Table 3, Table 4. For these test parameters, the missed detection rate shall be ≤1%.

Proposal 5: For intermodulation test, bandwidth of the interferer should be 5MHz and LP-WUS should be located at edge of the NR carrier. FFS guard RBs that are used for intermodulation requirements.

Proposal 6: Consider the test parameters for intermodulation of LP-WUR in Table 5. For these test parameters, the missed detection rate shall be ≤1%.

Proposal 7: Consider the test parameters for spurious response of LP-WUR in Table 6 and Table 7. For these test parameters, the missed detection rate shall be ≤1%.

- Reference

- R4-2417112 WF on UE RF requirements for LP-WUS, RAN4#112bis, Hefei, China, Oct 2024.

- TS38.101-1, v18.7.0