# 3GPP TSG-RAN WG4 Meeting #113	                 	   R4-2418846

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.13.2

Source: 		Qualcomm Incorporated

Title: 	Views on FR2 OTA test method on STxMP

Document for:	Approval

# Introduction

In RAN4#112bis, the WF on NR FR2 OTA in [1] was approved. In this paper, we provide our further considerations on the remaining issues for FR2 OTA test method on transmission with multi-panel (STxMP) testing.

# 2. Discussion

## EIRP PUMAX,f,c,k testing

With the confirmation from TE vendors, the baseline the baseline multi-RX/STxMP system is suitable to guarantee the same relative angular separations between specific UE test orientations and probes. Therefore, the default AoA separation and UE orientation for EIRP PUMAX,f,c,k testing was agreed as below [1].

Issue 1-1-1: Default AoA separation and UE orientation for EIRP PUMAX,f,c,k testing - Agreements: - RAN4 confirms the baseline multi-RX/STxMP system is suitable to guarantee the same relative angular separations between specific UE test orientations and probes. - The following test procedure can be considered as the solution to select default AoA separation and UE orientation EIRP PUMAX,f,c,k testing: - AoA1 direction for STxMP testing should re-use the legacy beam peak direction (from single carrier test) for the declared UE orientation (selected from any of the approved STxMP device orientations - For AoA1, no new BP search is necessary for device orientations used for STxMP test that have not been used for single carrier test - AoA2 direction is offset by AoA1 by the declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system

The remaining issues is whether a test direction other than the legacy beam peak direction without STxMP can be used for the first AoA, i.e., AoA1.

Issue 1-1-2: Whether a test direction other than the beam peak direction of single CC operation without STxMP can be used for first AoA - Agreements: - The test direction other than the beam peak direction of single CC operation without STxMP can be used for the first AoA if the test results based on the default AoA separation fail the limits in the spec. The following options can be considered: - Option 1: A local scan in xz plane is conducted. As long as test results do not pass the spec yet during scan, the local scan does not stop until AoA1 EIRP fail the legacy beam peak EIRP limit in both directions. FFS on the measurement grid of local scan. - Option 2: Alternative test directions on top of the default AoA separation can be declared by UE. FFS on the allowed offset of alternative test directions to the default test directions. - Option 1 and Option 2 only apply for EIRP PUMAX,f,c,k testing and will not have the impact on the EIRPmax and TRPmax test directions selection. - RAN4 to make down selection from Option 1 and Option 2 in next meeting

The intention of considering alternative test directions with local scan (option 1) or UE declaration is to provide more flexibility for UE implementation in case the peak EIRP per TCI is not always in line with legacy peak EIRP direction without STxMP. For Option 1, a local scan is in xz plan is necessary. In this case, the measurement grid of local scan also needs to specify for conformance test, therefore additional MU element might be added. In addition, it is possible that even with a local scarn, the proper test directions can not be found due to the limitation of test system. With above consideration, Option 2 is preferred as the alternative test directions as the backup of default test directions.

Proposal 1:  Take Option 2 as the baseline if the default AoA separation fail the limits in the spec. For option 2, the first AoA, i.e., AoA1 could be declared by UE. AoA2 direction is offset by AoA1 by the declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system.

## 2.2 Aggregated EIRP PUMAX,f,c testing

In [1], the additional margin for skipping rule for EIRPmax is agreed as follows.

Issue 1-2-2: Additional margin for skipping rule for EIRPmax testing - Agreements: - 0.5dB as an additional margin for EIRPmax test skipping rule.

We should note that the premise for skipping rule for EIRPmax and TRPmaxi is two layers are configured, i.e., one layer per TCI. That says 3.5dB margin needs to be guaranteed for skipping rule assuming two-layers with STxMP is configured. While when UE is configured by one layer with STxMP, the gain of EIRP at the test point with coherent phase would be up to 6dB.

Observation 1: When UE is configured by one-layer transmission with STxMP, the gain of EIRP at the test point with coherent phase would be up to 6dB.

Therefore, we have the following proposal.

Proposal 2: 3.5dB margin is used as the criteria for skipping rule for EIRPmax testing with two-layer transmission (i.e., one layer per TCI). 6.5dB margin criteria is applied for skipping rule for EIRPmax testing with one-layer transmission.

## 2.3 Aggregated TRP PTMAX,f,c testing

For Aggregated TRP PTMAX,f,c testing, Similar agreements were made in [1].

Issue 1-3-2: Additional margin for skipping rule for TRPmax testing - Agreements: - 0.5dB as an additional margin for TRPmax test skipping rule.

For TRPmax, it was agreed to consider 3dB + 0.5dB as the criteria for skipping rule in regulation test. This conclusion were assuming that TRP measurement at legacy EIRP peak direction would be with maximum value. While it is not always true for TRP measurement with 2AoA. For example, The TRP of antenna module 1 based on the peak EIRP direction may be smaller than the TRP of the other antenna module. The reason is that the antenna module with the best EIRP only indicates that the antenna module has the better gain, not the maximum TRP. This problem becomes more difficult to determine when using single-layer transmission. Therefore, we recommend FFS whether to apply skipping rule for TRPmax testing.

Proposal 3: RAN4 FFS whether to apply skipping rule for TRPmax testing. Both one-layer and two-layer transmission need to be considered.

## 2.4 Link polarization combinations

In RAN4#112bis, the link polarization combination issue was discuss and the following agreements were made.

Issue 1-4-1: Link polarization combinations - Agreements: - Further discuss the link polarization combinations for EIRP PUMAX,f,c,k, EIRPmax and TRPmax testing. - The following options can be considered: - Option 1: Limit the link polarization combinations for the 2AoA, and crossed link polarization (‘’ and ‘’) can be considered for the sake of lower interference - Other options are not precluded.

For EIRP measurement with 2AoAs, there are totally 4*4 polarization combinations. To simplify the test procedure, it was proposed to consider crossed link polarization (‘’ and ‘’). What we need to clarify here is that the combinations of the link polarizations (‘’ and ‘’) are equivalent to SSB transmission polarizations combinations from the TE in the measurement. In this case, we have the following proposal:

Proposal 4: Limit the link polarization combinations, i.e., SSB transmission polarizations in TE for the 2AoA EIRP measurement and crossed link polarization (‘’ and ‘’) can be considered.

# 3.	Conclusion

In this paper, we provide our considerations on test methodology for STxMP and have the following observations and proposals:

Proposal 1:  Take Option 2 as the baseline if the default AoA separation fail the limits in the spec. For option 2, the first AoA, i.e., AoA1 could be declared by UE. AoA2 direction is offset by AoA1 by the declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system.

Observation 1: When UE is configured by one-layer transmission with STxMP, the gain of EIRP at the test point with coherent phase would be up to 6dB.

Proposal 2: 3.5dB margin is used as the criteria for skipping rule for EIRPmax testing with two-layer transmission (i.e., one layer per TCI). 6.5dB margin criteria is applied for skipping rule for EIRPmax testing with one-layer transmission.

Proposal 3: RAN4 FFS whether to apply skipping rule for TRPmax testing. Both one-layer and two-layer transmission need to be considered.

Proposal 4: Limit the link polarization combinations, i.e., SSB transmission polarizations in TE for the 2AoA EIRP measurement and crossed link polarization (‘’ and ‘’) can be considered.

# 4.	References

- R4-2416580, WF on [112bis][325] NR_FR2_OTA, Qualcomm

- R4-2413534, WF on Rel-19 NR_FR2_OTA, Qualcomm

- TR 38.871, Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- 3GPP TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment

- R4-2411698, On FR2 OTA testing of STxMP, Samsung