# 3GPP TSG-RAN WG4 Meeting #113	R4-2418849

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.8.2.1

Source: 		Qualcomm Incorporated

Title: 	Discussion on coexistence study for NTN HPUE

Document for:	Approval

# Introduction

The new Rel-19 WI on enhanced requirements and test methodology for NR and IoT NTN was approved in [1] in RAN #103. In RAN4#112bis, coexistence study for NTN HPUE was discussed and the agreements were captured in WF [2]. The simulation results for NR NTN HPUE have been submitted in [3]. In this paper, we provide the coexistence simulation results for IoT NTN HPUE.

# 2. Discussion

## 2.1 Simulation results for NR NTN HPUE

For the coexistence between TN and NTN HPUE, it was agreed to focus on scenario 4 as the worst case [1].

Issue 1-2: Worst case scenario Agreement: - RAN4 to focus on scenario 4, and to skip scenario 5

For NR-NTN HPUE, we have submitted the simulation results in [3] and copied the simulation results in Table 1.

Note that in the below results, we did not consider 1.5km isolation distance. The wraparound functionality is disabled If the isolation distance is further considered, the Throughput loss will be further reduced.

Table 1 Required ACIR for Scenario 4 without wrap-around, no isolation distance for NR-NTN HPUE

| ACIR | 0 | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S4 avg –THP - no iso, WA disabled | 9.2 | 5.9 | 3.5 | 2 | 1.1 | 0.5 | 0.3 |
| S4 edge –THP - no iso WA disabled | 52.9 | 34.2 | 19.8 | 10.1 | 4.5 | 2.2 | 1.1 |

As shown in Table 1, when wrap-around is disabled, the required ACIR for Scenario 4 is less than 30dB even without 1.5km isolation distance.

Proposal 1: The ACLR of NR-NTN UE with PC3, i.e., 30dBc can be reused for NR-NTN HPUE PC2, PC1.5 and PC1 from coexistence perspective.

## 2.1 Simulation results for IoT-NTN HPUE

The simulation results for Scenario 4, IoT-NTN UL to TN UL with PC1 assumptions are provided in the Table 2. There are 2 cases for IoT-NTN simulations, one is with 12 UEs,15 kHz each and other one is with 36 UEs with 3.75 kHz each.

Table 2 Required ACIR for Scenario 4 without wrap-around, no isolation distance for IoT-NTN HPUE

| ACIR | 0 | 5 | 10 | 15 | 20 | 25 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S4 avg –THP - no iso WA disabled 12 UEs, 15kHz | 17.5 | 12.3 | 8 | 5.1 | 3 | 1.7 | 0.9 | - |
| S4 edge –THP - no iso WA disabled 12 UEs, 15kHz | 84 | 67.9 | 47.7 | 29.5 | 16.1 | 7.7 | 3.7 | - |
| S4 avg –THP - no iso WA disabled 36 UEs, 3.75 kHz | 22.9 | 16.8 | 11.7 | 7.7 | 4.8 | 2.8 | 1.5 | 1.2 |
| S4 edge –THP - no iso WA disabled 36 UEs, 3.75 kHz | 100 | 83.4 | 66.5 | 45 | 26 | 13.5 | 6.7 | 4.8 |

Note that, the flat ACLR model is used with FACLR = 0 dB, i.e. no BW scaling, which is same as ACLR modelling used in Rel-18 IoT-NTN coexistence study [4].

As shown in Table 2, when wrap-around is disabled, the required ACIR for Scenario 4 is less than 31dB even without 1.5km isolation distance.

Proposal 2: The ACLR of IoT-NTN UE with PC3, i.e., 37dBc can be reused for IoT-NTN HPUE PC2 and PC1 from coexistence perspective.

# 3.	Conclusion

In this paper, we provide our views on the coexistence study for NTN HPUE. The following proposals and observation are made:

Proposal 1: The ACLR of NR-NTN UE with PC3, i.e., 30dBc can be reused for NR-NTN HPUE PC2, PC1.5 and PC1 from coexistence perspective.

Proposal 2: The ACLR of IoT-NTN UE with PC3, i.e., 37dBc can be reused for IoT-NTN HPUE PC2 and PC1 from coexistence perspective.

# 4.	References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN

- R4-2417122, WF on co-existence study and HPUE for NR-NTN

- R4-2415307, On coexistence study for NTN HPUE

- R4-2217473, Way Forward on coexistence study for IoT over NTN