3GPP TSG RAN WG4 Meeting #113		 R4-2418854

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.6.2

Source: 	MediaTek Inc.

Title: 	Discussion on remaining open issues for DL fragmented carriers study

Document for:	Approval

1. Introduction

It had been discussed in previous RAN4 meeting that considering the spectrum hold be North America operators are usually non-contiguous blocks as stated in the reference [1][2][3]. In the RAN4#112bis, UE architecture assumption and initial evaluation based on existing test configurations were agreed and captured in the WF [4]. In this contribution, we continue discussion and provide our proposals on the open issues.

2. Discussion

2.1 Methods for reducing the number of UE Rx chains

Issue 2-3 Signaling related considerations

In the previous RAN4 meeting, there was remarkable progress on the DL fragmented carriers study. Baseline UE architecture, evaluation assumptions and test configuration assumptions were agreed. Signaling aspects, fallback behaviour and semi-static switching discussion were still open for further discussion.

For UE supporting DL NCCA with fully-shared RF Rx receiver, UE would be able to support higher order configurations. The UE capability report would need to inform network UE is capable for the CA combo with applying the new feature. Further, UE may also be capable to apply the new feature on more than one band. The signaling design shall consider these possible scenarios.

An extension of this new feature may be applying fully-shared RF Rx chain for inter-band CA with DL overlapping that has been existed in current configuration. However due to the operation condition for inter-band CA such as MRTD may usually larger than CP period, up to 33us, there would be problem on Rx AGC operation for fully-shared RF Rx chain architecture since AGC control of legacy UE is based on individual band.

Observation 1: Considering RX AGC operation of legacy UE, the fully-shared RF Rx architecture is only capable for intra-band and collocated fragmented carriers.

Proposal 1: Th new UE capability supporting fully-shared Rx chain for DL fragmented carriers is only applicable for intra-band and collocated carrier aggregation

Thanks to the company proposal in [5] for very good approaches for the new capability signalling considerations. For Rel-19 new UE supporting the new capability with adjusted RF requirements, it means the UE can support higher order CA configuration with the capability enabled. It would be nature the UE capability can be reported to network together with supported CA configurations with existing signalling framework. To minimize RAN2 signalling design effort, it would be nature to consider Alt 1.

Proposal 2: Alt. 1 : Legacy band combination reporting scheme + Rx sharing capability indication is preferred

Since the purpose for the fully-shared Rx feature is to support higher order CA configurations, it would be clear if UE can indicate whether is applies fully-shared Rx for intra-band DL NCCA per band in the highest order configuration reporting. UE may be possible to support the fully-shared Rx on multiple bands individually but this would also depends on baseband capability which is implementation dependent. Applying the new UE capability that RF parts can support higher order CA configurations with more CCs, the baseband processing capability may need to be extended to handle higher order CA combos including more DL carriers as well as more MIMO-layer processing capability.

Observation 2: Enabling DL fragmented carrier capability makes UE possible to support higher order configurations/MIMO layers from RF perspective. It does not guarantee baseband/modem has enough resources to support higher order configurations accordingly

Proposal 3: UE shall indicate whether is applies fully-shared Rx for intra-band DL NCCA per band in the highest order configuration reporting

Issue 2-4: Discussion on semi-statically switch hardware resources

Regarding the fallback behaviour, if UE can support CA configuration with enabling the new feature in the per-band DL NCCA UE capability report per BC, we need to consider fallback behaviour. If network configure fallback on the UE, there may be several possible fallback scenarios:

- For bands without mandatory 4Rx, the fallback configuration of the UE can be switching to non-CA of the band

- For bands with mandatory 4Rx, thanks to contribution in [6], there may be several possible fallback schemes illustrated and it is proposed to prioritize option 3 - dynamic switching between CA with Fully shared architecture and Non-CA for the band enabling fully-shared Rx.

It can be found that the common fallback behaviour is UE switch to non-CA for the band. And network would be able to configure UE and its fallback behaviour with existing scheduling/configuration flow.

Observation 3: The existing scheduling/configuration flow can be re-used for fallback control.

Proposal 4: For fallback approach, we propose prioritize UE be configured switching to non-CA for the band enabling fully-shared Rx

3. Conclusion

Issue 2-3 Signaling related considerations

Observation 1: Considering RX AGC operation of legacy UE, the fully-shared RF Rx architecture is only capable for intra-band and collocated fragmented carriers.

Proposal 1: Th new UE capability supporting fully-shared Rx chain for DL fragmented carriers is only applicable for intra-band and collocated carrier aggregation

Proposal 2: Alt. 1 : Legacy band combination reporting scheme + Rx sharing capability indication is preferred

Observation 2: Enabling DL fragmented carrier capability makes UE possible to support higher order configurations/MIMO layers from RF perspective. It does not guarantee baseband/modem has enough resources to support higher order configurations accordingly

Proposal 3: UE shall indicate whether is applies fully-shared Rx for intra-band DL NCCA per band in the highest order configuration reporting

Issue 2-4: Discussion on semi-statically switch hardware resources

Observation 3: The existing scheduling/configuration flow can be re-used for fallback control.

Proposal 4: For fallback approach, we propose prioritize UE be configured switching to non-CA for the band enabling fully-shared Rx

Reference:

- RP-242404 New SID: Study on NR FR1 DL Fragmented Carriers, RANP#105

- RP-233374, “Fragmented carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2402309: “Study on effective utilization of fragmented FR1 carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2417201 WF on fragmented DL carriers study, RAN4#112bis

- R4-2415510 On methods for reducing the number of UE Rx chains for fragmented carriers, Apple, RAN4#112bis

- R4-2415804 On methods for reducing the number of UE Rx chain, Huawei, RAN4#112bis