3GPP TSG RAN WG4 Meeting#113                                    R4-2418860

Orlando, US, 18th - 22nd November 2024

Title:	RRM Core requirements on CSI-RS based L1 measurement for LTM

Source:	China Telecom

Agenda item:			7.24.2.2

Document for:	Discussion

- Introduction

In the RAN4 #112bis meeting, the measurements related enhancements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM Core requirements on CSI-RS based L1 measurement for LTM.

- Discussion

Issue 2-3: CSI-RS based L1 measurement outside active BWP or with measurement gap

- Candidate solutions - Option 1: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. (MTK, QC, vivo, OPPO, Nokia) - Option 1a: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported. (Xiaomi) - Option 1b: RAN4 to define intra-frequency CSI-RS measurement requirements first. Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap. (Nokia) - Option 2: RAN4 at least support the following measurement types for CSI-RS based L1 measurement. (CATT) - Without gap - With gap - Option 2a: for CSI-RS based LTM, it is proposed to consider intra-frequency measurement, inter-frequency measurement without gap and inter-frequency measurement with gap, which are aligned with Rel-18 SSB based LTM. (CMCC) - Option 2b: prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. (Apple, Samsung, HW, OPPO, CATT) - Option 2c: CSI-RS based L1 measurements for candidate cells shall support - Measurement within active BWP, Measurements outside active BWP. (E///, ZTE)

In Rel-18, we have discussed intra-frequency measurement without GAP, inter-frequency measurement with GAP and inter-frequency measurement without GAP for LTM. For CSI-RS based L1 measurement, we think both intra-frequency measurement and inter-frequency measurement are important. Since intra-frequency measurement without GAP and inter-frequency measurement with GAP are typical, we could prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP, FFS inter-frequency measurement without GAP.

Proposal 1: Prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. FFS inter-frequency measurement without GAP.

Issue 2-4: whether SSB based L3 measurement is a pre-requisite of CSI-RS based L1 measurement

Tentative Agreement during online session: further discuss the two alternatives Alternative 1: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1. Alternative 2: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

For either FR1 or FR2, UE needs to perform SSB based L3 measurement on the neighbour cells firstly to obtain the coarse timing and SSB index information. So SSB based L3 measurement can be as the pre-requisite condition of configured CSI-RS L1 measurement.

For the above two alternatives, the differences are whether UE performs SSB-based L1 measurements and reports on the same candidate cell before CSI-RS based L1 measurements. In our opinion, the SSB-based L1 measurements and reports can be skipped under certain conditions. So, we prefer alternative 1, and the details of the conditions where SSB-based L1 measurements and reports can be skipped should be discussed further.

Proposal 2:

Alternative 1:

    - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements:

      - Step 1: SSB based L3 measurements [and reports]

      - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

        - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions.

        - Note: Step 2 is removed for FR1.

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

      - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

        - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

Issue 2-12: RTD assumption

- Further discuss based on the following assumption: - RTD is the received time difference among CSI-RS and SSB from different cells. - Alt 1: define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP. - Alt 2: only define requirements for RTD<CP. - NW shall only enable this when RTD<CP is guaranteed.

If restrict the RTD between serving cell and neighbour cell to be within CP, some cases with higher SCS are also restricted. So we think it better that support RTD>CP case for CSI-RS based L1-RSRP measurement.

Proposal 3: Support RTD>CP case for CSI-RS based L1-RSRP measurement.

Issue 2-13: category of CSI-RS based L1 RSRP measurement

- Candidate solutions: - Option 1: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion (MTK, CATT, E///, vivo) - Option 2: Categorize CSI-RS based L1-RSRP measurement into intra-frequency and inter-frequency (CATT, CTC, CMCC, Apple, Samsung, HW, OPPO, Nokia, ZTE)

For this issue, we prefer to follow the similar principle of CSI-RS based L3 measurement. So, option 2 is OK for us.

Proposal 4: Categorize CSI-RS based L1-RSRP measurement into intra-frequency and inter-frequency.

Issue 2-14: definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement

- Candidate solutions: - Option 1: Following CSI-RS based L3 measurement (CTC, CMCC, Apple, Samsung, OPPO, [Nokia?]) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: For definition of intra-frequency and inter-frequency, suggest to wait for more progress in other WGs. (Xiaomi) - Option 3: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy. (CATT) - One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell. - Option 4: Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency. (vivo) - Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules - No additional definition of intra-F/inter-F for CSI-RS based L1 measurements. - Option 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell. (HW) - It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

For alignment of the spec, we prefer to follow the definition on CSI-RS based L3 measurement. So option 1 is OK for us.

Proposal 5: Following CSI-RS based L3 measurement

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

  - It is applied for SCS = 60KHz

- the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: Prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. FFS inter-frequency measurement without GAP.

Proposal 2:

Alternative 1:

    - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements:

      - Step 1: SSB based L3 measurements [and reports]

      - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

        - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions.

        - Note: Step 2 is removed for FR1.

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

      - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

        - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

Proposal 3: Support RTD>CP case for CSI-RS based L1-RSRP measurement.

Proposal 4: Categorize CSI-RS based L1-RSRP measurement into intra-frequency and inter-frequency.

Proposal 5: Following CSI-RS based L3 measurement

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

  - It is applied for SCS = 60KHz

- the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

- References

[1] R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple Inc.