3GPP TSG RAN WG4 Meeting #113		 R4-2418862

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.6.3

Source: 	MediaTek Inc.

Title: 	Discussion of impacts on UE RF requirements and DL performance impacts

Document for:	Approval

1. Introduction

It had been discussed in previous RAN4 meeting and WF[4] was agreed. Open issues were also captured in the WF. In this meeting, we continue the discussion and provide our views and discussion points on UE RF requirements and DL performance impacts.

2. Discussion

2.1 Observations of impacts on DL Rx requirements

In last meeting, some baseline assumptions were agreed in [4] and copied below:

(a)	RAN4 agree on the general assumptions that are same for legacy NR RF performance evaluation:

- UE self-band uplink Tx re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB

- Reuse the Rx chain image rejection, which is 25dB, as the starting point.

(b) The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC, REFSENS, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos

The architecture block diagram for fully-shared receiver was also agreed as below,

Figure 1. Fully-shared Rx RF chain

It was also agreed to consider n2/n25, n3, n7, n66, n41, n26 as the example bands. So we choose n25 to start the study with. With above agreement, let’s look at the three test configurations in existing requirements. The Rx architecture assumption of existing requirements is based on partially-shared Rx chains. It is foreseen both CCs may have good performance w.r.t ACS/IBB/NBB as well as rejection to self-band UL transmission interference. If the Rx chain is shared by the two non-contiguous CCs, (fully-shared Rx chain), the channel selection filter would need to be adjusted to wider bandwidth to accommodate the two DL carriers including Wgap between the two CCs. The Tx rejection of shared Rx chain would be much worse. Not only SCC would still impacted by self-band UL Tx, PCC may also be impacted due to worse filter rejection ratio toward UL Tx transmission. It is foreseen the DL AGC would need to be adjusted to tolerate larger Tx leakage portion thus the gain setting would become lower that brings noise level raised up for both PCC and SCC. Another potential degradation may happen on in-gap ACS/IBB/NBB performance, these requirements would also need to be evaluated.

Figure 2. Rx requirements to be evaluated

Figure 3. Illustration on the two BCS for DL NCCA test

Observation 1: Not only SCC is impacted by self-band UL Tx, PCC may also be impacted by it due to channel selection filter would be adjusted to wider total bandwidth in fully-shared Rx chain architecture and RX AGC would be adjusted for larger Tx leakage portion. The Rx noise level would be raised up for both PCC and SCC

Proposal 1: Impacts on DL performance shall also consider degradation on PCC due to wider channel selection filter configured under fully-shared Rx chain architecture.

Let’s take an initial inspection on the potential performance impact with the agreed assumptions. It can be seen for example, a DL NCCA composed of 40+5MHz bandwidth combination, when testing in-gap ACS/IBB1/IBB2 on CC0, if BW of CC0 is larger than CC1, the image of in-gap interference may also fall into co-channel of CC0 that result in performance degradation as illustrated in Figure 4. The agreed assumption of Rx chain image rejection is 25dB so for the cases image of ACS/IBB1/IBB2/NBB overlaps with wanted CC, the maximum image to wanted signal power ratio shall not be larger than 25dB.

Proposal 2: For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC, the maximum power level of aggressor to wanted signal power ratio shall not be larger than 25dB

On the other hand, when measuring ACS/IBB1/IBB2 of CC0, the image of interference may fall into the co-channel range of CC1. Though this may not be an issue for the conformance test since the conformance test does measure REFSENS of CC1 when testing ACS/IBB1/IBB2 of CC0, this may be an implementation issue in the field as illustrated in Figure 5.

Figure 4. Illustration on impacts of the unwanted/interference image fall into co-channel CC

Figure 5. Illustration on impacts of the unwanted/interference image fall into the other CC

The existing requirements of n25(2A) is copied below:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |

Consider the PCC+SCC = 5+5MHz case, according to existing requirements based on partially-shared receiver architecture, PCC_sensitivity = -96.5dBm, SCC_sensitivity = -91.5dBm(RIBNC   = 5dB). This means there’s effectively ~ -93.15dBm extra Tx noise fall in SCC.

For fully-shared Rx chain,

Assume 4dB FE loss and 50dB Tx-Rx isolation

Wanted signal at LNA input = -96.5-4= -100.5dBm

To evaluate impact on ACS/IBB performance, we can assume Tx leakage at LNA input=27-4-50= -27dBm. We also assume Tx leakage level after down conversion is the same with blocking interference level, the worst case is in-gap blocking that has no filter rejection can help thus it can only be covered by ADC dynamic range. We try to calculate rejection level that LPF has to provide below:

ACS interference to signal ratio = 31.5dB

TX leakage to signal ratio = -27 – (-100.5+14) = 59.5dB

LPF rejection_ACS = 59.5-31.5 = 28dB.

IBB2 TX leakage to signal ratio = -27+100.5=73.5dB

IBB2 interference to signal ratio = -44 - (-96.5+6) = 46.5 ISR

LPF rejection_IBB2 = 73.5-46.5=27dB

From UE implementation, to provide such high filter rejection in RFIC would be quite difficult so it would need the help of digital filter after ADC. Before ADC, the RF Rx chain need to maintain linearity under such large Tx leakage + ACS level and small Tx-Rx frequency separation gap (15MHz only) thus sufficient receiving chain gain reduction would be necessary. The RX noise would be raised due to above mentioned reasons that RIBNC_PCC_5M = 13.1dB. REFSENS_scc= (-96.5dBm + 13.1dB) + -93.15dBm = -83dBm. So RIBNC_SCC_5M = 13.5dB so that the ACS requirements can be maintained. With the calculated RIBNC_PCC_5M and RIBNC_SCC_5M, the interference level of IBB1 and IBB2 is comparable with that of ACS case 1 and case 2, so there is also no need to relax IBB1 and IBB2 requirements.

For 40+5 MHz case, the 15KHz SCS REFSENS of existing spec for 40MHz = -79.5dBm. Total span of the baseband LPF would be same with 5+5 MHz case thus same receiving chain gain reduction is also applied. The calculated RIBNC_PCC_40M = 6.1dB, RIBNC_SCC_5M = 25.2dB. Adjusted requirements would look like the below table:

Observation 2:

For PCC=5MHz, SCC=5MHz, the fully-shared Rx chain may need RIBNC_PCC_5M = 13.1dB. RIBNC_SCC_5M = 13.5dB

For PCC=40MHz, SCC=5MHz, the fully-shared Rx chain may need RIBNC_PCC_40M = 6.1dB. RIBNC_SCC_5M = 25.2dB

Observation 3: ACS/IBB1/IBB2 can maintain same interference to wanted signal ratio with RIBNC_PCC and RIBNC_SCC in above observation.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 10 | 13.1 | 13.5 | FDD |
|  |  |  | Wgap = [TBD] | 25 | 0 | 0 |  |
| CA_n25(2A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | 6.1 | 25.2 | FDD |

Table 1. Adjusted requirement when enabling One Rx RF Chain

Proposal 3: Companies are encouraged to provide evaluations on RIBNC_PCC and RIBNC_SCC and impacts on ACS/IBB1/IBB2

3. Conclusion

2.1 Observations of impacts on DL Rx requirements

Observation 1: Not only SCC is impacted by self-band UL Tx, PCC may also be impacted by it due to channel selection filter would be adjusted to wider total bandwidth in fully-shared Rx chain architecture and RX AGC would be adjusted for larger Tx leakage portion. The Rx noise level would be raised up for both PCC and SCC

Proposal 1: Impacts on DL performance shall also consider degradation on PCC due to wider channel selection filter configured under fully-shared Rx chain architecture.

Proposal 2: For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC, the maximum power level of aggressor to wanted signal power ratio shall not be larger than 25dB

Observation 2:

For PCC=5MHz, SCC=5MHz, the fully-shared Rx chain may need RIBNC_PCC_5M = 13.1dB. RIBNC_SCC_5M = 13.5dB

For PCC=40MHz, SCC=5MHz, the fully-shared Rx chain may need RIBNC_PCC_40M = 6.1dB. RIBNC_SCC_5M = 25.2dB

Observation 3: ACS/IBB1/IBB2 can maintain same interference to wanted signal ratio with RIBNC_PCC and RIBNC_SCC in above observation.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 10 | 13.1 | 13.5 | FDD |
|  |  |  | Wgap = [TBD] | 25 | 0 | 0 |  |
| CA_n25(2A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | 6.1 | 25.2 | FDD |

Table 1. Adjusted requirement when enabling One Rx RF Chain

Proposal 3: Companies are encouraged to provide evaluations on RIBNC_PCC and RIBNC_SCC and impacts on ACS/IBB1/IBB2

Reference:

- RP-242404 New SID: Study on NR FR1 DL Fragmented Carriers, RANP#105

- RP-233374, “Fragmented carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2402309: “Study on effective utilization of fragmented FR1 carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2417201 WF on fragmented DL carriers study, RAN4#112bis