3GPP TSG-RAN WG4 Meeting #113	R4-2418896

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	7.1.1.3.4

Source:	Intel Corporation

Title:	Views on SRS insertion loss compensation and reporting enhancements

Document for:	Discussion

# Introduction

The Rel-19 WID on UE RF enhancements for NR FR1/FR2 and EN-DC Phase 4 was approved in RAN#103 with the following objectives related to 6Rx for handheld and FWA UEs [1]:

### 4.1 Objective of Core part WI The objectives of core part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: … 6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution. …

The topics of SRS insertion loss imbalance was discussed in previous meetings with agreements provided in [2-5] and summarized in Annex.  In this contribution we provide further views on the topic.

# Discussion

## Legacy UE behavior for SRS insertion loss compensation

Based on Rel-18 discussions in RAN1/4 there was no clear understanding among companies on the anticipated UE behavior in terms of SRS IL compensation during SRS transmissions with the following options mentioned:

- Option 1: UE does not make Tx power imbalance compensation.

- Option 2: UE performs Tx power imbalance compensation up to its maximum capabilities in non-power limited conditions (i.e., applies power compensation in case there is sufficient power to do so). For example, in case of 20dBm configured Tx power and 8dB SRS IL, the UE should be capable to compensate 3dB loss in case of 23dBm maximum output power.

- Option 3: Keep up to UE implementation and UE can choose whether and when to apply compensation.

The legacy UE behavior can be understood based on current specifications of the SRS power control mechanism in TS 38.213 and based on configured maximum output Tx power in TS 38.101-1.

The SRS transmission power in TS 38.213 is defined in a way that unless the SRS transmission power exceeds the configured maximum output power PCMAX,f,c limit (defined in TS 38.101-1), the actual SRS transmission power depends on L1 power control parameters only such as p0 for active UL BW, SRS bandwidth, pathloss and others. It can be observed that the SRS insertion loss is not taken into consideration, and it may be understood that UE is required to compensate any SRS insertion loss as long as the total power is below PCMAX,f,c (as SRS IL is not present in equations).

The UE configured maximum output power parameter PCMAX,f,c is defined in TS 38.101-1. The effect of SRS insertion loss is accounted in the PCMAX via the parameter ∆TRxSRS, which reflects the worst-case implementation in terms of SRS insertion loss (i.e., upper bound). It can be observed that PCMAX is not a pre-defined fixed value and represents a certain range of values, which gives flexibility to possible UE implementations.

PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass }

Observation #1:	The legacy UE behavior is based on current specifications of the SRS power control mechanism in TS 38.213 and based on configured maximum output Tx power in TS 38.101-1.

Let’s consider a simplified example of SRS power control equations under an assumption that all factors reducing PCMAX except for SRS IL are equal to 0 dB. The equations can be represented in the following form:

PSRS = min(PCMAX, PSRS,PC) PSRS,PC = PCMAX_L ≤ PCMAX ≤ PCMAX_H PCMAX_L = PPowerClass – ∆TRxSRS PCMAX_H = PPowerClass

In the Figure 1 below we provide an illustration of the SRS transmission power levels and effective SRS power imbalance between ports as a function of PSRS,PC under different assumptions of PCMAX selection (equal to either PCMAX,H or PCMAX,L) based on the equations above for the following cases

- Case A: ∆TRxSRS = 7dB, ∆TRxSRS,UE  =  7dB (UE-specific value is same as the spec value)

- Case B: ∆TRxSRS = 7dB, ∆TRxSRS,UE  =  3dB (UE-specific value is better than the spec value)

The following curves are illustrated in the figures:

- “P0” – SRS transmission power for the main SRS AP without SRS IL

- “P1” – SRS transmission power for the diversity SRS AP with SRS IL

- “P1,EFF” – SRS transmission power for the diversity SRS AP before applying SRS IL

- “P0-P1” – SRS transmission power imbalance between SRS AP with and without SRS IL (P0-P1)

Figure 1. SRS transmission power levels and effective SRS power imbalance

Based on the analysis above we make the following observations:

- Full SRS IL compensation is expected to be performed at least for power levels PSRS,PC ≤ PCMAX,L

- The existing definition of PCMAX implies that UE performs at least partial SRS IL compensation in the power range of PCMAX,L < PSRS,PC < PCMAX,H

- For Case A when the effective SRS IL is same as the min requirement (∆TRxSRS = ∆TRxSRS,UE) the effective SRS AP power imbalance does not depend on the selection of PCMAX level

- For Case B when the effective SRS IL is better than the min requirement (∆TRxSRS,UE <∆ TRxSRS,UE)

  - The effective SRS AP power imbalance may depend on the selection of PCMAX level

  - UE may underutilize PA capabilities and not perform SRS IL compensation up to its maximum capabilities in case PCMAX=PCMAX,L

To summarize, the UE behavior may depend on the specific SRS transmission power:

- Case 1 (non near max Tx power): In this scenario the SRS transmission power (PSRS) is below PCMAX,L. Based on TS 38.213 the UE is required to compensate any insertion loss and no further changes in the specification are needed.

- Case 2 (near max Tx power): In this scenario the actual required SRS transmission power is higher than PCMAX_L,f,c, but is still below the max transmission power PCMAX_H,f,c (PCMAX,L < PSRS < PCMAX,H). In this case the UE behavior is not fully defined and UE may or may not necessarily perform SRS IL compensation depending on specific SRS IL for this UE and chosen PCMAX,f,c value.

- Case 3 (max Tx power): In this scenario the actual required SRS transmission power is equal to PCMAX_H,f,c. In this case UE is not capable to perform any SRS IL compensation.

Observation #2:	For Case 1 (non near max Tx power) scenario the UE is supposed to always compensate any SRS insertion in SRS transmissions based on TS 38.213.

Observation #3:	For Case 2 (near max Tx power) scenario the UE behavior in terms of SRS IL is not fully specified and UE may or may not necessarily be required to perform SRS IL compensation up to its maximum power capabilities based on TS 38.101-1.

Proposal #1:	Clarify the expected legacy UE behavior for SRS IL compensation in Rel-19 timeframe as follows.

- For non near max Tx powe) scenario the UE is supposed to always compensate any SRS insertion in SRS transmissions based on TS 38.213.

- For near max Tx power scenario the UE behavior in terms of SRS IL is not fully specified and UE may or may not necessarily be required to perform SRS IL compensation up to its maximum power capabilities based on TS 38.101-1.

During RAN4 #112bis discussions it was mentioned that some of existing UEs may not necessarily follow the legacy UE behavior explained in 2.1. Given that there is no common understanding on the existing UE behavior, it is recommended to introduce the corresponding requirements/testing framework to ensure proper UE implementations for legacy UEs. The following framework is considered:

- Goal: Ensure that UE following existing RAN1/4 power control equations for SRS transmissions and provides equal power transmission across different transmissions and across Tx antennas.

- Ideally the requirements shall be introduced from early releases

- The testing shall cover a wide range of output transmit power configurations. At least scenarios with PSRS,PC ≤ PCMAX,L   shall be covered.

Proposal #2:	Introduce SRS output power testing framework in Rel-19 timeframe to guarantee proper UE behavior based on existing specifications.

## Enhancements for SRS insertion loss compensation

In RAN4 #112bis the following agreements on SRS IL compensation solutions were reached:

Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue - Candidate solutions - Option 1: - UE performs self-compensation of SRS IL up to its maximum Tx power capabilities - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - Details of SRS IL self-compensation and assistance framework are FFS - Other options are not precluded

First, it is important to confirm the RAN4 #112bis candidate solution regarding UE behavior, specifically that "UE performs self-compensation of SRS IL up to its maximum Tx power capabilities." The respective principle is the basis of all proposed enhanced solutions and should serve as the baseline for any Rel-19 solutions under consideration.

Proposal #3:	Confirm the Option 1 candidate solution and specify solutions, UE behavior, and requirements for scenarios where UE has sufficient power to compensate the power imbalance and require UE to perform SRS IL pre-compensation up to the maximum power capabilities.

Regarding signalling aspects, the following general approaches can be considered for the types of UE assistance provided considering proposals in RAN4 #112bis:

- Alt 1: The UE reports information on SRS insertion loss (or power imbalance) used for SRS transmissions to the network.

- Alt 2: The UE reports the configured maximum output power per SRS resource and the respective power headroom transmissions to the network.

Both mechanisms have the potential to address the technical issue and enable the network to obtain more precise information about the actual SRS transmission power imbalance between chains. In our view, the Alt 1 approach offers reasonably good flexibility and can accommodate both dynamic and static assistance use cases, whereas the Alt 2 approach would require dynamic power headroom reporting.

For the Alt 1 approach, the following mechanism is suggested to introduce the UE SRS IL pre-compensation mechanism in RAN4 specifications. As explained in section 2.2, when the effective SRS IL (∆TRxSRS,UE) is better than the minimum requirement (∆TRxSRS,UE < ∆TRxSRS), the UE may underutilize PA capabilities and not perform SRS IL compensation up to its maximum capabilities if PCMAX < PCMAX,H. To ensure that the UE performs more optimal SRS IL pre-compensation, it is possible to adjust the existing SRS PCMAX equations on PCMAX_L,f,c so that the UE applies compensation considering the UE-specific SRS IL value ∆TRxSRS,UE:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

The information on the UE SRS IL (∆TRxSRS,UE) should be provided to the network and may include either a static value (maximum SRS IL for the given UE) or more dynamic information on the actual SRS IL used by the UE. Static signalling results in a simplified UE implementation but does not provide NW sufficient information on the actual SRS power imbalance. Dynamic reporting of SRS IL imposes extra overhead and UE complexity but would allow NW to perform post-compensation of actual SRS power imbalance. Both options have their pros and cons but do not contradict each other. Therefore, it is suggested to consider both static and dynamic indications of SRS IL information as complementary solutions. The information can be provided with per SRS resource granularity.

Proposal #3:	Confirm the RAN4 #112bis Option 1 candidate solution and specify solutions, UE behavior, and requirements for scenarios where UE has sufficient power to compensate the power imbalance and require UE to perform SRS IL pre-compensation up to the maximum power capabilities.

Proposal #4:	Introduce UE assistance signalling on the information on the UE SRS IL (∆TRxSRS,UE).

    - Consider both static and dynamic reporting of SRS IL information

    - SRS IL information is provided with per SRS resource granularity

Proposal #5:	Adjust the existing equations on PCMAX_L,f,c so that UE applies compensation up to UE-specific SRS IL

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

Other aspects

The considered framework is generic, and we do not see a motivation to limit it to 8Rx cases only. It is recommended to make it applicable to all 2Rx/4Rx/6Rx/8Rx cases.

Furthermore, given the potential impact on UE implementation it is recommended to introduce new Rel-19 features as optional.

Proposal #6:	Introduce SRS IL compensation mechanisms as an optional feature in a generic manner for 2Rx/4Rx/6Rx/8Rx UEs.

# Conclusion

In this paper we provide views on the insertion loss imbalance across SRS ports. In summary, the following proposals are made:

Proposal #1:	Clarify the expected legacy UE behavior for SRS IL compensation in Rel-19 timeframe as follows.

- For non near max Tx powe) scenario the UE is supposed to always compensate any SRS insertion in SRS transmissions based on TS 38.213.

- For near max Tx power scenario the UE behavior in terms of SRS IL is not fully specified and UE may or may not necessarily be required to perform SRS IL compensation up to its maximum power capabilities based on TS 38.101-1.

Proposal #2:	Introduce SRS output power testing framework in Rel-19 timeframe to guarantee proper UE behavior based on existing specifications.

Proposal #3:	Confirm the RAN4 #112bis Option 1 candidate solution and specify solutions, UE behavior, and requirements for scenarios where UE has sufficient power to compensate the power imbalance and require UE to perform SRS IL pre-compensation up to the maximum power capabilities.

Proposal #4:	Introduce UE assistance signalling on the information on the UE SRS IL (∆TRxSRS,UE).

    - Consider both static and dynamic reporting of SRS IL information

    - SRS IL information is provided with per SRS resource granularity

Proposal #5:	Adjust the existing equations on PCMAX_L,f,c so that UE applies compensation up to UE-specific SRS IL

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

Proposal #6:	Introduce SRS IL compensation mechanisms as an optional feature in a generic manner for 2Rx/4Rx/6Rx/8Rx UEs.

# References

- RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#103, March 2024

- R4-2406585 “WF on requirements for 6Rx”, AT&T, RAN4#110bis, April 2024

- R4-2410751 “WF on requirements for 6Rx”, AT&T, RAN4#111, May 2024

- R4-2414285 “WF on requirements for 6Rx”, AT&T, RAN4#112, August 2024

- R4-2417209 “WF on requirements for 6Rx”, AT&T, RAN4#112bis, October 2024

# Annex: Previous agreements

The RAN4 #110bis agreements are captured in WF [2]:

## Sub-topic 4-1: General considerations for SRS IL imbalance issue Way forward: Further discuss the topic of SRS IL imbalance issues with the following proposals used as a starting point. Whether a solution is agreeable, depends on further discussion of the following candidate solutions and/or identified issues. - Proposal 1: Study the issues of insertion loss imbalance across SRS ports. - Proposal 2: Further discuss SRS IL for 6RX type of devices taking into consideration: - SRS insertion loss requirements for 6RX UEs - SRS IL impact on performance - SRS insertion loss compensation - UE assistance on SRS insertion loss (power imbalance) - Others - Proposal 3: Companies should, independent of their view, provide constructive technical inputs on SRS TX power imbalance topic. ## Sub-topic 4-2: Candidate solutions for the SRS IL imbalance issue Way forward: Further discuss the following options considering the UE behaviour in terms of the IL imbalance compensation and whether the UE can compensate the IL imbalance for AS-SRS for all power levels and whether NW and UE have consistent understanding for the possible compensation. - Option 1: Specify UE behaviour and requirements for scenarios, when UE has sufficient power to compensate the power imbalance (Case 2) and require UE to perform SRS IL compensation up to the maximum power capabilities. - Case 1 (non near max Tx power): In this scenario the SRS transmission power (PSRS) is below PCMAX_L,f,c. Based on TS 38.213 the UE is required to compensate any insertion loss and no further changes in the specification are needed. Per discussion during RAN4 meetings there seem to be no clear understanding whether all existing UEs do perform the SRS IL compensation under these conditions. So, it is recommended to consider specific requirements / conformance requirements in future to guarantee proper UE implementations. - Case 2 (near max Tx power): In this scenario the actual required SRS transmission power is higher than PCMAX_L,f,c, but is still below the max transmission power PCMAX_H,f,c. Based on the interpretation of TS 38.101-1 above, in this case the UE behaviour is undefined and UE may or may not perform SRS IL compensation. - Case 3 (max Tx power): In this scenario the actual required SRS transmission power is equal to PCMAX_H,f,c. (i.e. hypothetical scenario). In this case UE is not capable to perform any SRS IL compensation. - Option 2: At the current stage, the SRS IL imbalance does not affect the practical system performance, nor any enhancement to resolve the issue would work effectively. - Option 3: Not specifying reporting of SRS IL offsets due to IL imbalance. - Option 4: IL imbalance reporting mechanism for SRS AS including both static reporting and dynamic reporting. - Static reporting is up to UE implementation, and UE needs to indicate the power compensation behaviour to NW if UE reports statically. - Dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern, and several thresholds associated with capability class for the actual SRS IL reporting can be considered. - Option 5: IL imbalance reporting mechanism for SRS AS including both the configured maximum output power per SRS resource and the power headroom per SRS resource. - The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission. - Other options are not precluded. Consider some clarifications according to the RAN1 and RAN4 specs. - RAN1 specification on power control doesn’t require or imply that UE shall apply the same PCMAX for all AS-SRS transmission occasions - Under the minimum requirement ∆TRxSRS defined in RAN4 specification, it is up to UE implementation whether to apply exact value for corresponding AS-SRS transmission occasions.

The RAN4 #111 agreements are captured in WF [3]:

Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation. Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: After determining the set of initial considerations for the study, discuss candidate solutions considering the UE behaviour in terms of the IL imbalance compensation and whether the UE can compensate the IL imbalance for AS-SRS for all power levels and whether NW and UE have consistent understanding for the possible compensation.

The RAN4 #112 agreements are captured in WF [4]:

Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue dependent on the outcome of Issue 4-1-2 in the approved WF in R4-2410751. Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation. Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: Companies encouraged to provide compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain.

The RAN4 #112bis agreements are captured in WF [5]:

Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 4-2-1. Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: RAN4 to focus on the compromised solution proposals in Issue 4-2-1. Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: Companies are encouraged to provide comments and indication of support for the solution proposals listed below by RAN4#113. Proponent companies (in parentheses) for the candidate solution and solution proposals below are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed. Other proposals including expected performance gains and draftCRs are not precluded. - Candidate solutions - Option 1: - UE performs self-compensation of SRS IL up to its maximum Tx power capabilities - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - Details of SRS IL self-compensation and assistance framework are FFS - Other options are not precluded - Proposals - Proposal 1: Only introduce UE compensation and new accurate SRS power headroom reporting for non-CA/DC scenarios can be a compromised solution. Further study the granularity of new SRS power headroom report (China Telecom) - Proposal 2: IL imbalance reporting mechanism for SRS AS should include the configured maximum output power per SRS resource, the power headroom per SRS resource and ΔPPowerClass . The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission (Ericsson). - Proposal 3: Another way to resolve the SRS IL imbalance reporting issue would be to introduce two types of reporting for a UE: a “baseline” and an “advanced” reporting. The baseline reporting would not require any calibration at the UE and it could include e.g. the PHR and ΔPPowerClass information. The advanced reporting could in addition include the information on e.g. the configured maximum output power (Ericsson). - Proposal 4: Solution for IL imbalance issue as identified below. Reporting power threshold needs to be considered for static and dynamic reporting. The granularity of reporting could be per band (Spreadtrum). - If UE reports statically, report the actual SRS insertion loss with no UE self-compensation - If UE reports dynamically, UE report the difference value of each diversity branch output power to NW according to the SRS period (including periodic, semi-persistent and aperiodic) in real time - Proposal 5: Introduce semi-static UE assistance or capability signalling on the information on the UE SRS IL (∆TRxSRS,UE) (Intel). Adjust the existing equations on PCMAX_L,f,c so that UE applies compensation up to UE-specific SRS IL PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) } - Proposal 6: Given that specification defines the Tx power should be equally distributed across SRS ports for each SRS transmission, per SRS resource IL reporting would be sufficient (Huawei, HiSilicon). - Proposal 7: If dynamic reporting can be considered for SRS IL reporting, network configurable threshold related to e.g. historical change of PSRS can be considered in order to give the network authority for handling SRS IL reporting frequency (Huawei, HiSilicon). - Proposal 8: To keep the current UE implementation untouched and avoid specification impact to most extent, following solution can be considered in Rel-19 (Huawei, Hi Silicon): - UE is allowed to indicate whether it enables self-compensation on the SRS IL once the network requests such information, which would benefit the network by adjusting expectation on the antenna switching SRS based PMI estimation. - Proposal 9: Dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern, and several thresholds associated with capability class for the actual SRS IL reporting can be considered. (ZTE, Sanechips) - Proposal 10: For Rel-19 UE, at least, optionally indicates self-compensation of SRS IL up to its maximum Tx power capabilities.