3GPP TSG-RAN WG4 Meeting #113	R4-2418897

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	7.17.1

Source:	Intel Corporation

Title:	Views on general aspects of AI/ML testability and interoperability

Document for:	Discussion

# Introduction

In Rel-18 RAN WG4 conducted detailed studies on the testability and interoperability aspects of AI/ML enabled techniques for NR air interface as a part of FS_NR_AIML_Air SI [1] and the conclusions are summarized in TR 38.843 [2]. A new WI on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN #102 meeting with the latest version available in [3] with the following RAN4 objectives (the study stage objectives related to  RAN4 are marked in yellow). Furthermore, in accordance with the agreements in RAN #105 [4] the RAN4 work on the testability and interoperability was extended till the end of Rel-19.

### Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: … - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: … - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: … - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. … Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - … - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. ### Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

The work item stage discussions took place in previous meetings with agreements summarized in [4-8]. In this contribution we provide views and proposals on the remaining issues of general aspects of AI/ML testability.

# Discussion

## Post-deployment validation due to model change/drift

The topic of post-deployment functionality/drift validation was discussed during both SI and WI stages with the following conclusions reached in RAN4 #112bis [8]:

Issue 1-2: Post deployment testing - When operating in the field, two aspects of UE operation may impact performance: - Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance - Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance. - For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive: - Option 1: Conduct the validation of a change in functionality before its deployment/activation in already deployed UEs - Validation takes into account the UE hardware in which the model is to be deployed/activated. - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model) - FFS whether validation can be done at the device along with inference for another active functionality(ies) - FFS on the feasibility - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models. - Other aspects not precluded. - Option 2: Using performance monitoring and LCM procedures - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable - FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device. - For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed. - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

First of all, the RAN4 agreements may potentially have impact on RAN1/2 design, and we recommend to inform RAN1/2 WGs on the respective conclusions.

Proposal #1:	Inform RAN1/2 on RAN4 agreements on post-deployment validation due to model change/drift

### Case 1: Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance

Managing the performance of AI/ML model updates or changes, especially for UE-side models, is a key challenge in the AI/ML framework. According to study item agreements, AI/ML models may originate from a UE or a UE-side OTT server. There are two main scenarios to consider: 1) Model updates or changes that are transparent to the network; 2) Model updates that are non-transparent to the network.

AI/ML models can change over the lifetime of a UE due to reconfigurations, updates (e.g., additional training), and other reasons. These changes can affect the performance of the UE and, consequently, the overall network performance. Therefore, establishing a robust framework for model verification is an important task to be addressed.

While regular conformance tests are expected for certain default AI/ML models, the existing 3GPP conformance framework may not be suitable for post-deployment verification. Passing initial conformance tests does not guarantee maintained performance after AI/ML model updates or changes over the UE's lifetime.

Considering the agreements in previous meeting the following two options are considered.

Option 1: Conduct the validation of a change in functionality before its deployment/activation in already deployed UEs

This option involves testing AI/ML model/feature updates with validation taking into account the UE hardware in which the model is to be deployed/activated. This approach ensures that models meet minimum performance requirements, potentially reducing the need for extensive performance monitoring (e.g., not use frequent monitoring or use a subset of metrics depending on RAN1 design).

During RAN4 #112bis, it was discussed whether validation should occur in the field (on each device) or in lab conditions (per device type/model). If the AI/ML model update originates from a UE-side OTT server, device vendors can perform conformance testing before updating the model. This testing framework can be simplified and conducted by the UE vendor in the lab conditions (although not necessarily requiring certification lab testing). The validation information can then be shared with the gNB. So, least testing in the lab conditions is expected to be feasible (e.g. in case the model originates from UE-side OTT server). Meanwhile, the case of in-field testing further analysis is needed. In the latter case the set of test cases and model inputs may need to be pre-recorded at the UE and UE may need to perform extra inference to test functionality, which all have impacts on the implementation cost and complexity. Input from UE chipset vendors is needed to assess feasibility.

Same time we acknowledge that Option 1 may limit the flexibility of UE implementation and is not compatible with more adaptive AI-ML approaches such as training/updates performed directly at the UE side, or the use of localized AI/ML models optimized for specific conditions (i.e., models are not generalized for a wide set of conditions and may eventually have degraded performance under conformance test conditions).

RAN4 requirements may not cover all scenarios, including bandwidth, sub-band size, number of MIMO layers, and Tx antenna configurations. Conformance tests and requirements may not always be available, and terms like "test setup" and "minimum performance" may need further clarification.

Therefore, we believe that Option 1 alone is insufficient and should be used in conjunction with Option 2. For instance, changed functionalities that do not pass conformance testing can still be activated at the UE side, provided performance monitoring and LCM procedures are in place. The network needs to be informed on the respective information (whether UE passed testing) to make a decision on the details of configured performance monitoring framework.

Below we provide the summary of our observations and views on Option 1:

Observation #1: Option 1. Conduct the validation of a change in functionality before its deployment/activation.

- Can guarantee the minimum performance of models under various conditions.

- Applicable at least for the case when AI/ML model update comes from the UE-side OTT server, and for generalized AI-ML models with infrequent updates

- Not suitable for training/updates performed directly at the UE side, or in case of using localized AI/ML models optimized for specific scenarios.

- Can be applicable to a sub-set of scenarios covered by RAN4 specifications but cannot cover all possible deployment scenario.

- The testing can be performed in the lab conditions at least. The case of in-field on-device testing requires further analysis taking into account implementation cost/complexity.

- Changed functionalities that did not pass conformance testing can be activated at the device under an assumption that performance monitoring and LCM procedures are used in this case

Option 2: Using performance monitoring and LCM procedures

The second option is to not explicitly test the changes or updates to the models during UE lifetime (post-deployment), and instead, the network and/or UE would rely on performance monitoring mechanisms. We agree that the AI/ML performance monitoring framework is an integral part of the whole AI/ML feature, and it is important to define respective requirements to guarantee the control of performance. However, it is currently unclear whether the performance monitoring mechanism can provide reliable performance all the time and impose limited overhead at the same time. Also, it is yet unclear whether such test cases can fully replace conformance testing.

In previous meetings it was also proposed to include specific procedure for performance validation during model transfer/update, which we assume can be considered as part general performance monitoring framework. In our view, one approach is to include an emulation of conformance testing in the performance monitoring framework such that UE provides a comparison of AI-ML and non-AI-ML performance (if legacy behavior is defined). In the latter case the performance validation can be potentially effectively built-in into the AI/ML framework. The details of such approach may need to be further discussed with other WGs.

In accordance with WF [8] it is FFS on whether monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable. In our view, the respective mode of using performance monitoring is no different to Case 2 and performance monitoring can be applied for fallback management.

Observation #2: Option 2. Using performance monitoring and LCM procedures.

- The details of performance monitoring framework are not yet defined, and it is unclear whether the mechanism can guarantee minimum performance in the field.

- Applicable for a wide set of conditions

- Suitable for different types of models including fine-tuned models and localized models

Summary

In our view neither Option 1 or Option 2 are flexible enough, and the eventual design should take the pros/cons of both into account as follows. Therefore, it is suggested to support both approaches and further discuss conditions/scenario when each approach is applied.

Proposal #2:	Support both option 1 and option 2 approaches for Case 1 (Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance)

  - For Option 1

    - The testing can be performed in the lab conditions. FFS for in-field on-device testing feasibility considering UE implementation cost/complexity.

    - Changed functionalities that did not pass conformance testing can be activated at the device under an assumption that performance monitoring and LCM procedures are used in this case

    - The information on whether AI/ML model/feature update has passed conformance test (and potentially associated data) shall be conveyed to the network.

  - For Option 2:

    - Check with RAN1/2 on whether performance monitoring framework can be used to compare the performance of AI-ML model and non-AI-ML performance for use cases where legacy behavior is defined

### Case 2: Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance.

The Case 2 is focused on the scenario when performance impacts are caused by data drift / mismatch between the conditions encountered by the UE in the field and the training data. The AI-ML performance monitoring is expected to be used for this case, which is aligned with the original intention of the framework. No immediate actions need to be taken in RAN4, and we suggest waiting for the outcome of the final design in other WGs.

## Reference AI/ML models for performance requirements

Due to a diverse nature of AI/ML models implementations, there may be a significant variation in their complexity and performance, which is illustrated by RAN1 studies [2]. From RAN4 requirements perspective it is important to establish a common baseline for performance and complexity (for both 1-sided and 2-side models) and a big divergence in models’ complexity/performance may become a problem. To overcome the challenge, it is recommended to define reference AI/ML models for minimum performance requirements definition. Many existing demodulation requirements are defined under an assumption of a certain reference receiver (e.g., MMSE, R-ML, etc.) and a similar concept can be taken for 1-sided and 2-side AI/ML models. The models, whether partially or fully defined, would be utilized during the performance requirements definition stage to achieve a certain minimum level of performance under an assumption of a specific complexity level. The reference model concept is not necessarily meant to be standardized in detail, but rather should serve as a general guideline.

In previous meeting a reference model for beam management analysis was agreed for simulation purposes and a similar approach shall be adopted for all cases and for requirements definition purposes.

Proposal #3:	Define reference AI/ML models for one-sided and two-sided models for performance requirements definition considering performance/complexity trade-offs for all use cases.

# Conclusion

In this paper we provide our views on general aspects of AI/ML testability and interoperability aspects, and, in summary, make the following proposals:

Proposal #1:	Inform RAN1/2 on RAN4 agreements on post-deployment validation due to model change/drift

Proposal #2:	Support both option 1 and option 2 approaches for Case 1 (Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance)

  - For Option 1

    - The testing can be performed in the lab conditions. FFS for in-field on-device testing feasibility considering UE implementation cost/complexity.

    - Changed functionalities that did not pass conformance testing can be activated at the device under an assumption that performance monitoring and LCM procedures are used in this case

    - The information on whether AI/ML model/feature update has passed conformance test (and potentially associated data) shall be conveyed to the network.

  - For Option 2:

    - Check with RAN1/2 on whether performance monitoring framework can be used to compare the performance of AI-ML model and non-AI-ML performance for use cases where legacy behavior is defined

Proposal #3:	Define reference AI/ML models for one-sided and two-sided models for performance requirements definition considering performance/complexity trade-offs for all use cases.

# References

- RP-221348 SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #96, June 2022

- 3GPP TR 38.843 Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface

- RP-234039 WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, December 2023

- R4-2403712 WF on AI/ML, Qualcomm, RAN4 #110, February 2024

- R4-2406617 WF on AI/ML, Qualcomm, RAN4 #110bis, April 2024

- R4-2401570 WF on AI/ML, Qualcomm, RAN4 #111, May 2024

- R4-2414323 WF on AI/ML, Qualcomm, RAN4 #112, August 2024

- R4-2417212 WF on AI/ML, Qualcomm, RAN4 #112bis, October 2024