3GPP TSG-RAN WG4 Meeting #113	R4-2418899

Orlando, US, November 18th-22nd, 2024

Source: 	Ericsson

Title:  	Discussion on RRM core requirements for beam management

Agenda Item:	7.17.4.1

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #112bis, RAN4 discussed RRM requirements for AI/ML based beam management and the discussion summary is listed in [1]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

Issue 2-1: UE Rx beam knowledge

Proposals - Option 1: UE Rx beam is known - the UE Rx beam gain has to be taken into account when making the prediction - Option 2: UE Rx beam is unknown - Option 3: depends on some conditions - which conditions? - Option 4: others - Recommended WF - Option 1

The output type of the AI/ML model may be label (beam ID) & RSRP or label only.

- For the label type, the AI/ML model type is classifier network.

- For label & RSRP type, the AI/ML model type is CNN or MLP network.

Either of the two varieties in the training phase eventually demands the UE to identify beam IDs or measure the RSRPs by receiving and detecting reference signals/beams configured for the AI/ML model, with respect to the given Set A and Set B. In the way, the Rx beam shall always be enabled for the beam identification or measurement, such that the UE recognizes the association between the Tx beam and Rx beam, even if it is not specified how the UE conducts the Rx beam. Where the Rx beam is speculated through the process of Rx beam sweeping in order to detect and receive reference signals/beams.

Before the last meeting, we understood that the UE may retain the Rx beam information from the training phase to the inference phase, that is the reason why we proposed that UE always knows the Rx beam paired with the predicted Tx beam provided the predicted Tx beam is detectable in training phase and inference phase. However, after some off-line discussions with some UE vendors, it may not be always valid because some UEs may abandon the Rx beam information acquired from the training phase and such UEs may not rely on and remember the Rx beam information in the inference phase actually. We don’t think tremendous memory size could be saved by this, but we admit the possibility from UE implementation perspective.

To enhance the performance of the AI/ML model, we still think that retaining the Rx beam information during the training and inference stages is crucial.

As a result, we suppose there shall be a UE indication, e.g., UE capability or UAI, indicating whether the UE can maintain the Rx beam information from the training phase to the inference phase or not, i.e., whether the UE knows the Rx beam paired with the predicted Tx beam.

Proposal 1: UE may signal through an indication, such as UE capability or UAI, indicating whether it is able to know the Rx beam paired with the predicted Tx beam or not.

Furthermore, even if the UE has this capability, it’s observed that a UE still may not know the Rx beam associated with a Tx beam in the below two cases:

- Case Set A: the UE has not detected/measured the Tx Set A beam in training phase, but as we mentioned in above paragraph, if the TX beam isn’t trained, no reason to predict it directly in inference phase.

- Case Set B: the Tx beam is not in Set B. Same concern to the case, if the TX beam isn’t trained, no reason to predict it directly in inference phase.

Observation 1: In inference, UE may not know the Rx beam associated with a Tx beam in Set A, if a Tx beam falls into one of the two situations listed below:

- UE isn’t capable to store the Rx beam for Tx beam which has been detected/measured in training.

- UE has not been able to detect/measure the beam in training.

Issue 2-2: TCI State knowledge

- Proposals - Option 1: TCI state is known if the RS is QCL-ed with an RS measured within 1280ms - Option 2: TCI state QCL to an RS that is not in Set B is unknown even if this RS was included in the L1-RSRP report by UE within 1280ms before TCI state activation. - Option 3: The TCI state which is QCL to an RS in Set B is known: - if the corresponding predicted beam is reported in 1280ms before the TCI state switch command and SNR of the RS is above -3dB for spatial-domain beam prediction. - if the last observation occasion is within 1280ms before the TCI state switch command and SNR of the RS is above -3dB for temporal beam prediction. - Option 4: The known conditions of the predicted TCI state shall take below conditions into account: - The TCI state switch command is received during inference phase of AI/ML mode. - The predicted beam in Set A is the RS in target TCI state. - Option 5: RAN4 should consider relaxing known conditions for TCI states - Option 6: others - Recommended WF - To be discussed

Different from the legacy way as quoted in the below, defining an exact delay, i.e., 1280ms in the known conditions is not suitable for an AI/ML based known TCI state, i.e., a TCI state QCLed with a predicted Tx beam. We understand that switching to a TCI state, which is predictable in absence of measurement, is an essential improvement on beam management from the operation delay perspective.

Observation 2:  Switching to a TCI state, which is ‘known’ by the prediction without any measurement, is a valid use case.

In this manner, the criteria for known conditions based on the AI/ML model behavior must be reconstructed while taking training and interference into consideration.

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB

Proposal 2: For the prediction-based TCI state switch, RAN4 to study the known conditions based on prediction status instead of the measurement and corresponding delay.

Note: The known conditions may be regarded as the ‘predicted known’ conditions, if need to be distinguished from the ‘measured known’ conditions.

As listed in the following table, in the inference phase, the known conditions of TCI state have below relationships with the RS in the target TCI state.

| Known TCI state | Unknown TCI state |
| --- | --- |
| Detectable RS in Set A (even the RS isn’t in the prediction report by UE) and the RS in Set A is trained | Undetected RS in Set A, or Detected RS in Set A, but the RS isn’t trained |
| Detectable RS in Set B | Undetectable RS in Set B |

Proposal 3: The ‘predicted known’ conditions of the prediction-based TCI state shall take below conditions into account:

- The TCI state switch command is received during valid inference phase of AI/ML mode.

- The predicted beam in Set A is the RS in target TCI state.

- The predicted beam in Set A is detectable during training and inference phase, which implies the beam has been trained successfully and available in inference phase.

- The UE knows the Rx beam for the predicted beam.

Proposal 4: Below cases don’t need to be considered for the beam prediction-based TCI state switch.

- During training phase, measurement reporting on Set A and Set B also may enable the NW to configure a TCI state associated with a RS out of the measured Set A or Set B. However, it is not relevant to AI/ML model, instead the legacy known conditions can be applied. Hence, the known conditions for Set A or Set B during training phase are not in scope.

- During inference phase, it may occur that the TCI state is QCLed with a RS belonging to Set B. The legacy known conditions are applied for it. Hence, the known conditions for Set B during inference phase are not in scope.

Issue 2-3: RSRP Absolute accuracy

Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

We understand the concern by some companies is which one shall be referred as the benchmark, the top-K beams w.r.t. the predicted RSRP or the top-K beams w.r.t. the ground truth RSRP.

In our opinion, the beams that shall be taken care are the predicted beams. In other words, the prediction (accuracy) shall focus on the beams that the UE reports but not the beams supplied by the genie way.

If there are offsets between the two approaches, it’s questionable to adopt the top-K beams w.r.t. the ground truth RSRP. The below are two examples demonstrates our concern:

- For the case of a beam in the top-K predicted beams but not in the genie beams, the beam’s RSRP has not any requirement if the requirement is only applicable to the top-K beams w.r.t. the ground truth RSRP?

- On the contrary, for the case of a beam in the top-K genie beams but not in the predicted beams, why should we require the RSRP accuracy of the beam that the UE doesn't even report?

Proposal 5: For the absolute RSRP, the index i may be any beam index in top-K beams based on predicted L1-RSRP as per UE reporting.

Issue 2-4: Relative RSRP Accuracy

Regarding the relative RSRP accuracy, the proposals in the last meeting are captured in the below.

- Proposals - Option 1: The definition of relative RSRP accuracy shall be: - Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest value (including predicted and measured beams), if beam index n is not configured for corresponding measurement. - Relative RSRP accuracy= predicted L1-RSRP of beam index i – measured L1-RSRP of beam index n, where the beam index n owns the largest value (including predicted and measured beams), if beam index n is configured for corresponding measurement. - Option 2: Relative RSRP accuracy is the RSRP difference between the predicted L1-RSRP of different beams - Option 3: Relative RSRP accuracy is modified as follows: - Relative RSRP accuracy = predicted L1-RSRP of beam index i – predicted measured L1-RSRP of predicted beam index n, where the beam index n owns the largest predicted measured value obtained by measuring the predicted beams. - Option 4: Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value - Recommended WF - Option 2

Firstly, recap the legacy relative RSRP accuracy definition in Clause 10.1.20.1.2 of TS38.133, which is copied as follows:

﻿The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell.

The intention of the relative accuracy requirement is to guarantee the accuracy of the relative L1-RSRP, which allows the network to compare the received signal power from different beams (or cells) more accurately by comparing the relative differences among them. It consequently aids the UE and the network in determining the optimal beam or cell. Hence, for serving the purpose, the reference L1-RSRP of relative accuracy shall be assessed.

In legacy, there is no doubt that the reference L1-RSRP is the largest measured value of L1-RSRP. Differently, in AI/ML based beam management, even we understand there may be certain underlying advantage by adopting measured L1-RSRP as reference, however a simple way is to define the reference L1-RSRP is the largest predicted value of L1-RSRP, as a peer-to-peer comparison.

Proposal 6: Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams.

Issue 2-5: Requirements for beam predictions

Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed

In this issue, we share our views on the first issue ‘measurement period for prediction (how long will set B be measured before making a prediction)’.

To our understanding, the question shall be separated for BM-case 1 and BM-case 2. Based on the understanding of RAN1 studies, for BM-case1, one shot L1-RSRP measurement result on Set B is enough for the UE to predict Set A, for BM-case 2, a UE may measure Set B for multiple L1-RSRP measurement periods/occasions, and the exact number of the occasions may be configurable, depending on the RAN1 study results.

Proposal 7: Regarding measurement period for prediction (how long will set B be measured before making a prediction),

- For BM-case1, one shot L1-RSRP measurement result on Set B is enough for the UE to predict Set A.

- For BM-case 2, the UE may measure Set B for multiple L1-RSRP measurement periods/occasions before prediction. The exact number of the periods/occasions may be configurable which is to be studied and defined in RAN1.

Issue 2-7: Beam prediction KPIs:

Proposals - Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use combined Option 2 and 3: - Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB” - FFS: K = 2 and x = 3 as baseline - Option 4: Neither Option 1, 2, 3 is appropriate, a new metric is needed - Option 5: Option 5: Consider reliability metric based on relative difference in between the predictions due to change in the conditions/inputs. - consider defining multiple tests under different conditions but with the same outcome(same predicted beam) - Option 6: Other - Recommended WF - To be discussed Companies suggesting to use different metrics(new) should come up with a concrete proposal

We consider the metrics/KPIs including options for beam management requirements/tests as follow:

- In the case of the AI/ML model with label (beam ID) output type, the metrics/KPIs for beam management requirements/tests are derived by the beam prediction accuracy, i.e., the UE shall only fulfil beam prediction accuracy.

- In the case of the AI/ML model with label (beam ID) + RSRP output type, the Metrics/KPIs for beam management requirements/tests shall take both of the beam prediction accuracy and RSRP accuracy into account, i.e., the UE shall fulfil beam prediction accuracy and RSRP accuracy simultaneously.

Proposal 8: Metrics/KPIs for beam management requirements/tests are outlined by the below options:

- In the case of the AI/ML model with label (beam ID) output type

  - Beam prediction accuracy, i.e., Option 2, is adopted.

- In the case of the AI/ML model with label (beam ID) + RSRP output type

  - Beam prediction accuracy and RSRP accuracy, i.e., Option1, 2 an 3, are adopted simultaneously.

# Conclusion

Observation 1: In inference, UE may not know the Rx beam associated with a Tx beam in Set A, if a Tx beam falls into one of the two situations listed below:

- UE isn’t capable to store the Rx beam for Tx beam which has been detected/measured in training.

- UE has not been able to detect/measure the beam in training.

Observation 2:  Switching to a TCI state, which is ‘known’ by the prediction without any measurement, is a valid use case.

Proposal 1: UE may signal through an indication, such as UE capability or UAI, indicating whether it is able to know the Rx beam paired with the predicted Tx beam or not.

Proposal 2: For the prediction-based TCI state switch, RAN4 to study the known conditions based on prediction status instead of the measurement and corresponding delay.

Note: The known conditions may be regarded as the ‘predicted known’ conditions, if need to be distinguished from the ‘measured known’ conditions.

Proposal 3: The ‘predicted known’ conditions of the prediction-based TCI state shall take below conditions into account:

- The TCI state switch command is received during valid inference phase of AI/ML mode.

- The predicted beam in Set A is the RS in target TCI state.

- The predicted beam in Set A is detectable during training and inference phase, which implies the beam has been trained successfully and available in inference phase.

- The UE knows the Rx beam for the predicted beam.

Proposal 4: Below cases don’t need to be considered for the beam prediction-based TCI state switch.

- During training phase, measurement reporting on Set A and Set B also may enable the NW to configure a TCI state associated with a RS out of the measured Set A or Set B. However, it is not relevant to AI/ML model, instead the legacy known conditions can be applied. Hence, the known conditions for Set A or Set B during training phase are not in scope.

- During inference phase, it may occur that the TCI state is QCLed with a RS belonging to Set B. The legacy known conditions are applied for it. Hence, the known conditions for Set B during inference phase are not in scope.

Proposal 5: For the absolute RSRP, the index i may be any beam index in top-K beams based on predicted L1-RSRP as per UE reporting.

Proposal 6: Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams.

Proposal 7: Regarding measurement period for prediction (how long will set B be measured before making a prediction),

- For BM-case1, one shot L1-RSRP measurement result on Set B is enough for the UE to predict Set A.

- For BM-case 2, the UE may measure Set B for multiple L1-RSRP measurement periods/occasions before prediction. The exact number of the periods/occasions may be configurable which is to be studied and defined in RAN1.

Proposal 8: Metrics/KPIs for beam management requirements/tests are outlined by the below options:

- In the case of the AI/ML model with label (beam ID) output type

  - Beam prediction accuracy, i.e., Option 2, is adopted.

- In the case of the AI/ML model with label (beam ID) + RSRP output type

  - Beam prediction accuracy and RSRP accuracy, i.e., Option1, 2 an 3, are adopted simultaneously.

# References

- R4-2414323 WF on AI/ML, Qualcomm