3GPP TSG-RAN WG4 Meeting #113	R4-2418904

Orlando, US, November 18th-22nd, 2024

Source: 	Ericsson

Title:  	Discussion on RedCap RRM requirements for NTN for NR Phase 3

Agenda Item:	7.26.4.1

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RedCap RRM requirements mandated by the issues listed in the WF.

# Discussion

## Topic #2: General consideration on RRM requirements

#### Issue 2-2-1: Further discussion on cell reselection requirement for (e)RedCap UE with FR1-NTN

Online agreement <Agreement>: - The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands. - Both time-based and location-based cell reselection measurement criteria will be defined. - For both earth fixed and earth moving scenario - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined. - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

Since cell reselection to inter-RAT E-UTRAN cell is supported by RedCap UE in TN, we slightly prefer that inter-RAT (NTN to E-UTRA TN) measurement is supported and the requirements for it shall be defined.

Proposal 1: Define the requirements for inter-RAT (NTN to E-UTRA TN) measurement.

#### Issue 2-2-3: The Requirements to be further determined

Online agreement <Agreement>: - Define NR Rel-17 and Rel-18 Conditional Handover requirements. - Include location-based CHO and time-based CHO. - Support earth moving cell scenario - Define RACH-less Handover requirements. - Define SDT requirements (including CG-SDT and RA-SDT) in RRC_INACTIVE state. - FFS TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - FFS: MDT requirements in RRC_IDLE and RRC_INACTIVE state.

As we presented in the last meeting, since MDT isn’t supported in RedCap for TN, we don’t see the necessity to introduce the feature for RedCap for NTN, which conflict with the lower-cost, lower-complexity and energy-efficient design target of RedCap.

Proposal 2: To our opinion, MDT for NTN RedCap isn’t a fundamental feature, we lean toward preclude it.

#### Issue 2-3-1: RedCap R17 relaxed measurement

Online agreement <Agreement>: - FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline. - If defined, the requirements apply provided that target cell is GSO

The conditions for TN RedCap relaxed measurement requirements in TS38.133 are duplicated as follows.

4.2B.2.9 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap 4.2B.2.9.1 Introduction This clause contains the requirements for measurements on intra-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement critera: - Relaxed measurement criterion for a stationary UE defined in clause 5.2.4.9.3 in [1], - Relaxed measurement criteria for a stationary UE and not-at-cell edge defined in clause 5.2.4.9.4 in [1], - Both low mobility criterion and stationary criterion as defined in clause 5.2.4.9.1 and 5.2.4.9.3 or 5.2.4.9.4 in [1] respectively. - Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1], - Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.9.2 in [1], - Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.9.1 and 5.2.4.9.2 in [1] respectively.

The conditions for R17 NR NTN relaxation requirements in GSO in TS38.133 are duplicated as follows.

4.2C.2.7 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion The requirements in clause 4.2.2.9 apply provided that target cell’s satellite is GEO.

4.2.2.9 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion 4.2.2.9.1 Introduction This clause contains the requirements for measurements on intra-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement critera: - Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1], - Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.9.2 in [1], - Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.9.1 and 5.2.4.9.2 in [1] respectively.

Refer to R17 NR NTN relaxation requirements in GSO, the UE shall meet the conditions in Clause 4.2.2.9.1 for relaxed measurement even the UE is in a GSO scenario.

And, from the comparison in the above, it’s observed that TN RedCap relaxed measurement requirements already covers the conditions for R17 NR NTN relaxation requirements in GSO, which are highlighted in the above tables. But, it is noted that the relaxation factor is different, as below two tables listed, the relaxation factor=3 for NR and the relaxation factor=6 for RedCap.

Proposal 3: RedCap relaxed measurement requirements for cell reselection (Clause 4.2B.2.9, 4.2B.2.10, 4.2B.2.11 in TS 38.133 V18.7.0) can be applied to (e)RedCap NTN UE, i.e., relaxation factor K3 = 6 which is aligned with RedCap TN instead of 3 which is for Rel-17 NTN relaxation.

#### Issue 2-3-2: eDRX configuration

Online agreement <Agreement>: - For GSO: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used. - Further discuss for LEO.

For the LEO case, even we lean toward no limitation of eDRX in specification, we can be open to accept the suggestions in the on-line discussions in the last meeting, in order to engage the progress.

Proposal 4:

- For LEO earth moving:

  - Not define eDRX related requirements in RRC_Idle / RRC_Inactive mode

- For LEO earth fixed:

  - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN.

#### Issue 2-3-3: Multiple SMTCs

<Way Forward>: - Proposal 1 (HW): RAN4 to discuss support of multiple SMTCs for (e)Redcap UE with FR1-NTN bands. - Option 1: make it optional for RedCap UE to support 2 SMTC per MO. - Option 2: change the requirements so that measurement for different SMTCs are not parallel. - Proposal 2 (ZTE): Multiple SMTCs shall be reused for RedCap UEs in NTN scenario. - Proposal 2a (Ericsson): RedCap for NTN supports (at the least) 2 SMTC. - Proposal 3 (OPPO): RAN4 to not consider the support of multiple SMTCs for (e)RedCap UE with FR1-NTN bands. - Proposal 4 (QC): NTN RRM requirements defined for simultaneous measurements on multiple satellites are not applicable for RedCap NTN UE.

In NTN Rel-17, to measure the serving satellite and neighbor satellites, a UE shall at the least be capable of 2 SMTCs if SSBs from neighbor cells isn’t always captured by the SMTC which containing SSBs from the serving cell. Otherwise, only 1 SMTC shall cover the serving satellite and neighbor satellites always.

Besides, in NTN Rel-17, NLEO, simul is introduced as the number of satellites that UE can measure in parallel within a SMTC. If NLEO, simul =1 and the single SMTC covers the serving satellite and neighbor satellites, the UE has to measure the serving satellite and neighbor satellites in a TDM manner.

If the UE capable of only 1 SMTC doesn’t support parallelMeasurementWithoutRestriction, then the UE only can measure one NGSO satellite while operating the serving satellite in a TDM manner.

The above assumptions could be regarded as the minimal requirement to an NTN UE to be able to perform inter-satellite mobility while keeping the measurement/operation on the serving cell/satellite. We agree that the scheme still can work but it restricts the use cases.

Besides, RedCap for TN already supports 2 SMTCs already, even though the configurations are different from NTN SMTC configurations. We don’t observe the difficulty adopting more than one SMTC in a NTN way.

Observation 1: RedCap for TN already supports 2 SMTCs already, RedCap for NTN shall at the least have same capability.

Proposal 5: For RedCap UE for NTN, only supporting 1 SMTC may limit the use cases, we prefer that (at the least) 2 SMTCs shall be supported.

Proposal 6: RedCap for NTN UE supports 1 MG, and optionally supports more than 1 MG.

Proposal 7: If RedCap NTN UE doesn’t support simultaneous measurements between neighbour satellites and operations on the serving satellite, it can configure parallelMeasurementWithoutRestriction as not supported.

Proposal 8: If RedCap NTN UE doesn’t support parallel measurements of satellite in a SMTC, NLEO, simul can be set as 1.

The number of cells and number of SSB supported by RedCap for TN is duplicated as follows.

9.2B.3 Number of cells and number of SSB 9.2B.3.1 Requirements for FR1 For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 8 identified cells, and - 14 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

The number of cells and number of SSB supported by NR NTN is duplicated as follows.

9.2C.3 Number of cells and number of SSB 9.2C.3.1 Requirements for FR1 For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 8 identified cells, and - 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is not smaller than the number of configured RLM-RS SSB resources. - 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment. - In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

Compared with RedCap for TN, the number of cells and number of SSB supported by RedCap for NTN doesn’t need to be reduced further. Meanwhile, we may add the satellite number support for RedCap for NTN.

Proposal 9: Ruse the number of cells and SSB supported by RedCap TN for RedCap NTN. And the number of supported satellites is same as NR NTN.

#### Issue 2-3-5: The number of searchers and CSSF

<Way Forward>: - Proposal 1 (CMCC, CATT): Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

Proposal 10: Proposal 1 is fine with us, single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

#### Issue 2-3-6: FH in PRS measurement for NW verified location requirements

<Way Forward>: - Proposal 1 (HW): RAN4 to discuss whether to support FH in the requirements for NW verified location for (e)RedCap UE with FR1-NTN bands. - Proposal 2 (Ericsson, OPPO): Not support FH in PRS measurement for RedCap for NTN.

Proposal 11: Not support FH in PRS measurement for RedCap for NTN.

#### Issue 2-3-7: NCD-SSB

In the last meeting, RAN4 has the following agreements for NCD-SSB according to the WF: #### Issue 6-6-1: The impact of NCD-SSB for (e)RedCap UEs with FR1-NTN Online agreement <Agreement> - For (e)RedCap UEs with FR1-NTN, there is no need to define NCD-SSB specific measurement requirements.. Therefore, for this issue, suggest RAN4 to further focus on how to understand ‘no need to define NCD-SSB specific measurement requirements’ agreed in the last meeting and how to draft the CR. <Way Forward>: - Option 1: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause on this aspect (i.e, the requirements are only applicable to CD-SSB). - Option 2: Use the terminology of CD-SSB explicitly for (e)RedCap with FR1-NTN requirements. - Note: Other options are not precluded.

Proposal 12: We prefer Option 1, use the terminology of SSB without differentiating CD-SSB/NCD-SSB in requirement chapters, and add a clarification in a separate clause on this aspect (i.e., the requirements are only applicable to CD-SSB).

## Topic #3: Reduction in the number of UE Rx branches

#### Issue 3-2-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, firstly to discuss the following proposals one by one: - Proposal 1: RAN4 to consider the offset for the reletive cell-specific RSRP thresholds for 1Rx (e)RedCap UEs with FR1-NTN bands, including Qrxlevmin and Qqualmin: - -1 dB offset can be reused. - Proposal 2: RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring for 1Rx (e)RedCap UEs with FR1-NTN bands. - 6 NR inter-frequency carriers can be reused. - Proposal 3: The paremeters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)Redcap UE, and the requirements will be the same. - Proposal 4: For the measurements of intra- and inter-frequency NR cells, - +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline for 1Rx(e)RedCap UEs with FR1-NTN bands. - For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked for 1Rx (e)RedCap UEs with FR1-NTN bands. - Proposal 5: In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria for 1Rx (e)RedCap with FR1-NTN: - For 1 Rx RedCap by a margin of at least - 6 dB in FR1 for reselections based on ranking or - 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or - 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities. - Further discuss SDT requirements. - MDT requirements are related to issue 2-2-3, RAN4 to discuss them based on the conclusions on issue 2-2-3 later.

Proposal 13: The options in WF for 1Rx (e)RedCap aren’t controversial, we support to adopt them all, including:

  - Proposal 1:Qrxlevmin and Qqualmin:

    - -1 dB offset can be reused.

  - Proposal 2:the number of NR inter-frequency carriers:

    - 6 NR inter-frequency carriers can be reused.

  - Proposal 3: Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap

    - Same for 1RX and 2 RX

  - Proposal 4: For the measurements of intra- and inter-frequency NR cells,

    - +1 dB offset on the threshold of absThreshSS-BlocksConsolidation.

    - For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset for better cell ranked for 1Rx (e)RedCap for NTN.

  - Proposal 5: In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria for 1Rx (e)RedCap with FR1-NTN:

    - For 1 Rx RedCap by a margin of at least

      - 6 dB in FR1 for reselections based on ranking or

      - 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

      - 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

#### Issue 3-2-2: NR HO requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>: - For NR RACH-based HO requirements, - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-based HO requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs. - Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused. - Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO - Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO - Further discuss the requirements for RACH-less HO, Time/location-based CHO with/without L3 measurement and Satellite switching with re-sync.

Proposal 14: For RACH-less HO, Tsearch relaxion for 1Rx (e)RedCap UE for NTN is identical with that for RACH-based HO.

Proposal 15: For time/location-based CHO with L3 measurement, Tmeasure, i.e., Tidentify_intra_with_index or Tidentify_intra_without_index relaxion for 1Rx (e)RedCap UE for NTN reuses that for 1Rx (e)RedCap UE for TN.

Proposal 16: For time/location-based CHO without L3 measurement, no difference btween 1 Rx/2Rx (e)RedCap UE for NTN.

Proposal 17: For satellite switching with re-sync, Tsearch  can be relaxed further 1 SSB burst from [Tfirst_SSB] ms, for 1Rx (e)RedCap UE for NTN.

## Topic #4: The impact of HD-FDD

#### Issue 4-1-1: General HD-FDD related impact

<Way Forward>: - Proposal 1 (Xiaomi, CATT, QC, ZTE, CMCC): - RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions. - Proposal 1a (CATT): - To following the TN, at least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - Minimum requirement for L1 indication for RLM and LR - MAC-CE/DCI/RRC based uplink spatial relation switch delay - Scheduling availability of UE performing intra/inter measurements - Scheduling availability of UE during L1-RSRP measurement - Proposal 1b (QC): RAN4 to wait for further RAN1 process on the enhancements for HD RedCap and eRedCap UEs which may potentially impact the following aspects: - Additional latencies, Scheduling/Measurement restrictions, etc. due to DL/UL collisions.

There is still active work and progress in RAN1 regarding HD RedCap and eRedCap. In RAN4#118bis collision case 3 was further refined and RAN1 concluded that [2]:

Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS

Proposal 18: Regarding general HD-FDD related impact, agree on Proposal 1 in WF, check further conclusions from RAN1.

#### Issue 4-2-1: The impact on NR Handover due to HD-FDD

<Way Forward>: - Proposal 1 (vivo): RAN4 to reuse NR SAN FR1 – NR SAN FR1 Handover requirements for Red-cap NTN and additionally add the following applicability rule for HD-FDD case as current HO requirements for Redcap specified, - For RedCap NTN UE with HD-FDD, the handover requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU.

Proposal 19: Agree on the proposal in WF for impact on NR Handover due to HD-FDD.

#### Issue 4-2-2: The impact on NR Satellite switching with re-synchronization due to HD-FDD

<Way Forward>: - Proposal 1 (vivo): For requirements on NR Satellite switching with re-synchronization for Red-cap NTN, RAN4 to reuse the existing Satellite switching with re-synchronization requirements for NTN with adding the following applicability rules to adapt HD-FDD case: - For RedCap NTN UE with HD-FDD, the Satellite switching with re-synchronization requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU.

Proposal 20: Agree on the proposal in WF for the impact on NR Satellite switching with re-synchronization due to HD-FDD.

## Topic #5: Others

#### Issue 5-1-1: Specification structure

<Way Forward>: - Proposal 1 (CATT, HW): - Use new sections and section number with new suffix X in the specification to capture requirements for (e)RedCap UE with FR1-NTN bands. - Utilize the reference method for the requirements that can be reused. - Proposal 2 (Nokia): - Adopt RedCap subclauses under NTN sections (under Suffix C).

Proposal 21: We lean toward Proposal 1, in order to avoid further complexing the clauses for NTN.

#### Issue 5-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

<Way Forward>: - Proposal 1 (Nokia): - RAN4 to discuss whether a new capability is needed to declare RedCap support of NTN or whether this support is implicit from declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

It appears that some features applicable to (e) RedCap for NTN partially inherits from RedCap for TN and NR NTN respectively, but not the simple combination of all features of the two ones. Such that, declaring support of RedCap and NTN may lead to ambiguity, e.g., between the applicability of the features supported by them and the applicability of the features supported by (e) RedCap for NTN.

Proposal 22: RAN4 to define a new capability declaring (e) RedCap for NTN.

# Conclusion

Observation 1: RedCap for TN already supports 2 SMTCs already, RedCap for NTN shall at the least have same capability.

Proposal 1: Define the requirements for inter-RAT (NTN to E-UTRA TN) measurement.

Proposal 2: To our opinion, MDT for NTN RedCap isn’t a fundamental feature, we lean toward preclude it.

Proposal 3: RedCap relaxed measurement requirements for cell reselection (Clause 4.2B.2.9, 4.2B.2.10, 4.2B.2.11 in TS 38.133 V18.7.0) can be applied to (e)RedCap NTN UE, i.e., relaxation factor K3 = 6 which is aligned with RedCap TN instead of 3 which is for Rel-17 NTN relaxation.

Proposal 4:

- For LEO earth moving:

  - Not define eDRX related requirements in RRC_Idle / RRC_Inactive mode

- For LEO earth fixed:

  - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN.

Proposal 5: For RedCap UE for NTN, only supporting 1 SMTC may limit the use cases, we prefer that (at the least) 2 SMTCs shall be supported.

Proposal 6: RedCap for NTN UE supports 1 MG, and optionally supports more than 1 MG.

Proposal 7: If RedCap NTN UE doesn’t support simultaneous measurements between neighbour satellites and operations on the serving satellite, it can configure parallelMeasurementWithoutRestriction as not supported.

Proposal 8: If RedCap NTN UE doesn’t support parallel measurements of satellite in a SMTC, NLEO, simul can be set as 1.

Proposal 9: Ruse the number of cells and SSB supported by RedCap TN for RedCap NTN. And the number of supported satellites is same as NR NTN.

Proposal 10: Proposal 1 is fine with us, single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

Proposal 11: Not support FH in PRS measurement for RedCap for NTN.

Proposal 12: We prefer Option 1, use the terminology of SSB without differentiating CD-SSB/NCD-SSB in requirement chapters, and add a clarification in a separate clause on this aspect (i.e., the requirements are only applicable to CD-SSB).

Proposal 13: The options in WF for 1Rx (e)RedCap aren’t controversial, we support to adopt them all, including:

  - Proposal 1:Qrxlevmin and Qqualmin:

    - -1 dB offset can be reused.

  - Proposal 2:the number of NR inter-frequency carriers:

    - 6 NR inter-frequency carriers can be reused.

  - Proposal 3: Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap

    - Same for 1RX and 2 RX

  - Proposal 4: For the measurements of intra- and inter-frequency NR cells,

    - +1 dB offset on the threshold of absThreshSS-BlocksConsolidation.

    - For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset for better cell ranked for 1Rx (e)RedCap for NTN.

  - Proposal 5: In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria for 1Rx (e)RedCap with FR1-NTN:

    - For 1 Rx RedCap by a margin of at least

      - 6 dB in FR1 for reselections based on ranking or

      - 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

      - 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

Proposal 14: For RACH-less HO, Tsearch relaxion for 1Rx (e)RedCap UE for NTN is identical with that for RACH-based HO.

Proposal 15: For time/location-based CHO with L3 measurement, Tmeasure, i.e., Tidentify_intra_with_index or Tidentify_intra_without_index relaxion for 1Rx (e)RedCap UE for NTN reuses that for 1Rx (e)RedCap UE for TN.

Proposal 16: For time/location-based CHO without L3 measurement, no difference btween 1 Rx/2Rx (e)RedCap UE for NTN.

Proposal 17: For satellite switching with re-sync, Tsearch  can be relaxed further 1 SSB burst from [Tfirst_SSB] ms, for 1Rx (e)RedCap UE for NTN.

Proposal 18: Regarding general HD-FDD related impact, agree on Proposal 1 in WF, check further conclusions from RAN1.

Proposal 19: Agree on the proposal in WF for impact on NR Handover due to HD-FDD.

Proposal 20: Agree on the proposal in WF for the impact on NR Satellite switching with re-synchronization due to HD-FDD.

Proposal 21: We lean toward Proposal 1, in order to avoid further complexing the clauses for NTN.

Proposal 22: RAN4 to define a new capability declaring (e) RedCap for NTN.

# References

- R4-2414037, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT