# 3GPP TSG-RAN WG4 Meeting #113	R4-2418913

Orlando, Florida, US, 18th – 22nd November, 2024

Source:			Ericsson

Title:			New NTN S-band –UE RF requirements

Agenda item:		6.17.2

Document for:		Approval

# Introduction

In RAN#104, a WI ([1]) has been approved to specify new NTN S-band.

In last RAN4#112bis meeting, a way forward ([3]) was agreed highlighting the open issues related to UE-UE coexistence between the new NTN S-band and TN bands n2/n25. This contribution is further discussion those aspects.

# Discussion

## Background

The new NTN S-band would occupy the 2000-2020 MHz frequency range in UL and 2180-2200 MHz in DL. The adjacent and overlapping bands are shown in Figure 1.

Figure 1: Adjacent and overlapping bands to the new NTN S-band

## NTN-TN isolation distance for FR1-NTN

In the RAN4#112bis WF ([3]), issue 1-1-3 is related to the TN-NTN isolation distance:

To our view, such discussion will not help solving the UE-UE coexistence issue for this new NTN S-band.

To solve the simulation “case 1” issue and guarantee a minimum performance in the NTN network, RAN4 agreed to consider an isolation distance between the edge of a TN network and the location where a NTN UE could be connected to a satellite, considering that TN connection will be prioritized over NTN.

Nevertheless, while this isolation distance was assumed, no mechanism has been specified to prevent an NTN UE to connect to a satellite in the proximity or even inside the coverage of a TN network. In other words, an NTN UE could still connect to a satellite while in TN coverage, meaning also NTN UE and TN UE could still be in close proximity.

Observation1: A FR1-NTN UE might still connect to a satellite while it’s in TN coverage area (e.g. located in close proximity to a TN UE), no mechanism has been specified to prevent this situation.

Proposal1: RAN4 to agree that discussion on the isolation distance assumed for the NTN study will not address the UE-UE coexistence issue between the new NTN S-band and TN bands n2/n25.

## Less stringent coexistence limits

In the RAN4#112bis WF ([3]), issue 1-1-2 is related to the UE-UE coexistence issue:

As it was already done with other bands, it should be possible to relax the usual -50dBm/MHz limit but this should come with specifying corresponding restrictions on the channel bandwidth, frequency carrier position and/or RBs restrictions. For example, in band n106 coexistence with n5/n26 case, the limit was relaxed to -30dBM/MHz but this was possible because only 3 MHz channel bandwidth signal was possible in that band n106 and the channel raster (NR-ARFCN) for such signal is restricted to limited position.

Similar approach could be considered for this NTN band n252. This would require some iterations to determine the best compromise between a more relaxed coexistence limit (that would still be feasible) and a less stringent RBs/scheduling restriction.

# Conclusion

In this contribution, we further discuss the new S-band UE coexistence requirement with TN bands n2 and n25. We made the following observation and proposal:

Observation1: A FR1-NTN UE might still connect to a satellite while it’s in TN coverage area (e.g. located in close proximity to a TN UE), no mechanism has been specified to prevent this situation.

Proposal1: RAN4 to agree that any discussion on the isolation distance assumed for the NTN study will not address the UE-UE coexistence issue between the new NTN S-band and TN bands n2/n25.

# References

- RP-241661, New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- TR 36.811, Adding 2 GHz band LTE Frequency Division Duplex (FDD) (Band 23) for Ancillary Terrestrial Component (ATC) of Mobile Satellite Services (MSS) in North America

- R4-2417206, WF on NTN and IoT-NTN bands, Inmarsat

- R4-2416144, New NTN S-band –UE RF requirements, Ericsson